| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Message format of AMS measurement configuration and report for Self-organizing network |  |
| Date Submitted | 2009-08-31 |  |
| Source(s) | Jungje Son Samsung Electronics Co., Ltd. Anshuman Nigam Samsung India Software Operation Ying Li, Baowei Ji Samsung Telecommunications America | jungje.son@samsung.com anshun@samsung.com ying.l@samsung.com baoweiji@samsung.com |
| Re: | 802.16m Amendment Working Document: IEEE 802.16m-09/037, “Call for Contributions on Project 802.16m Amendment Content” |  |
| Abstract | The contribution proposes detailed text on support of Femtocells (Section 15.4) |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text

15.4.10.2 Low Duty Mode Patterns

A Low Duty Mode pattern is defined as a combination of AI and UAI which are in units of superframe. The start of the pattern is denoted by Start Offset. Several such patterns can be defined by the network. The collection of all such patterns in the network is referred to as the table of patterns indexed such that the pattern with least (AI+UAI) has the lowest index. The patterns are defined such that the (AI+UAI) of a pattern is multiplicative of the AI+UAI of the pattern of the preceding index.

For example, the AI of smallest cycle (AI+UAI) pattern is AI0 and UAI is UAI0. Then the next pattern (with larger cycle) is defined as AI1 and UAI1 where AI1 = AI0 and UAI1 = {(AI0 + UAI0)*MultiplicativeFactor – AI0}. Further higher cycle patterns are defined similarly. The number of patterns is given by NumberOfPatterns.

The macrocell BS/femtocell BS shall either broadcast or unicast upon MS request or pre-provision during initial network entry, the information of all the defined patterns.

Specific pattern for a femtocell may be decided by the operator or by SON.

A new TLV is defined for the purpose of providing the information of all the patterns to the MS as LDMPatternTableTLV.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| LDMPatternTableTLV { |  |  |
| LengthOfAI | 4 | AI remains fix and UAI is varied in multiplicative fashion for subsequent patterns |
| LengthOfLowestPatternIndexUAI | 4 |  |
| MultiplicativeFactor | 4 |  |
| NumberOfPatterns | 4 |  |
| } |  |  |

The LDMPatternTableTLV can be broadcasted in the AAI_NBR-ADV messages or can be unicast in the AAI_SCN-RSP when requested by the MS or it can be pre-provisioned by the network.

In order to have optimal search of the femtocells the AIs for patterns on different FAs should be non-overlapping. This shall be achieved by staggering the Start Offset of the patterns with respect to the FA. This is implicitly determined by the equation

SFN mod (AI+UAI) == (FAindex * AI)

Where, SFN is the Superframe number of the macro cell or femtocell from which the MS obtained the pattern information. This SFN denotes the Start Offset of the pattern.

FAindex is known from the AAI_NBR-ADV message.

The MS may request the macrocell BS/femtocell BS for the pattern index information of specific femtocells in AAI_NBR-REQ/AAI_SCN-REQ message. The macrocell BS/femtocell BS may provide the index of the pattern for the requested femtocell BSs in the unicasted AAI_NBR-ADV/AAI_SCN-RSP message.

Further the macrocell BS/femtocell BS may provide via broadcast (in AAI_NBR-ADV) or unicast (in AAI_SCN-RSP/ AAI_NBR-ADV) the indexes of the lowest and highest periodicity (AI+UAI) in the vicinity of the MS or may provide only the index of the lowest periodicity pattern in the vicinity of the MS.

A new TLV is defined for the purpose of the MS requesting specific Femtocells pattern index information as LDMPatternInfoReqTLV

A reserved value of 0xF for the PatternIndex indicates that pattern information is not available.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| LDMPatternInfoReqTLV { |  |  |
| NumberOfFemtocells | TBD |  |
| For(j=0; j<NumberOfFemtocells; j++) { |  |  |
| FemtocellIDType | 1 | 0: BSID 1: CSGID |
| FemtocellID | 24 | LSBs of BSID or full CSGID |
| } |  |  |
| } |  |  |

The LDMPatternInfoReqTLV can be embedded in AAI_NBR-REQ/AAI_SCN-REQ message.

A new TLV is defined for the purpose of providing the information of the patterns in use to the MS as LDMPatternInfoTLV.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| LDMPatternInfoTLV { |  |  |
| Mode | 2 |  |
| If (Mode == 0x00) { |  |  |
| NumberOfFemtocells | TBD | As per last LDMPatternInfoReqTLV sent by MS |
| For (j =0; j<NumberOfFemtocells; j++) { |  |  |
| if(FemtocellIDType == 0) |  | In the same order as per the last LDMPatternInfoReqTLV sent by MS |
| PatternIndex | 4 | A reserved value of 0xF indicates that pattern information is not available |
| Else { |  |  |
| NumberOfPatternIndexes | 4 | Different Femtocells in the same CSG may have different patterns. |
| For(i=0; i< NumberOfPatternIndexes; i++) { |  |  |
| PatternIndex | 4 |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Else if (Mode == 0x01) { |  |  |
| LowestPatternIndex | 4 |  |
| HighestPatternIndex | 4 |  |
| } |  |  |
| Else if (Mode == 0x10) { |  |  |
| LowestPatternIndex | 4 |  |
| } |  |  |
| } |  |  |

The LDMPatternInfoTLV can be embedded in AAI_NBR_ADV (broadcasted if the Mode is 0x10, unicasted if Mode is 0x00) and in AAI_SCN_RSP messages.