| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Low Duty Mode Patterns (15.4.10) |  |
| Date Submitted | 2009/8/29 |  |
| Source(s) | Andreas Maeder, Linghang Fan, Nader Zein, Tetsu Ikeda NEC | E-mail: andreas.maeder@nw.neclab.eu linghan.fan@eu.nec.com nader.zein@eu.nec.com t-ikeda@ap.jp.nec.com |
| Re: | IEEE 802.16m-09/0037 Call for Contributions on Project 802.16m Amendment Content P802.16m/D1 (15.4.10) |  |
| Abstract | This contribution defines pattern types for the low duty operation mode |  |
| Purpose | To be discussed and adopted by TGm for the P802.16m/D1 amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Low Duty Mode Patterns (15.4.10)

Andreas Maeder, Linghang Fan, Nader Zein, Tetsu Ikeda

NEC

# Motivation

This contribution proposes text for the Draft Amendment [1] Section 15.4.10 on the Femtocell BS Low Duty Mode. The superposition of available and unavailable intervals at the MSs due to sleep and idle mode form an activity pattern at the Femto ABS, which needs to be defined. Additionally, a default LDM pattern is proposed in order to avoid problems with scanning, hand-over and network entry. The default LDM pattern defines the frame instances where the Femto ABS is guaranteed available for radio operation.

# Proposed Text

-------------------------------------------------- Start of the proposed Text --------------------------------------------------

15.4.10 Low-duty Operation Mode

#### 15.4.10.1 General description

Besides the normal operation mode, Femtocell BSs may support low-duty operation mode, in order to reduce interference to neighbor cells. The low-duty operation mode consists of available intervals and unavailable intervals. During an available interval, the Femtocell BS may become active on the air interface for synchronization and signaling purposes such as paging, ranging or for data traffic transmission opportunities for the MSs.

During an unavailable interval, it does not transmit on the air interface. Unavailable interval may be used for synchronization with the overlay macro BS or measuring the interference from neighbor cells. The Femtocell BS may enter low-duty operation mode either if all MSs attached to the Femtocell BS are in idle or sleep mode, or if no MS is in the service range of the Femtocell BS at all.

#### 15.4.10.x LDM Patterns

A Sequence of availability and unavailability intervals forms an LDM pattern. The Default LDM pattern is the iteration of one available interval and one unavailable interval. Once a Femto ABS enters low duty mode, the Default LDM pattern is activated and the information on the pattern of the Default LDM is provided to AMSs so that they can scan and access the Femtocell BS timely.

The patterns of low-duty cycle of Femtocell BSs may be synchronized according to the subscriber group type (CSG-Open Femtocell BS, CSG-Closed Femtocell BS) of Femtocell BSs. The available interval of CSG-Open Femtocell BS may be more frequently occurred than the available of the CSG-Closed Femtocell BS.

The ABSs broadcast/unicast the information on the parameters of the Default LDM to the AMSs in their service coverage for each FA.

The Default LDM pattern parameters include:

- Available interval (in units of superframes)

- Unavailable interval (in units of superframes)

- Start superframe number

The Femto ABS in low duty mode becomes active on the air interface during the listening intervals of connected AMS in sleep mode, availability interval for paging, and available interval in Default LDM pattern. The Default LDM pattern is a subset of the active intervals at the Femto ABS. During an active interval, the Femto ABS supports all radio interface functionalities.

The Femto ABS may signal additional information on the active intervals to the connected AMS in order to support unsolicited transmissions.

-------------------------------------------------- End of the proposed Text ---------------------------------------------------

# References

[1]	IEEE P802.16m/D1 Draft