| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic Resource Transaction in Self-Organizing Femtocell Network (SON) |  |
| Date Submitted | 2009-09-14 |  |
| Source(s) | Guan-Yu Lin, Hung-Yu Wei National Taiwan University Yih-Shen Chen, I-Kang Fu, Paul Cheng MediaTek Inc. | b93901086@ntu.edu.tw hywei@cc.ee.ntu.edu.tw yihshen.chen@mediatek.com IK.Fu@mediatek.com |
| Re: | IEEE 802.16m-09/0037, “Call for Contribution on Project 802.16m Amendment Content” - Support for Self Organizing Networks (SON) (SDD Section 16) |  |
| Abstract | The SON support for femtocell has been considered in TGm SDD. However, the P802.16m/D1 draft standard has not provided the implementable details. This contribution suggests defining general signaling flow as the common assumptions for relative femto-SON proposal development. |  |
| Purpose | To be discussed and adopted by TGm into P802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamic Resource Transaction in Self-Organizing Femtocell Network

Guan-Yu Lin, Hung-Yu Wei

NTU

Yih-Shen Chen, I-Kang Fu

MediaTek Inc.

- Introduction

Femtocell, known as a home base station, is developed to enhance indoor coverage by reusing the licensed spectrum as part of the network infrastructure. As an example, WiMAX Femtocell Access Point (WFAP) serves mainly for indoor services, has very small coverage (e.g. < 35m), reuses the same air interface and is operated at the licensed spectrum as macro cells. It is highly possible that many femtocells are deployed in a small area, especially in urban or residential area.

As described in section 16 of SDD [1], it is recommended to add Self Organization Network(SON) functions for BSs to automate self configuration and self organization in .802.16m networks. In self configuration phase, it is to initialize and configure BSs automatically with minimum human intervention, which includes Cell Initialization and Neighbor Discovery issue. In self optimization phase, it is to analyze the reported SON measurement from the BS/MS and tune the BS parameters for better network performance, which considers Coverage and Capacity Optimization, Interference Management and Optimization, and Load Management and Balancing issue. In section 16.2.3 of SDD, it describes that:

The load of the cells, modification of neighbor lists, and the selection of alternative carriers should be automatically managed through inter-BS communication and the SON server.

The two phases above involve in network signaling and resource management. In this contribution, we propose two types of framework for inter-BS resource transaction (direct inter-BS communication and BS communication through FON) to satisfy the two requirements. The former proposes a framework for direct resource truncation between BSs, while the latter manages resource with SON server.  Messages formats are proposed accordingly.

- Dynamic Resource Transaction

Messages for DRT (Dynamic Resource Transaction) are proposed, which includes seven message types:

  - DRT_Request (type=0000): The message is sent by a BS to ask for specific resource blocks from neighboring BS.

  - DRT_Grant (type=0001): The message is sent by a BS to reply for the received DRT_Request.

  - DRT_Release (type=0010): The message is sent by a BS to announce the permanent release of specific resources.

  - DRT_Rent (type=0011): The message is sent by a BS announce the temporary release of specific resources.

  - DRT_Info (type=0100): The message is sent in inter-BS and BS/AMS negotiation to exchange resource and interference information.

  - DRT_Req_to_Release (type=0101): The message is sent by a BS to request a specific neighboring BS to stop from using specific radio resource blocks.

  - DRT_Reserved_resource_notification (type=0110): The message is sent by a macrocell BS to inform femtocell BS of specific reserved resource. Note that, the message is for macrocell BS only.

  - DRT_ACK (type =0111): The message is sent by a BS to reply for the received DRT_Grant and DRT_Reserved_resource_notification.

  - DRT_Req_to_Exchange (type=1000) : The message is sent by a BS to request a specific neighboring BS to exchange resource. The message should specify the resource blocks to be exchanged.

With the seven signaling message types, the radio resource can dynamically be reallocated between femto-to femto and macro-to-femto BS with corresponding signaling flow.

- Application Scenarios

To implement these concepts in section 16 of SDD, we propose an inter-BS resource transaction method. The method not only provides a platform for neighbor environment report and inter-BS signaling but is useful for self optimization in negotiating the usage and scheduling of radio resource among BSs. Namely, the dynamic resource transaction method has potential for some useful applications.

The first application is interference mitigation. The basic principle to implement interference avoidance with this method is that each BS avoids using high interference blocks and reduces the interference to other BSs. The former can be achieved by requesting or exchanging for alternative blocks, and the latter can be done by the interference report of other BSs. This method is especially useful for a newly deployed BS to select carrier frequency and channel bandwidth with acceptable interference level. For example,

- BSs request low interference radio blocks so as to cause low interference to/from other cells.

- BSs release radio blocks which suffer severe interference or may cause severe interference to others. The latter case may need interference alerting and report to make the decision.

- BSs exchange radio resource blocks to get low interference blocks and release high interference blocks. Note that since interference varies from one location to the other location, one radio resource block may be poor to one but good to others.

These operations can do interference mitigation so as to improve resource utilization.

The second application is resource management during femtocell/macrocell handover by resource reservation. For example, we design that high speed MS will not handover into the cell with small coverage. Furthermore, with the dynamic resource transaction method (DRT method), inter- BS reserved resource region is pre-negotiated, then macrocell BS schedule high speed MS be served by that negotiated resource region. After detecting the high speed MS served by macrocell BS and being approaching Femto BS’s coverage area, FemtoBS should reschedule its subordinated MSs not be served by that negotiated resource region. Based on DRT method, this operation can be easily implemented and it not only solves frequent handover problem but reduce the interference to high-speed MS.

- Text Proposal

------------------------------------------------------- Start of the Text------------------------------------------------------------

[Adopt the following text modifications into P802.16m/D1]

15.6 Self Organization Network

Self Organizing Network (SON) functions can automate the configuration of ABS parameters for optimizing network performance, coverage and capacity. The air interface support for SON functions is to perform measurement/reporting of air interface performance metrics and the subsequent adjustments of ABS parameters.

The SON server is assumed be available in backhaul network when SON functions are enabled. The SON server can exchange information among different ABSs through backhaul network for inter-ABS coordination.

15.6.x Support of Interference Mitigation

When supporting the femtocell interference mitigation by radio resource blocking, the SON server shall be capable to instruct femto ABS the usage of the allocated radio resource region. The usage indication includes the permanent blocking, conditional blocking and resume.

The femto ABS shall not schedule data transmission over the radio resource regions which are marked as permanent blocking. The femto ABS may schedule data transmission over the radio resource regions which are marked as conditional blocking unless the condition is met. The condition may be a predefined interference level. A blocked radio resource region is allowed for data transmission after it is marked as “resume”.

------------------------------------------------------- End of the Text------------------------------------------------------------

Appendix:  Examples on Message Format Design (informative)

X Dynamic Resource Transaction

Dynamic resource transaction (DRT) is a framework for self-organizing Network (SON) in femtocell deployments. The goal is to provide a platform for SON signaling and its self optimization. In an overlay Macro-Femto network architecture, DRT framework facilitates adaptive radio resource allocation between BSs, where the usage scenario may be macro-to-macro, femto-to-macro,or femto-to-femto.

X.1 DRT Message

The message types of DRT framework is defined as follows:

  - DRT_Request (type=0000): The message is sent by a BS to ask for specific resource blocks from neighboring BS.

  - DRT_Grant (type=0001): The message is sent by a BS to reply for the received DRT_Request.

  - DRT_Release (type=0010): The message is sent by a BS to announce the permanent release of specific resources.

  - DRT_Rent (type=0011): The message is sent by a BS announce the temporary release of specific resources.

  - DRT_Info (type=0100): The message is sent in inter-BS and BS/AMS negotiation to exchange resource and interference information.

  - DRT_Req_to_Release (type=0101): The message is sent by a BS to request a specific neighboring BS to stop from using specific radio resource blocks.

  - DRT_Reserved_resource_notification (type=0110): The message is sent by a macrocell BS to inform femtocell BS of specific reserved resource. Note that, the message is for macrocell BS only.

  - DRT_ACK (type =0111) : The message is sent by a BS to reply for the received DRT_Grant and DRT_Reserved_resource_notification.

  - DRT_Req_to_Exchange (type=1000) : The message is sent by a BS to request a specific neighboring BS to exchange resource. The message should specify the resource blocks to be exchanged.

X.2 Signaling Flow of DRT For Direct Inter-BS communication

The DRT operations are as follows:

X.2.1 Rent/Release resource

When a BS (named “giver BS”) has available resources, it sends DRT_Rent to inform their neighboring or overlaid BS of temporary availability of specific resources. When the giver BS intends to release the resource permanently, DRT_Release is sent. The DRT_Rent and DRT_Release message should specify related interference information of these unused resources. After receiving the message, the BS requiring extra resource (named “acceptor BS”) should send DRT_Request back to claim its request. Upon receiving DRT_Request, the giver BS responds with DRT_Grant. If the giver BS only agrees to release or rent part of its available resources, then the agreed blocks should be specified in DRT_Grant. Upon receiving DRT_Grant, acceptor BS should return an DRT_ACK. When giver BS receives the DRT_ACK, the negotiation is done.

X.2.2 Request for resource

For an acceptor BS requiring extra resource allocation, it can send DRT_Request, in which the desired radio block region (and/or required SINR value) is specified. After receiving the DRT_Request , the giver BS sends a DRT_Grant back to specify the available resource region that satisfying acceptor BS’s requirement. The acceptor BS then returns an ACK to obtain the ownership of the blocks.

X.2.3 Request to release resource

For a BS (named “originator BS”) requesting for allocation suspension of some radio blocks in the neighboring cells, it sends a DRT_Req_to_Release message to the target BS. Upon the reception of the message, if the target BS determines to accept the request, it stops the allocation of the specified resources. The target BS sends a DRT_Grant back to respond the request. The originator BS returns an ACK to acknowledge the grant message.

X.2.4 Negotiation of resource reservation

Two approaches can be applied to adjust the size of resource reservation between femtoBS and macroBS. The first approach is to directly assign part of macro’s own resource as reserved resource, where no explicit signaling message between macroBS and femtoBS is required. The second approach is to assign femtoBS’ resource blocks as reserved resource. Before the assignment, macroBS should first obtain resource from femtoBS by operations showed in Figure 1 or Figure 2.. After getting the ownership of desired resource blocks, macroBS then applies the first approach to increase the size of resource reservation. After the adjustment of reserved resource size, marcoBS should notify femtoBS the allocation change of reserved resource by DRT_ Reserved_resource_notification message.

X.2.5. Exchange resource

When a originator BS wants to exchange resource with its neighboring BS (target BS), it sends DRT_Req_to_Exchange to specify the desired resource blocks to be exchanged. After receiving DRT_Req_to_Exchange , the target BS decides whether to exchange resource and reply DRT_Grant to agree or disagree the request. On the reception of DRT_Grant , the originator BS returns an ACK to acknowledge the grant message. The operation is shown in Figure 4.

The corresponding signaling flows are shown as follows. Note that the giver-acceptor BS pair and target/originator pair can be femto BS-to-femto BS , marco BS-to-femto BS,or femto BS to marco BS .

DRT(type=0111)

DRT(type=0111)

DRT(type=0010,0011)

DRT(type=0001)

DRT(type=0000,0101)

Acceptor/

originator BS

Giver/

target BS

Figure 1. Acceptor-active Signaling Flow

DRT(type=0001)

DRT(type=0000)

Acceptor BS

Giver BS

Figure 2. Giver-active Signaling Flow

DRT(type=111)

DRT(type=0010,0011)

DRT(type=0001)

DRT(type=0000)

DRT(type=0110)

DRT(type=0111)

Macro BS

Femto BS

Figure 3. Signaling Flow for Resource Reservation

DRT(type=0111)

DRT(type=0001)

DRT(type=0000)

DRT(type=0110)

Macro BS

Femto BS

DRT(type=111)

DRT(type=0111))

DRT(type=0111)

DRT(type=0111)

DRT(type=0001)

DRT(type=1000)

originator BS

target BS

Figure 4. Signaling Flow for Resource Exchange

X.3 Signaling Flow of DRT Through SON Server

In the DRT signaling flow based on SON server infrastructure, the message functions are the same as that as described in X.2. However, the message from originator BS is first sent to the SON server, and SON server sends needed message to the destination BS. Namely, SON server controls all the inter-BS communication messages.

DRT(type=0111)

DRT(type=0001)

DRT(type=0000,0101)

Acceptor/

originator BS

Giver/

target BS

Figure 5. Acceptor-active Signaling Flow for DRT_Request and DRT_Req_to_Release

SON

Server

DRT(type=0111)

DRT(type=0010 , 0011)

DRT(type=0001)

DRT(type=0000)

Acceptor BS

Giver BS

SON

Server

Figure 6. Giver-active Signaling Flow for DRT_Request and DRT_Req_to_Release

DRT(type=111)

DRT(type=0111)

DRT(type=0111))

DRT(type=0010,0011)

DRT(type=0111)

DRT(type=111)

DRT(type=0001)

DRT(type=0000)

DRT(type=0110)

DRT(type=0111)

Macro BS

Femto BS

Figure 7. Signaling Flow for Resource Reservation

DRT(type=0001)

DRT(type=0000)

DRT(type=0110)

Macro BS

Femto BS

SON Server

SON Server

DRT(type=0111)

DRT(type=0001)

DRT(type=1000)

originator BS

SON

Server

Figure 8. Signaling Flow for Resource Exchange

target BS

X.4 Trigger Condition for DRT operation

X.5 Dynamic Resource Transaction Message

Table xxx DRT message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DRT _Message_format() { | - | - |
| Sender BS ID | 8 | - |
| Receiver BS ID | 8 | It may be macrocell/femtocell ID |
| Transaction Request Type | 3 | 0000 : DRT_Request 0001 : DRT_Grant 0010 : DRT_ Release 0011 : DRT_Rent 0100 : Request to Release 0101 : DRT_Info 0110:: DRT_Reserved_resource_notification 0111 : DRT_ACK 1000 : DRT_Req_to_Exchange |
| If (Transaction Request Type==0000){ | - | - |
| Specific Requested Radio Resource | TBD | Specify the desired radio resource |
| } | - | - |
| If (Transaction Request Type==0010,0011){ | - | - |
| Number_of_Unit_Transaction_Radio_Resource (N_UTRR) | TBD | Depend on radio resource planning and interference level |
| For (i=0; i< N_UTRR; i++) { | - | - |
| Interference Information | TBD | Provide the interference strength of unit radio resource so that the receiver can decide how to do transaction. |
| } | - | - |
| If (Transaction Request Type==0001){ |  |  |
|  | TBD | Reply agreement or disagreement message to resource requester. |
| } |  |  |
| If (Transaction Request Type==0100){ |  |  |
| Specific Stop-to-Use Radio Resource |  | Specify the radio resource that induces high interference. The message will be sent to the interference source BS. |
| } |  |  |
| If (Transaction Request Type==0101){ | - | - |
| Interference Source | TBD | Report the interference source ID if the ID causes high interference.(The message may be sent directly to the interference ID or to the macrocell). |
| } | - | - |
| If (Transaction Request Type==0101){ | - | - |
| Reserved Resource Allocation | TBD | Notify the reserved resource allocation |
| } |  |  |
| If (Transaction Request Type==0111){ |  |  |
| Acknowledge received DRT_Grant |  | Acknowledge received DRT_Grant |
| } | - | - |
| If (Transaction Request Type==1000){ |  |  |
| Resource exchange | TBD | Specify the target BS ID, resource blocks to be exchanged. |
| } | - | - |
| } |  |  |

Reference

[1] IEEE 802.16m-09/003r9a, “IEEE 802.16m System Description Document”