| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Ranging Preamble Codes for Non-Synchronized AMSs in IEEE P802.16m/D1 (15.3.9.2.4.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon LG Electronics Hwasun Yoo, Si-Hyun Park Samsung Electronics Co., Ltd. | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com hwasun.yoo@samsung.com |
| Re: | IEEE 80216m-09/0037, “Call for Contributions on Project 802.16m Amendment Content” P802.16m/D1 : 15.3.9.2.4.2 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Ranging Preamble Codes for Non-Synchronized AMSs in IEEE P802.16m/D1 (15.3.9.2.4.1)

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, and Young-Hyoun Kwon

LG Electronics

Hwasun Yoo, Si-Hyun Park

Samsung Electronics Co., Ltd.

# 1. Introduction

This contribution proposes the text of ranging channel section to be included in IEEE P802.16m/D1 [1].

# 2. Proposed Changes

This contribution proposes the text of ranging channel for clarification.

- The current equations for rp and sp are a little bit complicated. Modify it a simple formula without any changes of its principle/results.

- It is needed to describe the code partitioning exactly between initial and handover ranging codes.

- It is needed to describe the root index r0, not rp, signaling by SFH.

# Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” July 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in the subclause 15.3.9.2.4.2 of [1]

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

[Modify the following text in the 15.3.9.2.4.2]

# 15.3.9.2.4.1.        Ranging channel for non-synchronized AMSs

# Ranging Preamble codes

The Zadoff-Chu sequences with cyclic shifts are used for the ranging preamble codes. The pth ranging preamble code xp(k) is defined by

(254)

where

p	is the index for pth ranging preamble code which is made as the spth cyclic shifted sequence from the root index rp of Zadoff-Chu sequence.

(255)

(256)

where rns0 is broadcasted in the S-SFH and  Mns is the number of cyclic shifted codes per ZC root index according to Table 737. NTOTAL is the total number of initial (0~NIN -1) and handover preamble codes (NIN ~NIN +NHO -1) per sector.

NCS	is the unit of cyclic shift according to the cell size. It is defined by  where Mns is the number of cyclic shifted codes per ZC root index according to Table 737.

NRP	is the length of ranging preamble codes defined as NRP =139 for ranging channel Format 0 in Table 721 and NRP =557 for ranging channel Format 1 in <<Table 721732>>.

The number of cyclic shifted codes per root index (Mns), the start root index of ZC code (rprns0), and the ranging preamble code partition information are broadcasted by S-SFH. The number of cyclic shifted codes per root index is defined in Table 737. The start root index of ZC code is defined by rns0=4×kns+1 (=1, 5, 9, …, or 65) where kns is a cell specific value broadcasts through S-SFH. The ranging preamble code partition information indicate the number of initial and handover ranging preamble codes and is defined in Table 738.

Ranging channel configurations

Table 737 - The number of cyclic shifted codes per ZC root index, Mns

| index | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Mns | 1 | 2 | 4 | 8 |

Table 738 - Ranging preamble code partition information table, NIN and NHO

| Partition Index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of initial ranging preamble codes, NIN | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 | 24 | 24 | 24 | 24 | 32 | 32 | 32 | 32 |
| Number of handover ranging preamble codes, NHO | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 | 8 | 16 | 24 | 32 |

Ranging channel configurations

The information for ranging time resource allocation is indicated by the S-SFH in a regular allocation. ….

…. [Maintain all texts under Table 738]….

------------------------------------------------------------------------- Text End ---------------------------------------------------------------------------