| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Physical Ranging Channel for Synchronized AMSs in IEEE P802.16m/D1 (15.3.9.1.4.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, Young-Hyoun Kwon LG Electronics Hwasun Yoo, Si-Hyun Park Samsung Electronics Co., Ltd. Pei-Kai Liao, Yih-Shen Chen and Paul Cheng MediaTek Inc. Yan-Xiu Zheng ITRI Xinrong Wang, Yang-Seok Choi Intel Corporation | Voice: +82-31-450-7902 E-mail: camille@lge.com, samji@lge.com, hgcho@lge.com hwasun.yoo@samsung.com pk.liao@mediatek.com, yihshen.chen@mediatek.com zhengyanxiu@itri.org.tw xinrong.wang@intel.com, yang-seok.choi@intel.com |
| Re: | IEEE 80216m-09/0037, “Call for Contributions on Project 802.16m Amendment Content” P802.16m/D1 : 15.3.9.1.4.2 |  |
| Abstract | This contribution proposes the text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Physical Ranging Channel for Synchronized AMSs in IEEE P802.16m/D1 (15.3.9.1.4.2)

HyunWoo Lee, Jin Sam Kwak, HanGyu Cho, and Young-Hyoun Kwon

LG Electronics

Hwasun Yoo, Si-Hyun Park

Samsung Electronics Co., Ltd.

Pei-Kai Liao, Yih-Shen Chen and Paul Cheng

MediaTek Inc.

Yan-Xiu Zheng

ITRI

Xinrong Wang, Yang-Seok Choi

Intel Corporation

# 1. Introduction

The ranging channel structure for synchronized AMSs has not been defined yet. This contribution proposes the text of ranging channel section to be included in IEEE P802.16m/D1 [1].

# Reference

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” July 2009.

[2] IEEE C802.16m-09/2006, “Proposed Changes to the Physical Resource Mapping of Ranging Channel for Synchronized AMSs in IEEE P802.16m/D1 (15.3.9.2.4.2),” August 2009.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: current text in the subclause 15.3.9.1.4.2 of [1]

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

[Modify the following text in the 15.3.9.1.4.2]

# Ranging Channel Structure for synchronized AMSs

The ranging channel for synchronized AMSs is used for periodic ranging. The AMSs that are already synchronized to the target ABS are allowed to transmit the periodic ranging signal. For femtocell, the ranging channel for synchronized AMSs can be used for initial ranging, handover ranging, and periodic ranging.

The physical structure in the ranging channel for synchronized AMSs occupies 72 subcarriers by K OFDMA symbol time. For femtocell, K is equal to 1 and the rest of subcarriers can be used for data transmission.

Figure (f.1) illustrates the basic unit of ranging channel structure in the time domain where Tg and Tb are defined in 15.3.2.4.

- The basic unit

- An example in the type-1 subframe

Figure f.1 – Ranging channel structure for synchronized AMSs in the time domain

Power control operation described in subclause 15.3.9.4.3 applies to ranging signal transmission.

------------------------------------------------------------------------- Text End ---------------------------------------------------------------------------