| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized Text Proposal on Femtocell Reliability |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Inuk Jung, Jin Lee, Ronny Yongho Kim, Kiseon Ryu LG Electronics I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng MediaTek Inc. Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha ETRI Byung Moo Lee, Young-Chul Oh, Jeong-Hwi Kim, Jong-Sik Lee and Seong-Choon Lee KT Sung-Yeop Pyun, Yong Jin kwon, Helena W and Dong-Ho Cho KAIST Howon Lee and Bang Chul Jung KI ITC Ying Li, Zhouyue Pi, Baowei Ji, Anshuman Nigam, Jung Je Son Samsung Electronics Markus Mueck Infineon | cooper@lge.com IK.Fu@mediatek.com sjjung@etri.re.kr bmlee73@gmail.com psy83@comis.kaist.ac.kr hwlee81@gmail.com yli2@sta.samsung.com MarkusDominik.Mueck@infineon.com |
| Re: | Call for comments for Letter Ballot 30 / Topic: Femto Reliability |  |
| Abstract | Text proposal for Femtocell Reliability |  |
| Purpose | For member’s review and adoption into P802.16m_D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized Text Proposal on Femtocell Reliability

Inuk Jung, Jin Lee, Ronny Yongho Kim, Kiseon Ryu

LG Electronics, Inc.

I-Kang Fu, Kelvin Chou, Yih-Shen Chen, Paul Cheng

MediaTek Inc.

Soojung Jung, Juhee Kim, Eunkyung Kim, Jaesun Cha

ETRI

Byung Moo Lee, Young-Chul Oh, Jeong-Hwi Kim, Jong-Sik Lee and Seong-Choon Lee

KT

Sung-Yeop Pyun, Yong Jin kwon, Helena W and Dong-Ho Cho

KAIST

Howon Lee and Bang Chul Jung

KI ITC

Ying Li, Zhouyue Pi, Baowei Ji, Anshuman Nigam, Jung Je Son

Samsung Electronics

Markus Mueck

Infineon

- Introduction

This document further clarifies procedure to support reliability for femtocell ABS. The baseline text has been adopted from the text included in the current SDD.

- Proposed Harmonized Text

On the 80216m_D1, page 494, add proposed text in 15.4 with the proposed text as follows:

--------------------------------------------------Start of the Text----------------------------------------------------

15.4.13 Femtocell BS Reliability

Femtocell BS shall disable downlink air interface transmitter as soon as the connection with the service provider network is lost for a configurable pre-defined time. In such a case, the femtocell BS should support the mechanisms to ensure service continuity of the AMSs prior to disabling air interface. For example, the BS initiated handover depicted in 15.2.6. When a femtocell BS is going to disable the air interface by instruction or by accident, it shall broadcast an indication of the air interface down through AAI_SON-ADV with Action type 4 for Reason bit 0, specified in section 15.2.6.x, repeatedly until it disables air interface and set cell bar bit to 1 in order to prevent MS entry or reentry from other cells. If handover is to be performed, the indicator may also inform whether the handover will be coordinated or not for AMS to decide what handover procedure it will perform (e.g. uncontrolled handover, coordinated handover etc). The AMS may initiate HO to a BS based on the recommended list given by AAI_SON-ADV message or any previously cached neighbor BS list of its preference.

In this case the femtocell ABS shall store MAC context information of the serving AMSs (e.g. Basic capabilities, Security capabilities, etc.). Such context information allows AMS to perform optimized network reentry when returning back to the femtocell BS upon its recovery.

At the expected uptime, if provided, the AMS may scan the previous femtocell BS if not directed by the network to decide whether to return to it via HO or not.

In the case of Femto ABS resource adjustment due to interference mitigation which may affect some AMS that are originally connected with the Femto ABS, the Femto ABS may send resource-adjustment information to the subordinated AMSs to prevent MSs from out of service using the AAI_SON-ADV with Action Type 4 of Reason bit 1.

Before executing resource adjustment, the Femto ABS may request some subordinated AMSs to perform handover to neighbor cell if required. The MAC message may be unicast/multicast to the AMSs. The Femto ABS should finish HO initiation and HO preparation phases before it does resource adjustment.

When supporting the Femto ABS reliability improvement functions, the Femto ABS is assumed to equip with backup power buffer to ensure the functions can be performed in a period of time when normal power supply is not available. It is also assumed that Femto ABS is capable to learn the backhaul connection status from the modem (e.g. xDSL or DOCSIS) and configure the message/signaling to support the functions identified in this section.

---------------------------------------------------End of the Text----------------------------------------------------

Annex of SON DG draft for AAI_SON-ADV message

15.2.6.x   AAI_SON-ADV message

This message is used by an ABS to broadcast relevant SON information for action types as defined below. Action types 1 and 2 can be sent by any ABS. Action types 3 and 4 are specific to Femto ABS only.

This message contains the following parameters:

- Action type

    - 1st value: ABS Reconfiguration

    - 2nd value: ABS Restart

    - 3rd value: ABS Scanning

    - 4th value: ABS Reliability

- If(Action type == 1st , 2nd , or 3rd value)

    - Unavailability start time (UST)

    - Unavailable Time Interval (UTI) of the ABS

- If(Action type == 4th value)

    - Reason

      - 0b0: Power down

      - 0b1: Power reduction

    - If(Reason == 0b1) dB value of Tx power reduction

    - Expected down time

    - Expected uptime

    - Recommended BSID list