| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal on Reliable HO procedure |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim LG Electronics | [cooper, ksryu, ronnykim]@lge.com |
| Re: | Call for comments for Letter Ballot 30 / Topic: 6m HO |  |
| Abstract | Text proposal for Handover |  |
| Purpose | For member’s review and adoption into P802.16m_D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal on Reliable HO procedure

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim

LG Electronics, Inc.

- Introduction

This document captures the proposed text on the Handover Section. The objective of this contribution is to amend ambiguous HO signaling procedure that may cause the AMS or the ABS to misleadingly proceed in action upon HO message loss.

- Motivation and Problem Statement

In case a HO message has been lost without any acknowledgment of the receiver side, the HO procedure can go wrong, leaving the ABS and AMS to proceed differently in unsynchronized manner. This is no problem during the HO execution phase. However, upon a BS initiated HO, if the AAI_HO-CMD is sent but not received by AMS without any acknowledgment, the AMS will be served until disconnect time, but will not be allocated any DL/UL resource after that. AMS will not know what happened and such state is not described in the specification.

HARQ with Local NACK may be used to cover such message loss, but its usage is optional and also is assumed to be not easy to implement, so this may not always prevent the loss of AAI_HO-CMD message.

In case of MAC level ACK, the assumption is that such MAC level ACK will be mandatory, which may worsen the case of AAI_HO-CMD loss. In other words, the BS will re-transmit the AAI_HO-CMD when no ACK has been received by AMS, which also implies a new setting for action/disconnect time or any dedicated CDMA ranging (SBS-TBS negotiation) etc. Furthermore, in case the disconnect time may expire during such 2/3 way acknowledgement exchange. In such case, shall the AMS just perform network re-entry while S-ABS continuously retransmitting AAI_HO-CMD until HO-FAIL is received by T-ABS via backhaul?

Such case is shown in Fig 1.

Figure 1. Unreliable HO state call flow in case of AAI_HO-CMD loss

- Proposal of HO Reliability

As mentioned above, we propose a simple way to avoid such complicated ACK procedure. Fig. 2 describes the HO procedure with the proposed HO reliability procedure.

Figure 2. HO call flow with reliability support

Since the ambiguity originates from how to act upon expiry of disconnect in case AMS did not successfully receive AAI_HO-CMD from ABS, the ABS shall assume that AMS has successfully received the AAI_HO-CMD and proceed as normal (stop DL/UL allocation after disconnect time).

If AMS did not successfully receive the AAI_HO-CMD message, the T-ABS will report HO-FAIL to S-ABS after the ‘HO ranging initiation deadline’ expires. However, during this period, the AMS will experience interruption.

Upon receiving HO-FAIL from T-ABS, the S-ABS will check the presence of AMS via the ‘AAI_CHK-REQ’ (AMS Check Request). Upon receiving the ‘AAI_CHK-RSP’ (AMS Check Response) the AMS responds with ‘AMS presence check signal-RSP’ which allows the AMS to resume data exchange and further HO coordination.

- Proposed Text

On the 80216m_D1, page 97 line 27, add proposed text in 15.2.5.10.4 as follows:

--------------------------------------------------Start of the Text----------------------------------------------------

15.2.5.10.4 HO Execution

…

If the AAI_HO-CMD message includes no target ABS, or if all the target ABSs that are included in the AAI_HO-CMD message are unreachable as defined above, the AMS shall inform the serving ABS of it's preferred target ABS by sending the AAI_HO-IND message with code 2 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with code 2, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AAI_HO-CMD message includes no target ABS, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS.

If AMS does not successfully receive the AAI_HO-CMD message in case of ABS initiated HO, the S-ABS proceed as usual but shall seek the presence of AMS either upon receiving HO-FAIL notification from the T-ABS or upon the expiration of the ‘HO ranging initiation deadline’ timer. Upon either condition triggered, S-ABS shall transmit AAI_CHK-REQ message to which the AMS shall response with AAI_CHK-RSP message. If S-ABS does not receive the response message, it shall assume this as a HO drop case.

An AMS may request bandwidth for the residual data in the buffer before the expiration of Action Time. The serving ABS may send information about any unallocated requested bandwidth to the target ABS over the backhaul so that the target ABS may allocate uplink resource immediately after receiving the dedicated ranging code from the AMS or after Action Time if CDMA ranging is omitted.

---------------------------------------------------End of the Text----------------------------------------------------