| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Network Entry MAC Management Messages (15.2.6.1) |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Hyunjeong Kang, Youngkyo Baek Samsung Electronics | hyunjeong.kang@samsung.com |
| Re: | IEEE 802.16m-09/0044, “IEEE 802.16 Working Group Letter Ballot #30: Announcement” |  |
| Abstract | The contribution proposes the text to suggest necessary parameters of network entry MAC management messages. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Network Entry MAC Management Messages (15.2.6.1)

Hyunjeong Kang, Youngkyo Baek

Samsung Electronics

- Introduction

This contribution proposes parameters of MAC management messages used during network (re-)entry.

In the P802.16m/D1 [1], some parameters for AAI_RNG-REQ/RSP, AAI_SBC-REQ/RSP and AAI_REG-REQ/RSP messages. But additional parameters are supposed to be included in these messages to support the network entry procedures in [1].

The security capability parameters are proposed in AAI_SBC-REQ/RSP messages; those parameters are necessary before PKM transaction. The proposed parameters are the features of capability class defined in a contribution [2] that was accepted in session #63 but not implemented in [1].

AAI_RNG-REQ/RSP messages may include AMS’s paging information such as Paging Cycle, Paging Offset, Temporary ID and Paging group ID when an AMS performs network re-entry or location update. In current [1], only temporary ID is defined in the AAI_RNG-REQ/RSP message. The remaining paging information i.e., Paging Cycle, Paging Offset and Paging group ID are proposed.

Reference

[1] P802.16m/D1, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, July 2009

[2] IEEE C802.16m-09/1432r2, Fast Device Capability Negotiation for IEEE 802.16m System

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Modify the Table 682-parameters for AAI_RNG-REQ as follows.]

| Name | Value | Usage |
| --- | --- | --- |
| MS_Random | AMS generates a random number MS_Random which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. | It shall be included when the AMS is attempting network entry without its STID which the ABS assigns |
| MAC version | Version number of IEEE 802.16 supported by the AMS. |  |
| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, in combination with a serving BSID, it indicates that the AMS is currently attempting unprepared HO. If Bit#1 is set to 1, in combination with a paging controller ID, indicates that the AMS is attempting network reentry from idle mode to the ABS. If Bit#2 is set to 1, it indicates that the AMS is currently attempting prepared HO. If Bit#3 is set to 1, it indicates that the AMS is initiating the idle mode location update process. Bit#4: ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. Bit#5-7 : reserved. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| Serving BSID | The BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| Paging Controller ID | The Paging Controller ID which the AMS currently maintains in idle mode. | It shall be included when the AMS is attempting to perform reentry or location update |
| Temporary ID | The temporary ID which the AMS is assigned for idle mode and currently maintains. |  |
| Deregistration Identifier (DID) | The identification of the paging group that the AMS is previously belonging to. The ID is composed of temporary identifier, paging cycle and paging offset. |  |
| Paging cycle change | Paging cycle requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update. |
| Power Down Indicator | Indicates the AMS is currently attempting to perform location update due to power down. | It shall be included when the AMS is attempting to perform location update due to power down |
| CMAC_KEY_COUNT | The AMS’s current value of the CMAC_KEY_COUNT, which is used to generate the security keys in the target ABS. | It shall be included during re-entry, secure Location Update or HO |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, if the AMS has a CMAC tuple necessary to expedite security authentication. |

[Modify ‘Paging Information’ on Page 51, Line 6 in Table 683 – parameters for AAI_RNG-RSP as follows.]

| Paging Information | Paging Cycle | New Paging Cycle of AMS 0b0000 – 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed. |
| --- | --- | --- | --- |
|  | Paging Group ID | New PGID of AMS |  |
|  | Paging Offset | New PAGING_OFFSET of AMS |  |

Temporary ID

The temporary ID which the AMS is assigned for idle mode and currently maintains.

[Add the sentence in subclause 15.2.6.3 AAI_SBC-REQ after line 44 on Page 51]

| Security negotiation parameters | Authorization policy and MAC encryption support | EAP-based authorization is supported. |
| --- | --- | --- |
|  | PN Window Size | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks. Bit #0= 1 Bit #1 = 8 Bit #2 = 16 Bit #3 = 32 Bit #4 = 64 Bit #5 = 128 |
|  | Auth type for EAP | Auth Type for EAP shall only be included when EAP-based authorization is supported. Bit #0: device authentication support Bit #1: user authentication support |

[Add the sentence in subclause 15.2.6.4 AAI_SBC-RSP after line 54 on Page 51]

| Security negotiation parameters | Authorization policy and MAC encryption support | EAP-based authorization is supported. |
| --- | --- | --- |
|  | PN Window Size | Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks. Bit #0= 1 Bit #1 = 8 Bit #2 = 16 Bit #3 = 32 Bit #4 = 64 Bit #5 = 128 |

-------------------------------  Text End  ---------------------------------------------------