| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on CMAC calculation and reducing size of ICV part(15.2.5.3) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Youngkyo Baek Jicheol Lee Samsung Electronics | E-mail: youngkyo.baek@samsung.com Phone : +82-31-279-7321 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30, on P802.16m/D1 |  |
| Abstract | This contribution proposes CMAC calculation function and reducing the size of ICV part based on some rationale. |  |
| Purpose | Accept the proposed specification changes on IEEE 802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on CMAC calculation and reducing size of ICV part (15.2.5.3)

Youngkyo Baek, Jicheol Lee

Samsung Electronics

# Introduction

This contribution proposes CMAC calculation and reducing the size of ICV part.

CMAC digest/tuple is attached to a MAC management message in order to authenticate the received MAC management message. According to the IEEE Std 802.16-2009[2], CMAC construction follows the NIST Special Publication 800-38B [3] and use the LSB 64bits of output of AES-CMAC algorithm as a CMAC value. We suggest using AES-CMAC algorithm similarly to IEEE Std 802.16-2009[2], but NIST Special Publication 800-38B [3] recommends the length of CMAC as follows.

“For most applications, a value for Tlen(i.e.the length of CMAC) that is at least 64 should provide sufficient protection against guessing attacks. A value of Tlen that is less than 64 shall only be used in conjunction with a careful analysis of the risks of accepting an inauthentic message as authentic.

In particular, a value of Tlen smaller than 64 should not be used unless the controlling protocol or system sufficiently restricts the number of times that the verification process can return INVALID, across all implementations with any given key. Tlen should satisfy the following inequality:

Tlen ≥ lg(MaxInvalids / Risk),

Where 1) ‘MaxInvalids’ is a limit on the number of times that the output is the error message INVALID before the key is retired, across all implementations of the verification process for the key , and 2) ‘Risk’ is the highest acceptable probability for an inauthentic message to pass the verification process.”

Hence if we assume that MaxInvalids = 212 and Risk = 2-20, then Tlen ≥ 32.

That means if it is guaranteed that CMAC key is updated before 212 invalid CMAC messages are detected then the size of CMAC can be 32bits.

Similarly CCM mode follows the NIST Special Publication 800-38C [4] and use the MSB 64bits of output of CCM algorithm as an ICV part. NIST Special Publication 800-38C [4] recommends the length of ICV as same as CMAC.

Hence, if it is guaranteed that TEK is updated before 212 invalid ICV are detected then the size of ICV part can be 32bits.

# References

[1] IEEE P802.16m/D1, “Part 16: Air Interface for Broadband Wireless Access Systems(Advanced Air Interface)”, July. 2009.

[2] IEEE Std 802.16-2009, “Part 16: Air Interface for Broadband Wireless Access Systems”,MAY.2009

[3] NIST Special Publication 800-38B,” Recommendation for Block Cipher Modes of Operation: The CMAC Mode for Authentication”, MAY 2005.

[4] NIST Special Publication 800-38C,” Recommendation for Block Cipher Modes of Operation: The CCM Mode for Authentication and Confidentiality”, MAY 2004.

# Proposed Text #1

Add the proposed text to the section 15.2.5.3 as follows.

----------------------------------------------------- Start of Proposed Text#1 ---------------------------------------------------

15.2.5.3.2 Calculation of Cipher-based message authentication code (CMAC)

An ABS or AMS may support management message integrity protection based on CMAC—together with the AES block cipher. The CMAC construction as specified in NIST Special Publication 800-38B shall be used.

The calculation of the keyed hash value contained in the CMAC Digest attribute and the CMAC Tuple shall use the CMAC algorithm with AES. The DL authentication key CMAC_KEY_D shall be used for authenticating messages in the DL direction. The UL authentication key CMAC_KEY_U shall be used for authenticating messages in the UL direction. UL and DL message authentication keys are derived from the AK (see 15.2.5.2.1.4 for details).

The CMAC Digest and CMAC Tuple attributes shall be only applicable to the PKM version 3. In the PKM version 3 protocol, the AKID used in the computation of the CMAC value shall be the 64-bit AKID of the AK from which the CMAC_KEY_* was derived.

The CMAC Packet Number Counter, CMAC_PN_*, is a 4-byte sequential counter that is incremented for each MAC Management Message which contains a CMAC Tuple or CMAC Digest TLV in the context of UL messages by the AMS, and in the context of DL messages by the ABS.

If STID is unknown (e.g., in network reentry situation) then STID 0 should be used. The CMAC_PN_* is part of the CMAC security context and shall be unique for each MAC management message with the CMAC tuple or digest. Any tuple value of {CMAC_PN_*, CMAC_KEY_*} shall not be used more than once. Either the reauthorization process or PMK update without reauthorization should be initiated (by ABS or AMS) to establish a new PMK/AK before the CMAC_PN_* reaches the end of its number space.

CMAC key update should be completed by key agreement 3 way handshakes before messages with invalid CMAC are detected over the MaxInvalid threshold(212) times for the same CMAC key. ABS initiates PMK update by sending PKMv3 Key_Agreement-MSG#1 message to AMS and finally CMAC key is updated. AMS sends PKMv3 CMAC-Invalid message to the ABS and, if received, the ABS initiate PMK update by sending PKMv3 Key_Agreement-MSG#1 message to AMS and finally CMAC key is updated.

The digest shall be calculated over a field consisting of the AKID followed by the CMAC_PN_*, expressed as an unsigned 32-bit number, followed by the 12-bit STID and 4-bit FID on which the message is sent, followed by 16-bit of zero padding (for the header to be aligned with AES block size) and followed by the entire MAC management message with the exception of the CMAC tuple or digest.

The LSBs of the digest shall be truncated to yield a 32-bit length digest. Note: This is different from the recommendation in NIST special publication 800-38B where the MSB is used to derive the CMAC value.

i.e., if CMAC_KEY_* is derived from AK:

CMAC value <= Truncate(CMAC (CMAC_KEY_*, AKID | CMAC_PN |STID|FID|16-bit zero padding | MAC_Management_Message), 32), where AKID = Dot16KDF(AK, 0b0000|AKSN|MSID*|BSID|“AK”, 64).

If the digest is included but the AMS has no assigned STID, e.g., An AAI_RNG-REQ message (e.g., in network reentry situation), then STID 0 and FID 0 should be used.

--------------------------------------------------- End of Proposed Text#1------------------------------------------------------

# Proposed Text #2

Modify the text in the section 15.2.5.3.1.1 as follows.

----------------------------------------------------- Start of Proposed Text#2 ---------------------------------------------------

15.2.5.3.1.1 AES-CCM

PDU payload format

The MAC PDU payload shall be prepended with a 2-bit EKS and a 22-bits PN (Packet Number). The EKS and PN shall not be encrypted. The plaintext PDU shall be encrypted and authenticated using the active TEK, according to the CCM specification. This includes appending an 48-byte integrity check value (ICV) to the end of the payload and encrypting both the plaintext payload and the appended ICV.

The ciphertext message authentication code is transmitted so that byte index 0 (as enumerated in NIST Special Publication 800-38) is transmitted first and byte index 37 is transmitted last (i.e., LSB first). The processing yields a payload that is 711 bytes longer than the plaintext payload.

………………

……………

Receive Processing rules

On receipt of a PDU the receiving AMS or ABS shall decrypt and authenticate the PDU consistent with the NIST CCM specification configured as specified above.

Packets that are found to be not authentic shall be discarded.

Receiving ABS or AMSs shall maintain a record of the highest value PN receive for each SA.

The receiver shall maintain a PN window whose size is specified by the PN_WINDOW_SIZE parameter for SAs. Any received PDU with a PN lower than the beginning of the PN window shall be discarded as a replay attempt. The receiver shall track PNs within the PN window. Any PN that is received more than once shall be discarded as a replay attempt. Upon reception of a PN, which is greater than the end of the PN window, the PN window shall be advanced to cover this PN.

TEK update should be completed before MPDUs with ICV error are detected over the MaxError threshold(212) times for the same TEK. ABS updates TEKs by itself and AMS initiates TEK update by sending a PKMv3 TEK-Invalid message to the ABS.

--------------------------------------------------- End of Proposed Text#2------------------------------------------------------

# Proposed Text #3

Add the proposed text to the section 15.2.6.xxx Privacy key management messages(AAI_PKM-REQ/AAI_PKM-RSP) as follows.

----------------------------------------------------- Start of Proposed Text#3 ---------------------------------------------------

15.2.6.xxx.8. PKMv3 TEK-Invalid message

The ABS transmits the PKMv3 TEK-Invalid message in order for the AMS to send PKMv3 TEK-Request message to the ABS. If the AMS receives the PKMv3 TEK-Invalid message, it shall send the PKMv3 TEK-Request message.

Meanwhile, the AMS transmits the PKMv3 TEK-Invalid message in order to trigger TEK update. If the ABS receives the PKMv3 TEK-Invalid message, it discards current TEKDLE and uses TEKULE as TEKDLE, and derives a new TEK for TEKULE.

Code: 8

Attributes are shown in Table Table xxx.8.

Table xxx.8 PKMv3 TEK-Invalid message attributes

| Attribute | Contents |
| --- | --- |
| SAID | Security association identifier |
| Key Sequence Number | AK sequence number |
| CMAC digest | Message digest calculated using AK. |

The CMAC Digest attribute shall be the final attribute in the message’s attribute list.

15.2.6.xxx.9. PKMv3 CMAC-Invalid message

The AMS transmits the PKMv3 CMAC-Invalid message in order to trigger PMK update. If the ABS receives the PKMv3 CMAC-Invalid message, it sends PKMv3 Key_Agreement-MSG#1 message to the AMS.

Code: 9

Attributes are shown in Table Table xxx.9.

Table xxx.9 PKMv3 CMAC-Invalid message attributes

| Attribute | Contents |
| --- | --- |
| Key Sequence Number | AK sequence number |
| CMAC digest | Message digest calculated using AK. |

The CMAC Digest attribute shall be the final attribute in the message’s attribute list.

--------------------------------------------------- End of Proposed Text#3------------------------------------------------------