| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Management message for ARQ operation (15.2.6) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0044, IEEE 802.16 Working Group Letter Ballot #30 / Topic: Management message (Section 15.2.6) |  |
| Abstract | This contribution proposes clarification texts for ARQ section (15.3.8) |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Management message for ARQ operation

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

In the current IEEE P802.16m/D1 document [1], the messages for proper ARQ operation does not specified. To support exact ARQ operation, this contribution proposes the message format for ARQ. All message contents are almost similar in IEEE P802.16-2009.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Add the following table at line 41 of the table 675 on page 30:]

| Message name | Message description | Connectino |
| --- | --- | --- |
| ●●● | ●●● | ●●● |
| AAI_ARQ-Feedback | Stand-alone ARQ feedback | Basic |
| AAI_ARQ-Discard | ARQ discard | Basic |
| AAI_ARQ-Purge | ARQ Purge | Basic |
| AAI_ARQ_Reset | ARQ Reset | Basic |
| AAI_SLP-REP | Sleep Request | Basic |
| ●●● | ●●● | ●●● |

[Add the following text and table from line 46 on page 86:]

15.2.6.x. AAI_ARQ-Feedback message

An AAI_ARQ-Feedback message is used for receiver to inform the reception status of a number of ARQ blocks or ARQ sub-blocks. ARQ feedback IE or ARQ feedback IE2 (see table 721 and 722) is included in the AAI_ARQ-Feedback message.

Table xxx—AAI_ARQ-Feedback message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_ARQ-Feedback_Message_Format() { | - |  |
| Mangement Message type = xx | 8 |  |
| ARQ_feedback_IE or ARQ_feedback_IE2 | variable | See table 721 and 722 |
| } |  |  |

15.2.6.x+1. AAI_ARQ-Discard message

The transmitter sends this message when it wants to skip a number of ARQ blocks. The procedure in the receiver after receiving an AAI_ARQ_Discard message is described in the 15.3.8.7.3.3.

Table xxx—AAI_ARQ-Discard message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_ARQ-Discard_Message_Format() { | - |  |
| Mangement Message type = xx | 8 |  |
| Flow ID | 4 | See table 721 and 722 |
| SN | 10 | ARQ block SN of the last block in the transmission window that the transmitter wants to discard. |
| Reserved | 2 | Shall be set to zero |
| } |  |  |

15.2.6.x+1. AAI_ARQ-Purge message

The receiver sends this message when it wants to skip a number of ARQ blocks. The ARQ purge procedure is described in the 15.3.8.7.3.4.

Table xxx—AAI_ARQ-Purge message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_ARQ-Discard_Message_Format() { | - |  |
| Mangement Message type = xx | 8 |  |
| Flow ID | 4 | See table 721 and 722 |
| SN | 10 | ARQ block SN of the last block in the reception window that the receiver wants to purge. |
| Reserved | 2 | Shall be set to zero |
| } |  |  |

5.2.6.x+3. AAI_ARQ-Reset message

The transmitter or receiver may send this message. The message is used in a dialog to reset the parent connection’s ARQ transmitter and receiver state machines. The detail ARQ reset procedure is described in the 15.3.8.7.4.

Table xxx—AAI_ARQ-Reset message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_ARQ-Reset_Message_Format() { | - |  |
| Mangement Message type = xx | 8 |  |
| Flow ID | 4 | Corresponding connection ID to perform ARQ reset procedure. |
| Type | 2 | 0b00 = Original message from Initiator 0b01 = Acknowledgment from Responder 0b10 = Confirmation from Initiator 0b11 = Reserved |
| Direction | 2 | 0b00 = UL or DL 0b01 = UL 0b10 = DL 0b11 = Reserved |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”