| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to Table 693 - MIMO feedback modes (15.3.7.2.6.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Sangheon Kim Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes clarifications to the table of MIMO feedback modes in case of operation with the open-loop region. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to Table 693 - MIMO feedback modes (15.3.7.2.6.4)

David Mazzarese et Al.

Samsung Electronics

# Introduction

The operation of MIMO feedback modes depends on the indication of “measurement method indication” in the feedback allocation A-MAP IE, which indicates the usage of the OL region. Referring to Table 691, which defines the types of OL regions, it is clear that we need to specify the MIMO modes and operation parameters for the cases where measurement method indication = 0b1.

According to a decision made in session #62, the STC rate is limited to 4 for MIMO feedback modes 0 and 1, as can been seen in the PFBCH encoding types. This is also clarified in Table 693.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify Table 693 on line 1 page 368 in section 15.3.7.2.6.4 as follows>

Table 693 – MIMO feedback modes

| MIMO Feedback mode | Description and type of RU | Feedback content | Type of RU | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- | --- |
| 0 | OL SU MIMO SFBC/SM (Diversity: DLRU, NLRU) | 1. STC Rate 2. Wideband CQI | Diversity (DRU, Mini-band based CRU) | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate ≥ 2: SM CQI 2 ≤ STC Rate ≤ 4: SM CQI In DLRU: Mt=2 for SM. In Miniband based CSLRU: Mt≥2 for SM | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate = 2: SM CQI In DLRU only. |
| 1 | OL SU MIMO SM (Diversity: NLRU) | 1. STC Rate 2. Wideband CQI | Diversity (Mini-band based CRU) | MIMO mode 1 1 ≤ STC Rate ≤ 4 | MIMO mode 5 STC Rate = 1/2 |
| 2 | OL SU MIMO SM (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband Selection | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 1 1 ≤ STC Rate ≤ 8 | MIMO mode 5 STC Rate = 1/2 |
| 3 | CL SU MIMO (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband PMI 4.Subband selection [5. Wideband PMI] 6. Wideband correlation matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 2 1 ≤ STC Rate ≤ 8 | N/A |
| 4 | CL SU MIMO (Diversity: NLRU) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation matrix | Diversity (Mini-band based CRU) | MIMO mode 2 (Mt=1) | N/A |
| 5 | OL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband Selection 3. Stream indicator | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 3 | MIMO mode 3 |
| 6 | CL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband PMI 3. Subband Selection [4. Wideband PMI] 5. Wideband correlation matrix | Localized (Subband based CRU, Mini-band based CRU) | MIMO mode 4 | N/A |
| 7 | CL MU MIMO (Diversity: SLRU, NLRU) | 1. Wideband CQI [2. Wideband PMI] 3. Wideband correlation matrix | Diversity (Mini-band based CRU) | MIMO mode 4 | N/A |