| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Modification of PFBCH Encoding Type 1 to support 20 MHz (15.3.9.3.1.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: d.mazzarese@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes modifications to the PFBCH encoding type 1 to support 21 subbands indexes in 20 MHz |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Modification of PFBCH Encoding Type 1 to support 20 MHz (15.3.9.3.1.1)

David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides modified amendment text for PFBCH and SFBCH contents to be included in Section 15.3.9.3.1 of the 802.16m Amendment Working Document [1].

In PFBCH encoding type 1, used for reporting the preferred subband index, we propose the addition of subband indexes 12 to 20, in order to cover the operation with 21 subbands in 20 MHz bandwidth, which is the maximum number of subbands according to [1].

Additionally, the row with index 63 in the PFBCH encoding type 1 has been added, as it was accepted at session 62 but not implemented.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Proposed AWD text Change

<Modify the section 15.3.9.3.1.1 on line 34, page 444 as shown >

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 6 bits of information, providing wideband channel quality feedback, and MIMO feedback.

Table 741 PFBCH Feedback Content

| PFBCH Feedback Content | Related MIMO feedback mode | Description/Notes |
| --- | --- | --- |
| CQI | 0, 1, 2, 3, 4, 5, 6, 7 | 1) Wideband CQI 2) Subband CQI for Best -1 subband |
| STC Rate Indicator | 0, 1, 2, 3 |  |
| Subband index | 2, 3, 5, 6 | Subband selection for best-1 subband |
| PMI | 3, 4, 6, 7 | - wideband PMI - subband PMI for best-1 subband |
| Event-driven Indicator (EDI) for request for switching MFM | N/A | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| Event-driven Indicator (EDI) for Bandwidth Request Indicator | N/A | This is used to request UL bandwidth. 2 sequences (two services) |
| Event-driven Indicator (EDI) for Frequency partition selection (FPS) | N/A | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| Event-driven Indicator (EDI) for Buffer management | N/A | Indicates occupancy status of HARQ soft buffer |

For PFBCH transmission, three encoding types are defined. Encoding type corresponding to MFM and feedback format in Feedback Allocation A-MAP IE is used.

# PFBCH Encoding Type 0

Table 742 - Contents Encoding Type 0 in PFBCH

| Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | STC rate = 1, MCS=0000 |  |
| 1 | STC rate = 1, MCS=0001 |  |
| 2 | STC rate =1, MCS=0010 |  |
| 3 | STC rate =1, MCS=0011 |  |
| 4 | STC rate =1, MCS=0100 |  |
| 5 | STC rate =1, MCS=0101 |  |
| 6 | STC rate =1, MCS=0110 |  |
| 7 | STC rate =1, MCS=0111 |  |
| 8 | STC rate =1, MCS=1000 |  |
| 9 | STC rate =1, MCS=1001 |  |
| 10 | STC rate =1, MCS=1010 |  |
| 11 | STC rate =1, MCS=1011 |  |
| 12 | STC rate =1, MCS=1100 |  |
| 13 | STC rate =1, MCS=1101 |  |
| 14 | STC rate =1, MCS=1110 |  |
| 15 | STC rate =1, MCS=1111 |  |
| 16 | STC rate =2, MCS=0000 |  |
| 17 | STC rate =2, MCS=0001 |  |
| 18 | STC rate =2, MCS=0010 |  |
| 19 | STC rate =2, MCS=0011 |  |
| 20 | STC rate =2, MCS=0100 |  |
| 21 | STC rate =2, MCS=0101 |  |
| 22 | STC rate =2, MCS=0110 |  |
| 23 | STC rate =2, MCS=0111 |  |
| 24 | STC rate =2, MCS=1000 |  |
| 25 | STC rate =2, MCS=1001 |  |
| 26 | STC rate =2, MCS=1010 |  |
| 27 | STC rate =2, MCS=1011 |  |
| 28 | STC rate =2, MCS=1100 |  |
| 29 | STC rate =2, MCS=1101 |  |
| 30 | STC rate =2, MCS=1110 |  |
| 31 | STC rate =2, MCS=1111 |  |
| 32 | STC rate =3, MCS=0100 |  |
| 33 | STC rate =3, MCS=0101 |  |
| 34 | STC rate =3, MCS=0110 |  |
| 35 | STC rate =3, MCS=0111 |  |
| 36 | STC rate =3, MCS=1000 |  |
| 37 | STC rate =3, MCS=1001 |  |
| 38 | STC rate =3, MCS=1010 |  |
| 39 | STC rate =3, MCS=1011 |  |
| 40 | STC rate =3, MCS=1100 |  |
| 41 | STC rate =3, MCS=1101 |  |
| 42 | STC rate =3, MCS=1110 |  |
| 43 | STC rate =3, MCS=1111 |  |
| 44 | STC rate =4, MCS=1000 |  |
| 45 | STC rate =4, MCS=1001 |  |
| 46 | STC rate =4, MCS=1010 |  |
| 47 | STC rate =4, MCS=1011 |  |
| 48 | STC rate =4, MCS=1100 |  |
| 49 | STC rate =4, MCS=1101 |  |
| 50 | STC rate =4, MCS=1110 |  |
| 51 | STC rate =4, MCS=1111 |  |
| 52 | Reserved |  |
| 53 | Reserved |  |
| 54 | Reserved |  |
| 55 | Event-driven Indicator (EDI) for Buffer management (80% full) | Event-driven for buffer management |
| 56 | Event-driven Indicator (EDI) for Buffer management (full) |  |
| 57 | Event-driven Indicator (EDI) for request for switching MFM | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| 58 | Event driven indicator (EDI) for frequency partition 0 indication (reuse-1) | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| 59 | Event driven indicator (EDI) for frequency partition 1 indication (reuse-3) |  |
| 60 | Event driven indicator (EDI) for frequency partition 2 indication (reuse-3) |  |
| 61 | Event driven indicator (EDI) for frequency partition 3 indication (reuse-3) |  |
| 62 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 1) | Event-driven Indicator for Bandwidth request |
| 63 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 2) |  |

# PFBCH Encoding Type 1

Table 743 - Contents Encoding Type 1 in PFBCH

| Sequence Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | Subband index 0 | Subband index for Best-1 subband (refer 15.3.9.3.1.4 feedback format) |
| 1 | Subband index 1 |  |
| 2 | Subband index 2 |  |
| 3 | Subband index 3 |  |
| 4 | Subband index 4 |  |
| 5 | Subband index 5 |  |
| 6 | Subband index 6 |  |
| 7 | Subband index 7 |  |
| 8 | Subband index 8 |  |
| 9 | Subband index 9 |  |
| 10 | Subband index 10 |  |
| 11 | Subband index 11 |  |
| 12 | Subband index 12 |  |
| 13 | Subband index 13 |  |
| 14 | Subband index 14 |  |
| 15 | Subband index 15 |  |
| 16 | Subband index 16 |  |
| 17 | Subband index 17 |  |
| 18 | Subband index 18 |  |
| 19 | Subband index 19 |  |
| 20 | Subband index 20 |  |
| 12 21 | Reserved | Reserved |
| … | … |  |
| 54 | Reserved |  |
| 55 | Event-driven Indicator (EDI) for Buffer management (80% full) | Event-driven for buffer management |
| 56 | Event-driven Indicator (EDI) for Buffer management (full) |  |
| 57 | Event-driven Indicator (EDI) for request for switching MFM | Indicate request to switch MIMO feedback Mode between distributed and localized allocations |
| 58 | Event driven indicator (EDI) for frequency partition 0 indication (reuse-1) | AMS informs ABS about the frequency partition index (for MIMO feedback modes 0,1,4,7) |
| 59 | Event driven indicator (EDI) for frequency partition 1 indication (reuse-3) |  |
| 60 | Event driven indicator (EDI) for frequency partition 2 indication (reuse-3) |  |
| 61 | Event driven indicator (EDI) for frequency partition 3 indication (reuse-3) |  |
| 62 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 1) | Event-driven Indicator for Bandwidth request |
| 63 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 2) |  |

# PFBCH Encoding Type 2

Encoding Type 2 in PFBCH is used for PMI reporting. The PMI of the i -th codebook entry, C (Nt, Mt, NB, i), is mapped into sequence index i in PFBCH.

# 15.3.9.3.1.2 Secondary fast feedback control channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

Table 726 SFBCH Feedback Content

| SFBCH Feedback Content | Related MIMO feedback mode | Description/Note |
| --- | --- | --- |
| Subband CQI | 2, 3, 5, 6 | Reporting of average and differential CQI of selected sub-bands. Reporting of transformed coefficients is TBD |
| Subband index | 2, 3, 5, 6 | Indicating the selected subbands |
| Subband PMI | 3, 6 | Precoding Matrix Indicator of one sub-band for CL MIMO |
| Stream Indicator | 5 | It is needed for OL MU MIMO only and used to indicate which spatial stream to estimate CQI |
| STC Rate Indicator | 2, 3, 5, 6 |  |
| PFBCH Indicator | 2, 3, 5, 6 | One bit indicator is used for indicating the transmission of PFBCH in the next SFBCH opportunity. In the transmission of PFBCH, encoding type 0 is used. |

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------