| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changed to the types of open-loop regions (15.3.7.2.6.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes some corrections to Table 691 (types of open-loop regions) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changed to the types of open-loop regions (15.3.7.2.6.2)

David Mazzarese et Al.

Samsung Electronics

# Introduction

Table 691 in [1] specifies the types of OL regions and the MIMO modes supported in each type. Table 691 shows that in OL Region Type 2, MIMO mode 2 is supported. This is incorrect, as MIMO mode 2 is a CL MIMO mode. MIMO mode 1 (OL MIMO) should be supported instead. For OL Region type 1, MIMO mode 5 is supported in miniband based CRU and subband based CRU. However, it only shows MIMO feedback mode 1, which only supported miniband based CRU. We need to add the support of MIMO feedback mode 2 for subband based CRU.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify Table 691 “Types of open-loop regions” on line 45 page 366 in section 15.3.7.2.6.2 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 691 – Types of open-loop regions

|  | MaxMt | MIMO mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL Region Type 0 | 2 streams | MIMO Mode 0 MIMO Mode 1 (Mt = 2 streams) | 0 | DRU |
| OL Region Type 1 | 1 stream | MIMO Mode 5 (Mt = 1 streams) | 1 | Miniband based CRU (diversity allocation) Subband based CRU (localized allocation) |
|  |  |  | 2 | Subband based CRU (localized allocation) |
| OL Region Type 2 | 2 streams | MIMO Mode 2 1 (Mt = 2 streams) MIMO Mode 3 (Mt = 2 streams) | 5 | Subband based CRU (localized allocation) |

OL region type 2 is supported by MIMO feedback mode 5, which provides channel state information for downlink multiuser transmission with MIMO mode 3. Single-user downlink transmission with MIMO mode 1 is also allowed in OL region type 2 based on the same feedback, but in this case the non-adaptive precoder used with MIMO mode 1 shall be the same as if the downlink transmission was made with MIMO mode 3.

-------------------------------  Text End  ---------------------------------------------------