| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of the notations for the DL CL SU-MIMO codebook subset (15.3.7.2.6.6) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes to clarify the notations used for the closed-loop codebook subset |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clarification of the notations for the DL CL SU-MIMO codebook subset (15.3.7.2.6.6)

David Mazzarese et Al.

Samsung Electronics

# Introduction

The base codebook for 4 transmit antennas is denoted as C(4,Mt,6), and its closed-loop subset is defined in section 15.3.7.2.6.6.2.5.2 by specifying the indexes of the codewords of the base codebook that belong to the subset, such as C(4,Mt,6,m) for m=0…15. Yet, this codebook subset is also referred to in section 15.3.7.1.2.1 (Non-adaptive precoding) in Table 687, using the notation C(4,Mt,4), which is undefined in [1]. Thus we propose to define the notation C(4,Mt,4) for the closed-loop codebook subset in section 15.3.7.2.6.6.2.5.2.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section 15.3.7.2.6.6.2.5.2 on line 45 page 389 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.6.6.2.5.2 CL SU-MIMO subset

15.3.7.2.6.6.2.5.2.1 CL SU-MIMO subset for four transmit antennas

Codebook subset selection for four transmit antennas is specified in Table 712. The notation C(4, Mt, 4) denotes the CL SU-MIMO codebook subset. C(4,1,4,m) = C(4,1,6,m) for the values of m specified in table 712.

Table 712—Subset selection of the base codebok codebook for four transmit antennas

| Rank | One | Two | Three | Four |
| --- | --- | --- | --- | --- |
| Subset notation | C(4,1,4,m) m = 0 to15 | C(4,2,4,m) m = 0 to15 | C(4,3,4,m) m = 0 to15 | C(4,4,4,m) m = 0 to 5 |
| Subset selection | C(4,1,6,m) m = 0 to15 | C(4,2,6,m) m = 0 to15 | C(4,3,6,m) m = 0 to15 | C(4,4,6,m) m = 0 to 5 |

-------------------------------  Text End  ---------------------------------------------------