| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed descriptions of differential CQI and feedback periods for the feedback formats of the SFBCH (15.3.9.3.1.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Sangheon Kim Hokyu Choi, Heewon Kang Samsung Electronics | E-mail: d.mazzarese@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution specifies the timing of short feedback and long feedback periods in the MIMO feedback formats. It also specifies the equations to determine the subband CQI and differential CQI. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed descriptions of differential CQI and feedback periods for the feedback formats of the SFBCH (15.3.9.3.1.4)

David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

This contribution specifies the timing of short feedback and long feedback periods in the MIMO feedback formats. It also specifies the equations to determine the subband CQI and differential CQI. At the same time, we regroup the descriptions of CQI assumptions under section 15.3.9.3.1.3 (Channel quality indicator(CQI) definition).

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Proposed AWD text

<Modify the section 15.3.9.3.1.4 on line 36, page 450 as shown >

----------------------------------------- Start of Proposed Text 1 --------------------------------------------

# 15.3.9.3.1.4 Feedback format

# Feedback format for MFM 0,1,4,7

Feedback formats for MFM 0, 1, 4, and 7 are listed in Table xx 745. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report if MFM = 4 or 7, or by puncturing the 2q-th short period report if MFM = 0 or 1, where q is defined in the feedback allocation A-MAP IE. The wideband CQI is one average CQI corresponding to the MIMO mode signaled by the combination MFM and STC rate, with averaging over the whole band. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

Table 745 - Feedback formats for MIMO feedback mode 0, 1, 4, and 7

| MFM | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 1 | PFBCH | 2 | Short | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 |
|  |  |  | Long | Wideband CQI and STC rate | N/A | Joint encoding of CQI and STC rate Encoding type 0 Long term FPI for FFR |
| 4 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |
| 7 | PFBCH | 2 | Short | Wideband CQI | N/A | STC rate = 1 Encoding type 0 |
|  |  |  | Long | Wideband PMI | N/A | PMI for rank 1 Encoding type 2 |

# Feedback format for MFM 2

The detailed format is listed in Table 727 746. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI). Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 746 - Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 2 | Short | Subband CQI and STC rate (rate=1 and 2) | N/A | Joint encoding of CQI and STC rate with PFBCH PFBCH encoding Type 0 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 1) | SFBCH | 1 | Subband index Subband CQI STC rate PFBCH indicator |  | 3, 4, or 5 4 1~3 1 | Subband index for 5, 10, or 20MHz Support of STC rate 1 to 8 |
| 2 (M = 3) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 23 = 6 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 5, 8 or 11 1~3 1 |  |
| 3 (M = 5) | SFBCH | 2 | Short | Subband avg CQI differential CQI | 4 25 = 10 |  |
|  |  |  | Long | Subband index Wideband STC rate PFBCH indicator | 3, 10, or 16 1~3 1 |  |

# Feedback format for MFM 3

The detailed format is listed in Table 728 747. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after subband index. For feedback format 0 with PFBCH, Then subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI). Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 747 - Feedback formats for MIMO feedback mode 3

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 STC rate = 1 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 7 or 9 1~3 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 5, 8 or 11 1~3 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Wideband STC rate Subband avg CQI PFBCH indicator | 4, 9 or 14 1~3 4 1 |  |

Table 748 - Feedback formats for MIMO feedback mode 3 for differential codebook

| Feedback Format | FBCH | Number of Reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2-4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband index STC rate Base PMI CQI PFBCH indicator | 3, 4 or 5 1 to 3 3-6 4 1 | Subband index for 5, 10, or 20MHz PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 1~3 4 2×3~2×6=6~12 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 8 or 10 1~3 4 3×3~3×6=9~18 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index STC rate Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 1~3 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

# Feedback format for MFM 5

The detailed format is listed inTable 729 749. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI). Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 749 - Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Short | Subband index Subband CQI Stream Index PFBCH indicator | 3, 4, or 5 4 1~2 1 | Subband index for 5, 10, or 20MHz |
| 1 (M=2) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 22=4 (1~2) 2 |  |
|  |  |  | Long | Subband index PFBCH indicator | 4, 7, or 9 1 | Subband index for 5, 10, or 20MHz |
| 2 (M =3) | SFBCH | 2 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 23=6 3 (1~2) |  |
|  |  |  | Long | Subband index PFBCH indicator | 5, 8, or 11 1 | Subband index for 5, 10, or 20MHz |
| 3 (M =5) | SFBCH | 3 | Short | Subband avg_CQI Subband differential CQI Stream Index | 4 25=10 (1~2) 5 |  |
|  |  |  | Long | Subband index PFBCH indicator | 3, 10, or 16 1 | Subband index for 5, 10, or 20MHz |

# Feedback format for MFM 6

The detailed format is listed in Table xxx 750. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report according to long term feedback period in feedback allocation A-MAP IE. The long period report shall start by puncturing the first short period report. For feedback format 0 with 3 reports using the PFBCH, Subband PMI shall be transmitted first after subband index. For feedback format 0 with PFBCH, Then subband CQI and subband PMI are transmitted alternately and subband index is transmitted in the long term period by PFBCH encoding type 1. If the PFBCH indicator in SFBCH is set to ‘1’, PFBCH is transmitted by puncturing SFBCH in the next feedback opportunity regardless of short-term and long-term feedback using encoding type 0.

The CQI of subband m shall be computed as follows for the first short period report following a long period report: (Subband m CQI index) = (Subband avg CQI index) + (Subband m differential CQI). Subband avg CQI index is an average measure of the CQI over the M reported subbands. The possible differential CQI values are {-1, 0, +1, +2}. The AMS shall ensure that the reported differential CQI will produce a value of subband m CQI in the range of 0 to 15.

Table 750 - Feedback formats for MIMO feedback mode 6

| Feedback Format | FBCH | Number of reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M = 1) | PFBCH | 3 | Short | Subband CQI | N/A | PFBCH encoding Type 0 |
|  |  |  | Short | Subband PMI | N/A | PFBCH encoding Type 2 |
|  |  |  | Long | Subband index | N/A | PFBCH encoding Type 1 |
| 1 (M = 2) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 22 (3~6)2 | Subband index for 5, 10, or 20MHz |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 7 or 9 4 1 |  |
| 2 (M = 3) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 23 (3~6)3 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 5, 8 or 11 4 1 |  |
| 3 (M = 4) | SFBCH | 2 | Short | Subband differential CQI Subband PMI | 24 (3~4)4 |  |
|  |  |  | Long | Subband index Subband avg CQI PFBCH indicator | 4, 9 or 14 4 1 |  |

Table 751 - Feedback formats for MIMO feedback mode 6 for differential codebook

| Feedback Format | FBCH | Number of Reports | Report period | Feedback Fields | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short | Subband CQI Differential PMI | 4 2~4 | Diff. PMI (Nt=2: 2 bit, Nt=4: 4 bit, Nt=8: 4 bit) |
|  |  |  | Long | Subband selection Base PMI CQI PFBCH indicator | 3, 4 or 5 3-6 4 1 | PMI for reset. CQI for reset |
| 1 (M=2) | SFBCH | 2 | Short | Diff CQI Differential PMI | 2×2 = 4 2×2~2×4=4~8 | Diff. PMI( Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 4, 7 or 9 4 2×3~2×6=6~12 1 |  |
| 2 (M =3) | SFBCH | 2 | Short | Diff CQI Differential PMI | 3×2 = 6 3×2~3×4=6~12 | Diff. PMI (Nt=2: 2 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband selection Avg CQI Base PMI PFBCH indicator | 5, 8 or 10 4 3×3~3×6=9~18 1 |  |
| 3 (M =4) | SFBCH | 3 | Short | Diff CQI Differential PMI | 4×2=8 4×2~4×4=8~16 | Diff. PMI (Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband) |
|  |  |  | Long | Subband index Avg CQI Base PMI PFBCH indicator | 4, 9 or 14 4 4×3~4×4=12~16 1 | Subband index for 5, 10, or 20MHz Base PMI (Nt=2: 3 bits per subband, Nt=4: 4 bits per subband, Nt=8: 4 bits per subband) 2 long reports will puncture short report continuously. |

------------------------------------------- End of Proposed Text 1 --------------------------------------------

<< Modify section “15.3.7.2.6.1 Downlink post-processing CINR measurement feedback” as follows >>

----------------------------------------- Start of Proposed Text 2 --------------------------------------------

15.3.7.2.6.1 Downlink post-processing CINR measurement feedback

The reported channel quality indicator has two types: wideband CQI, subband CQI.

The wideband CQI is one average CQI over the whole band.

The subband CQI is one average CQI over the subband.

For MU-MIMO feedback modes, the CQI is calculated at the AMS assuming that the interfering users are scheduled by the serving ABS using rank-1 precoders orthogonal to each other and orthogonal to the rank-1 precoder represented by the reported PMI.

----------------------------------------- End of Proposed Text 2 --------------------------------------------

<< Modify “section 15.3.9.3.1.3 Channel quality indicator(CQI) definition” as follows >>

----------------------------------------- Start of Proposed Text 3 --------------------------------------------

15.3.9.3.1.3 Channel quality indicator(CQI) definition

The CQI feedback together with the rank feedback (when applicable) composes the spectral efficiency value reported by the AMS. This value corresponds to the measured block error rate which is the closest, but not exceeding, a specific target error rate.

The CQI report shall be based on the assumption of specific reference parameters, such as: target error rate value, H-ARQ mode and block size. In order to allocate the AMS with MCS level and rank appropriate for the actual requirements, the ABS should make adjustments to the AMS reported spectral efficiency, by considering parameters values different from the reference ones and by adapting it to delay and mobility conditions.

For MU-MIMO feedback modes with codebook-based feedback, the CQI is calculated at the AMS assuming that the interfering users are scheduled by the serving ABS using rank-1 precoders orthogonal to each other and orthogonal to the rank-1 precoder represented by the reported PMI.

Wideband CQI

Wideband CQI is one average CQI over whole band corresponding to the MIMO mode. In the case where the number of DL frequency partitions FPCT > 1, the wideband CQI is one average CQI over the corresponding frequency partition.

Subband CQI

Subband CQI is represented by one base CQI (ave_CQI) over the selected M subbands (or whole bandwidth TBD) plus a differential CQI, which is used for best-M feedback.

For best-M feedback, the subband selection and Ave_CQI and Diff_CQI may be reported at different intervals, and share the same FBCH with TDM manner. Subband selection and Ave_CQI will puncture the Diff_CQI at every [4th TBD] report if TDM is enabled.

[All subband CQIs across the whole bandwidth may be organized in hierarchical tree (TBD)]

[Reporting of transformed coefficients is TBD, where AMS linearly transforms all the subband CQIs (whole band) into a series of progressively insignificant coefficients, and quantize the coefficients to few bits for feedback. The transformed values are sent over one or several reports]

Specified CQI (TBD)

Specified CQI is the CQI specified for a certain RU indicated by DLRU parameters in feedback allocation A-MAP IE.

----------------------------------------- End of Proposed Text 3 --------------------------------------------