| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the feedback polling A-MAP IE (15.3.6.5.4.12) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | Modifications are proposed for the feedback polling A-MAP IE in order to support periodic allocations, and assignment of single-BS MIMO feedback or multi-BS MIMO feedback. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed changes to the feedback polling A-MAP IE (15.3.6.5.4.12)

David Mazzarese et Al.

Samsung Electronics

# Introduction

Modifications are proposed for the feedback polling A-MAP IE in order to support periodic allocations, and assignment of single-BS MIMO feedback or multi-BS MIMO feedback. The resource allocation is inspired from the UL persistent allocation A-MAP IE. In addition to the feedback assignment by the feedback polling A-MAP IE in [1], we add single-BS MIMO feedback information. It allows to feedback the full subbands for localized MIMO feedback modes, and all frequency partitions for distributed MIMO feedback modes.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.6.5.4.12” on line 17 page 356 as follows>

-------------------------------  Start of Text Proposal ---------------------------------------------------

15.3.6.5.4.12 Feedback Polling A-MAP IE

The information element shown in Table 685 is used by BS to schedule feedback header transmission by the MS.

Table 685 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | [4] | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| If (d ==0b00){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b00){ |  |  |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| MCS I_SizeOffset | [2 or 4] 5 | MCS level for transmitting feedback header |
| Resource Allocation Index | [TBD] 11 | Indicating the first Distributed LRU index for transmission |
| Allocation Duration (d) | [3] | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header transmission is released. If d == 0b111, the pre-scheduled feedback header transmission shall be valid until the BS commands to release it. |
| } |  |  |
| Period (p) | [2] 3 | Transmit feedback header every [2p superframe frame ] |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO operation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| If (Dedicated UL allocation == 0b1){ |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding with Mt = 2 if Nt=2 or 4, or Mt = 1 if Nt=1 - Non-adaptive precoding shall be used at the MS. Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Transmit_Correlation_Matrix | 1 | - 0b0: MIMO feedback mode with or without transmit correlation matrix - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit_Correlation_Matrix shall be equal to 0b0 if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. CM determines whether or not to feedback the transmit correlation matrix. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2 (SU-MIMO and MU-MIMO): - 0b0: 1 - 0b1: 2 - If Nt=4 (SU-MIMO and MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8: (MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| MFM | 3 | - MIMO Feedback Mode for which the AMS shall feedback CQI and CSI to the ABS |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 3 | 0b000: report all subbands 0b001~0b111: Number of best subbands to report Num_best_subbands ≤ Nsub |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | 1 | Codebook Feedback Mode and Codebook Coordination Enable 0b0: standard with CCE disabled 0b1: transformation with CCE disabled |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| TRU | [3] | - Target RU indicating which RUs or which type of RU to work on for feedback |
| ICT | [3] 2 | 0b00: PMI restiction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | [3] | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: not feedback of CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 3 2 | Maximum number of users supported in Co-MIMO, 0b000 corresponds to 2 users in maximum are supported in the same resource. This will impact the CQI estimation at the MS side. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Measured Logical Carrier Index | [3] | - TBD |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

If MIMO_feedback_IE_type = 0b1, the AMS should transmit using MIMO mode 0 if it has multiple transmit antennas.

On the allocated UL resource at the feedback periods indicated by the feedback polling IE, the AMS shall send the MIMO feedback IE with priority, and it may fill the remaining parts of the allocated resource with other MAC management messages or payload data if available.

-------------------------------  End of Text Proposal  ---------------------------------------------------