| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to MIMO midamble (15.3.5.4.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Jerry Pi, Jiann-An Tsai, David Mazzarese Samsung Electronics Co., Ltd. | +1-972-761-7944 zpi@sta.samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes changes to MIMO midamble in P802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to MIMO midamble (15.3.5.4.2)

Jerry Pi, Jiann-An Tsai, David Mazzarese

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of MIMO midamble to be included in P802.16m/D1 ([1]).

The proposed change enables regular spacing of midamble subcarriers per transmit antenna, thereby reducing the PAPR of the midamble. The proposed change also increase the frequency sampling of midamble from once per 24 subcarriers to once per 9 subcarriers.

- References

[1] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”, July 2009.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Replace section 15.3.5.4.2, page 282, line 56 – page 283, line 28 with the following text:

The MIMO midamble signal transmitted by the ABS antenna is defined by Equation (196):

k≠Nused−12k=Nused−1                                   (196)

where bk is a coefficients modulating subcarriers in the midamble symbol defined from equation below

bk=1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod(3∗Nt)=g+k−sN1∗NscmodNt+Nt∗(BSIDmod3)bk=1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmod27=g+smod8+9×((BSID+n)mod3)      (197)

where k is the subcarrier index (0 ≤k ≤ Nused -1), Nused is the number of used subcarriers, Nt is a number of transmit antennas, G(x) is the Golay sequence defined in Table 1 (0 ≤ x ≤ 2047), fft is the FFT size used, u is a shift value (0 ≤ u ≤ 127), where the actual value of u is derived from u = mod(BSID, 128), offsetD(fft) is an FFT size specific offset as defined in Table 2, g is a ABS transmit antenna index, Nt is number of ABS transmit antennas, parameter s = 0, for k≤(Nused - 1)/2 and s = 1, for k>(Nused - 1)/2, and n is the frame number.

-------------------------------  Text End  ---------------------------------------------------