| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Change for Uplink FFR Section (15.3.14.2) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | HanGyu Cho, Wookbong Lee LG Electronics | Voice: +82-31-450-1883 E-mail: hgcho@lge.com |
| Re: | 80216m-09_0044, “Letter Ballot #30” |  |
| Abstract | In the current UL FFR section, there is no description on sector-specific FFR pattern, which is necessary for ULPC with UL FFR. This document describes a method to choose a sector-specific FFR pattern. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for Uplink FFR section (15.3.14.2)

HanGyu Cho, Wookbong Lee

LG Electronics

- Introduction

According to the UL FFR section of P802.16m/D1, UL FFR allows the system to designate a different UL IoT control parameter, i.e. γIoTper frequency partition. Moreover, detailed signaling method for γIoT is shown in Table 50. According to Table 50, each sector adopts sector-specific FFR pattern. In the current UL FFR section, however, there is no description on sector-specific FFR pattern, which is necessary for ULPC with UL FFR. This document describes a method to choose a sector-specific FFR pattern based on cell ID.

- 2. Proposed text change in the P802.16m/D1

--------------------------------------------------------------------- Text Start --------------------------------------------------------------------------------

Insert the following text and Figure xx in Line 13, Page 227 in Section 15.3.14.2

UL IoT control parameter, γIoT, signalled according to Table 50, is used in ULPC based on sector-specific FFR pattern. When UL FFR is applied in the cell (i.e., UFPCT>1), the different FFR pattern is used by different BSs. When UFPCT=4, for example, each BS chooses one of the three FFR patterns (Sector 1, sector 2 and sector 3 pattern) as shown in Fig. xx (when UFPCT=3, the same FFR patterns are used only excluding FP0). The index of FFR pattern is set by a particular BS with its Cell_ID. That is, each BS adopts the FFR pattern corresponding to the sector k decided by the following equation:

Sector k = mod(Cell_ID,3)

Figure xx shows an illustration of UL FFR when UPFCT is four.

Figure xx. The basic concept of UL FFR

--------------------------------------------------------------------- Text End --------------------------------------------------------------------------------