| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose text of AAI PHY control Messages (15.2.6.x) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm, Kiseon Ryu, HanGyu Cho LG Electronics | wbong@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution provides the proposed text of AAI PHY control Messages. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Propose text of AAI PHY control Messages (15.2.6.x)

Wookbong Lee, Bin-Chul Ihm, Kiseon Ryu, HanGyu Cho

LG Electronics

- Introduction

This contribution provides the proposed text of AAI PHY control Messages for the IEEE 802.16m Amendment. We propose to have two PHY control Message as one of additional broadcast information (ABI).

All of this information is required for Physical layer control and does not require version update.

One requires frequent updates while the other requires less frequenct updates.

So, we would like to define AAI_PHYCtrl_T1 as a frequent update type message and AAI_PHYCtrl_T2 as a less frequent update type message.

Examples of contents for AAI_PHYCtrl_T1 are NI information for uplink power control, FFR partition resource metrics, and BC_SI [1]. And examples of contens for AAI_PHYCtrl_T2 are FFR power boosting value, default RSSI and CINR averaging parameter, OL Region related information, Power control related information.

- References

[1] IEEE P802.16m/D1 Draft

- Text proposal

[Insert the following new subcluase into Section 15.2.6.x

-------------------------------  Text Start  ---------------------------------------------------

### 15.2.6.x   AAI_PHYCtrl_T1 (Physical Control type1) message

An AAI_PHYCtrl_T1 is one of additional broadcast message (ABI). It should be broadcasted by the ABS at a periodic interval to adapt a physical control parameter. An ABS should generate AAI_PHYCtrl_T1 message in the format shown in Table xxx.

Table xxx – AAI_PHYCtrl_T1 message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_PHYCtrl_T1_Format() { |  |  |
| Message type | TBD |  |
| Parameter BitMap (PBM) | TBD |  |
| if (1st MSB of PBM == 1) |  |  |
| for (i=0;i<FPCT;i++) |  |  |
| NIi | TBD | i-th partition’s NI value |
| } |  |  |
| NIsounding | TBD |  |
| } |  |  |
| if (2nd MSB of PBM == 1) |  |  |
| FFR partition resource metric | TBD |  |
| } |  |  |
| if (3rd MSB of PBM == 1) |  |  |
| BC_SI | 8 ~ 16 | 8 bit for 2Tx, 16 bit for 4 and 8Tx |
| } |  |  |
| … |  |  |
| } |  |  |

### 15.2.6.y   AAI_PHYCtrl_T2 (Physical Control type2) message

An AAI_PHYCtrl_T2 is one of additional broadcast message (ABI). It should be broadcasted by the ABS at a periodic interval to adapt a physical control parameter. An ABS should generate AAI_PHYCtrl_T2 message in the format shown in Table xxx.

Table xxx – AAI_PHYCtrl_T2 message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_PHYCtrl_T2_Format() { |  |  |
| Message type | TBD |  |
| Parameter BitMap (PBM) | TBD |  |
| if (1st MSB of PBM == 1) |  |  |
| default RSSI and CINR averaging parameter |  |  |
| } |  |  |
| if (2nd MSB of PBM == 1) |  | For open loop power control mode 1 |
| γIoT | 4 |  |
| if (FPCT != 1) |  |  |
| Δ | 3 | For boosted frequency partitions, γIoT is applied for OLPC mode 1. For deboosted frequency partition, γIoT- Δ is applied for OLPC. |
| } |  |  |
| α | 3 |  |
| β | 1 |  |
| SINRmin | 4 |  |
| } |  |  |
| if (3rd MSB of PBM == 1) |  | For OL-Region |
| OL-Region-ON | 1 |  |
| OL-Rank1-Config | 3 |  |
| SB-OL-Region-2Size | 4 |  |
| } |  |  |
| if (4th MSB of PBM == 1) |  | Downlink FFR power boosting/deboosting value |
| FP1_power | 4 |  |
| FP2_power | 4 |  |
| FP3_power | 4 |  |
| } |  |  |
| … |  |  |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------