| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Propose text of AAI PHY control Messages (15.2.6.x) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm, Kiseon Ryu, HanGyu Cho LG Electronics | wbong@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | This contribution provides the proposed text of AAI PHY control Messages. |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Propose text of AAI PHY control Messages (15.2.6.x)

Wookbong Lee, Bin-Chul Ihm, Kiseon Ryu, HanGyu Cho

LG Electronics

- Introduction

- References

[1] IEEE P802.16m/D1 Draft

- Text proposal

- Remedy #1

[Modify the text in line8 ~ 45, page 86 as follows,

-------------------------------  Text Start  ---------------------------------------------------

15.2.6.29 AAI_System Configuration Descriptor(SCD) Message

An AAI_SCD shall be transmitted by the ABS at a periodic interval to define a system configuration.

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change			INTEGER (0 ~ 15)

BS_Restart_Count				INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON			INTEGER (0..1) OPTIONAL

OL-Rank1-Configuration		INTEGER (0..7) OPTIONAL

SB-OL-Region-2Size		INTEGER (0..15) OPTIONAL

}

BC_SI						INTEGER (0 ~ 3)

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

γ					INTEGER (0..15) OPTIONAL

γ					INTEGER (0..15) OPTIONAL

γ					INTEGER (0..15) OPTIONAL

γ					INTEGER (0..15) OPTIONAL

Alpha (α	)						INTEGER (0.. 7) OPTIONAL

Beta (β)							INTEGER (0..1) OPTIONAL

Default_SINRmin					INTERGER (0..15) OPTIONAL

}

-- ASN1STOP

15.2.6.30 AAI_UL_Power_Control(ULPC) Message

An AAI_ ULPC broadcast NI values for the UL power control parameters estimated in BS.

-- ASN1START

AAI_ULPCMessage ::= SEQUENCE {

UL_NI_Information :: = SEQUENCE {}

IOT_Sounding				INTEGER (0..127) OPTIONAL

IOT_FP0					INTEGER (0..127) OPTIONAL

IOT_FP1					INTEGER (0..127) OPTIONAL

IOT_FP2					INTEGER (0..127) OPTIONAL

IOT_FP3					INTEGER (0..127) OPTIONAL

}

}

-- ASN1STOP

For the open-loop power control, UL noise and interference level shall be broadcast to AMSs in the given BS coverage by BS. All the UL noise and interference noise level are quantized in 1 dBm steps from -150 dBm to -23 dBm.

For the open-loop power control, UL noise and interference level shall be broadcast to AMSs in the given BS coverage by BS. All the UL noise and interference noise level are quantized in 0.5 dB steps as IoT level from 0 dB to 63.5 dB.

The absolute power level of NI is transformed from IoT level as:

NI=PTN+IoT+10log10(Δf)

Where,

PTN is the thermal noise power density in 00C

Δf is the subcarrier spacing.

15.2.6.31 AAI_IM Message

An AAI_IM broadcast FFR partition resource metric, BC_SI, NIP_threshold, power level for each frequency partition.

-- ASN1START

AAI_IM Message ::= SEQUENCE {

FFR partition resource metric :: = SEQUENCE { }

BC_SI						INTEGER (0..15) OPTIONAL

NIP_th_1					INTEGER (TBD) OPTIONAL

NIP_th_2					INTEGER (TBD) OPTIONAL

FP1_power					INTEGER (TBD) OPTIONAL

FP2_power					INTEGER (TBD) OPTIONAL

FP3_power					INTEGER (TBD) OPTIONAL

}

- Remedy #2

[Delete rows for

FFR partition resource metrics,

MIMO rank 1 OL region signaling

MIMO rank 1 OL region

NI information for UL power control

Default RSSI and CINR averaging parameter

Tx Power report

In table 676]