| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change to MIMO channel feedback header (AWD - 15.2.2.1.3) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm LG Electronics | Voice: +82-31-450-1883 E-mail: wbong@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Provide text change proposal to multi-BS MIMO feedback header |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change to MIMO channel feedback header (AWD - 15.2.2.1.3)

Wookbong Lee, Bin-Chul Ihm

LG Electronics

- Introduction

There is some error in table 656 MIMO channel feedback header. And there are no details for additional information as well as PMI_coordination_subset.

- 2. Proposed text change in the 802.16m amendment

Note:

Black text: existing text in AWD (P802.16m/D1)

Red Strikethrough text: deleted text

Blue text: new text

--------------------------------------------------------------------- Text Start --------------------------------------------------------------------------------

Remedy #1 Line 40, page 9 to line 32, Page 10

Delete empty rows in table 656 and Modify as follows;

15.2.2.1.3 MIMO channel feedback header 15.2.2.1.3 Multi-BS MIMO feedabck header

The header shown in Table 656 includes MIMO channel multi-BS MIMO feedback header. AMS feedback multi-BS MIMO feedback header once it receive Feedback Polling A-MAP IE and FT == 0b1.

Table 656 - AAI_FFR-REP message format Multi-BS MIMO feedabck header

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| If (ICT == 0b00, 0b01, 0b10, 0b10 or 0b11) { |  | ICT is defined in Feedback Polling A-MAP IE |
| if (ICT == 0b10 or 0b11) { |  | ICT is defined in Feedback Polling A-MAP IE |
| CQI | [4] |  |
| } |  |  |
| Change count | 8 | Change count of current AMS’s AAI_NBR-ADV message |
| for (i=1; i<= N_multiBS_reports; i++){ |  |  |
| Temp_BSID | [TBD]4 | Diversity set member ID |
| PMI | [3 to 6] | PMI from the rank-1 base codebook [or base codebook subset] |
| if (ICT == 0b00 or 0b01){ |  |  |
| Additional measurement metric | 2 | SINR gain assuming the reported PMI set is coordinated over CQI for single BS MIMO feedback reported via previous FFBCH. 0b00: 0.25dB 0b01: 0.50dB 0b10: 1.00dB 0b11: above 1.50dB |
| PMI_subset_size | 1 | Indication whether 1 PMI or a PMI set is feedback 0b0: 1 PMI 0b1: multiple PMIs |
| If (PMI_subset_size == 0b1){ |  |  |
| PMI_coordination_subset | [1] | Set of PMIs from the rank-1 base codebook or base codebook subset 0b0: correlation level n1 (cf Table 54) 0b1: correlation level n2 (cf Table 54) |
| } |  |  |
| } |  |  |
| if (ICT == 0b10 or 0b11){ |  |  |
| if (CPI == 0b0){ |  | CPI is defined in Feedback Polling A-MAP IE |
| CPMI | [TBD] | Concatenating PMI for neighboring cells |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Change count

This indicates the AAI_NBR-ADV message change count of the Temp_BSID. In case system information of the corresponding Temp_BSID changes, the serving ABS will provide the AMS with necessary updates by referring to this change count parameter.

Remedy #2 Line 17 to 20, page 229

Modify as follows;

Table 52 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for serving and neighboring cell | For PMI coordination among multiple ABSs |
|  | PMI_coordination_subset indication | Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell |
|  | Additional measurement metric | Optional aAdditional feedback for indicating the potential interference level improvement associated with the PMIs reported for the neighboring cells. This can be used for resolving conflict from multiple AMS. |
|  | CPMI for neighboring cell | Concatenating PMI for neighboring cell |
| Event-driven feedback | Requesting Multi-BS MIMO | For AMS reporting of its preference on single BS precoding with Multi-BS MIMO coordination using a MAC management message |

--------------------------------------------------------------------- Text End --------------------------------------------------------------------------------