| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed change for Downlink FFR section (AWD - 15.3.14.1) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Wookbong Lee, Bin-Chul Ihm, HanGyu Cho LG Electronics | Voice: +82-31-450-1883 E-mail: wbong@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30 |  |
| Abstract | Provide a mechanism for FFR power boosting level |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed change for Downlink FFR section (AWD - 15.3.14.1)

Wookbong Lee, Bin-Chul Ihm, HanGyu Cho

LG Electronics

- Introduction

Currently there is no mechanism for indicating FFR partition power level.

There is two way to indicating each FFR partition’s power level, implicit and explicit way.

Implicit way is using MIMO midamble by allocating same power level according to the FFR partition power.

However, in current midamble can’t support this easily since FFR partition is done in a PRU level. Within a PRU, the performance of midamble is quite bad. Sometimes only one pilot is in a PRU for a certain transmit antenna.

Explicit way is indicating each partitions power level in a broadcast message. When estimating CQI for each partition, MS need to estimate other cell’s power level also. Explicitly signal all these information is quite large. So, we propose selecting FFR power level pattern as a function of Cell ID as we did for interlaced pilot pattern.

- 2. Proposed text change in the 802.16m amendment

--------------------------------------------------------------------- Text Start --------------------------------------------------------------------------------

Remedy #1 Line 35, Page 222

Insert the following text;

When FFR is applied in the cell (i.e., FPCT>1), the different FFR power pattern is used by different BSs. When FPCT=4, for example, each BS chooses one of the three FFR patterns (Sector 1, sector 2 and sector 3 pattern) as shown in Fig. 428 (when FPCT=3, the same FFR patterns exist only excluding FP0). The index of FFR power pattern is set by a particular BS with its IDCell. That is, each BS adopts the FFR power pattern corresponding to the sector k decided by the following equation:

Sector k = floor(k/255)

Remedy #2 Line 31, Page 224

Insert the following text;

Each frequency partition’s power differences relative to the reference power level are broadcasted in ABI as shown in the Table xx.

Table xx - FFR partition power boosting/de-boosting level for sector 1

| Syntex | Size (bits) | Notes |
| --- | --- | --- |
| FP1_power | [4] | xdB boosted over reference signal |
| FP2_power | [4] | xdB de-boosted over reference signal |
| FP3_power | [4] | xdB de-boosted over reference signal |

--------------------------------------------------------------------- Text End --------------------------------------------------------------------------------