| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 16m AAI_MC-ADV and AAI_NBR-ADV message format (15.2.6) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Xiangying Yang Muthaiah Venkatachalam Etemad Kamran Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.6” |  |
| Abstract | This contribution proposes text for HO network re-entry procedure in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

16m AAI_MC-ADV and AAI_NBR-ADV message format (15.2.6)

Xiangying Yang, Muthaiah Venkatachalam, Etemad Kamran

Intel

- Design for 16m AAI_MC-ADV

There are two possible ways to allow MS to obtain network multi-carrier configuration depending on how consistent such information is across the network and how frequent it may change. Both can fit in the framework in this contribution.

I.1 provision/unicast network wide multi-carrier configuration for MS

This method is to allow the MS to store the network multi-carrier configuration and its associated operator ID either through pre-provisioning, over-the-air provision, or unicast from BS during the first network entry, as shown in Fig-1. MC configuration shall carry a version number (or change count) associated with the current configuration. For future network entry, the MS shall send its stored MC configuration version number (for example in RNG-REQ, SBC-REQ or REG-REQ) and receive confirmation of valid version number in response from BS (for example in RNG-RSP, SBC-RSP or REG-RSP). If there is any change detected, the BS shall carry the updated information in response to overwrite the old version stored on the MS. This approach is the most efficient when the configuration is likely static in long term and consistent across the same operator network.

MS acquired MC configuration + version#  through

pre-provisioning

OTA provisioning

Unicast during previous NE

MS

BS

Store the

(MC configuration, version#)

MS sends version#

BS sends confirmation

Or updated configuration with new version#

a new Network Entry starts

Figure 1

I.2 Broadcast of network multi-carrier configuration

The network operator may also choose to broadcast network wide MC configuration in MC-ADV so that all connected MS and those who are entering the cell will always obtain the latest information over the air. The broadcast shall still include a mandatory version number to be compatible with I.1 operations. To have flexibility, we can have a flag to turn on/off this broadcast based on the need (as shown in Fig-2).

Note not matter which method is used, I.1 or I.2, all available carriers (in the network or from the operator) shall have their distinct carrier index. A given BS may operate on all available carriers or a subset of these carriers.

I.3 serving BS configuration broadcast

Serving BS broadcast its own configuration of carrier in its downlink control. Due to the limited capacity of physical broadcast channel. It makes sense to have this info in some broadcast message so that the BS can control its flexible format and flexible broadcast schedule. For simplicity, we can also just use the MC-ADV message defined above, so that MC-ADV looks like in Fig-2. Note that for backward compatibility and complexity reduction, each carrier shall has its own MAC operation with associated 48bit BS-ID.

With the proper network MC configuration information acquired by MS in Section I.1/I.2, and listening to the serving BS configuration broadcast in Section I.2, the MS shall be able to perform all MC related operations within the same physical BS.

BS MC configuration

carrier index available

preamble index

BS-ID

MAC protocol

Other BS specific info

MC-ADV

Configuration Version #

MC config broadcast flag

Network MC configuration (optional)

carrier index

carrier frequency

bandwidth

duplexing mode

Flag=1

Figure 2

- Design for 16m AAI_NBR-ADV

There are a few reasons that 16e MOB_NBR-ADV message format needs improvement in 16m.

- 16m use SFH to carry system information instead of TLV based DCD/UCD.

- 16m is to support deployment of multiple cell types and HO between them. 16e MOB_NBR-ADV relies on delta TLV encoding for overhead reduction, which may not be efficient when cell types are different.

We propose new AAI_NBR-ADV message format that is flexible enough to meet 16m deployment requirement as well as achieve good overhead reduction. A conceptual structure is illustrated in the Figure 3.

The AAI_NBR-ADV message carries system information of ABS of a particular cell type. Each AAI_NBR-ADV message contains the following information

- Segment control information (total number of cell types, cell type of this message, segment information if this message is partitioned into multiple segments)

- parameters shared by this particular cell type

-	carrier index (this refer back to the “carrier index” in network MC configuration)

-	MAC protocol (e.g. 16m BS, 16e BS, 16e/16m mixed mode BS)

-	Cell type (e.g. [macro/pico, public/private], private will only appear in unicast )

-	Others

- Network entry-related information in AAI_NBR-ADV: SFH IEs can be flexibly carried in AAI_NBR-ADV so as to shorten scanning time. If no SFH or not full SFH information is carried, MS can obtain it during scanning.

- system information of each BS, with two possible encoding formats, where

  - encoding format 1 uses SFH format (except those parameters already defined in cell type parameters), with a control_bitmap indicating whether a particular subpacket is included or not (not included due to overhead or less importance). Except that the serving BS is of this cell type too, the 1st BS in the list always uses this encoding format. The control_bitmap allows serving BS to broadcast minimal system information for certain neighbor BS (e.g., 16e BS or overlay pico-cells) so that MS can obtain the information via listening to LZone or scanning.

  - encoding format 2 uses 16e-like delta TLV format. The reference BS for delta encoding is always the serving BS if they are of the same type, and if not the reference BS is the 1st BS in this cell type. In case of

Figure 3 AAI_NBR-ADV structure with two different encoding formats

To perform mobility (e.g. handover or cell reselection), the MS must process network MC configuration as mentioned in Section I.1/I.2, serving BS configuration as in Section I.3, and neighbor list mentioned in Section II.

Proposed text #1:

Modify the following text into section 15.2.6

-------------------------------------------text #1 start----------------------------------------------

15.2.6.x7.2.3.1.1 AAI_MC-ADV (Multi-Carrier Advertisement) Message

Table xxx AAI_MC-ADV MAC management message format

| Field | Size (bit) | Description |
| --- | --- | --- |
| Multi-carrier configuration change count | 4 | A change count associated with a particular configuration for all possible carriers in the network. Incremented by 1 upon each update. |
| MC configuration broadcast flag | 1 | 0: only broadcast serving BS specific info 1: broadcast multi-carrier definition fro all available carriers in the network. |
| If (MC configuration broadcast flag == 1) { |  |  |
| Number of Carrier Groups | 4 | Groups of contiguous carriers |
| Uniformity Flag | 1 | 0: All Carriers have the same Protocol Version, Carrier Type Duplexing Mode and Transmit Power. 1: otherwise |
| Physical Carrier Index of Current Carrier | 6 | The carrier that ABS broadcast this message |
| For (i=1; i<= Number of Carrier Groups; i++){ |  |  |
| Multi-Carrier Configuration Index | 6 | Index associated to Table 648 |
| Start Frequency Assignment Index | 6 | Frequency Assignment Index of the first carrier in carrier group #i |
| Number of Carriers | 4 | Number of carriers in carrier group #i |
| For (j=1; j <= Number of Carriers; j++) { |  |  |
| Physical Carrier Index | 6 |  |
| Preamble Index | 6 |  |
| If (Uniformity Flag == 1) ¦ ¦ (i == 1 && j == 1) ) { |  |  |
| Protocol Version | 2 | “00” IEEE 802.16-2009 only mode “01” IEEE802.16m and IEEE 802.16-2009 mixed mode “10” IEEE 802.16m only mode “11” Reserved |
| Carrier Type | 1 | “0” for fully configured “1” for partially configured |
| Duplexing Mode | 1 | “0” for TDD “1” for FDD |
| Transmit Power | 6 | Maximum transmit power level |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Serving BS Carrier Number | 6 | //start broadcasting serving BS specific info |
| For (i=0; i<Serving BS Carrier Number-1; i++){ |  |  |
| Physical Carrier Index | 6 |  |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) |  |  |
| { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m MAC version. |
| Preamble Index | 6 |  |
| } |  |  |
| } |  |  |

15.2.6.10 AAI_NBR-ADV

…….

Each AAI_NBR-ADV message carries

- AAI_NBR-ADV change count

- number of total cell types

- segment information for this AAI_NBR-ADV message

- system information of ABSs from one or more cell types, which is of variable size. For each cell type, shared configuration parameters are carried as follows

- FA information

- BW information

- Duplex mode

- Carrier index (6 bits, refer to the “Multi-carrier configuration index” defined in AAI_MC-ADV)

- Number of MAC protocol versions supported (4 bits)

- list of supported MAC protocol versions (4 bits per entry)

Within each cell type, a list of ABSs are provided and the following parameters are carried for each ABS

- Index of neighbor ABS in the whole neighbor list maintained by serving ABS

- 48bit BS-ID

- ABS A-Preamble index

- indication whether full system information or partial information is carried for this ABS, which includes

- SFH information

- ESI information

- Index to MC configuration,

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI_NBR-ADV.

Table XXX—AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell types | 3 |  |
| Cell type in this message | 3 |  |
| Total Number of Segments | TBD | Number of segments for this cell type |
| Segment Index | TBD | Segment index of this message in the specific cell type |
| BS number M | TBD | Total number of BSs to be included in this segment |
| Physical carrier index | 6 | Consistent with AAI_MC-ADV “carrier configuration index” |
| Number of MAC protocol versions supported: P | 4 | Greenfield: 1 Mixed-mode: >1 |
| For (i=0; i<P; i++) |  |  |
| { |  |  |
| MAC protocol version | 4 | Consistent with REV.2 definition, with new MAC protocol version 9 defined for 16m. |
| } |  |  |
| for (i=0; i<M; i++) |  |  |
| { |  |  |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 |  |
| A-PREAMBLE info | TBD |  |
| SFH_encoding_format | 2 | 00: full Subpkt information 01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket or extended broadcast information. For macrocell ABS, the bitmap shall be 1111. |
| If( SFH_encoding_format =00 |  | //encoding format type-1 |
| { |  |  |
| If( Control_bitmap[0] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[1] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt2 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[2] ==1 ) |  |  |
| { |  |  |
| SFH Subpkt3 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[3] ==1 ) |  |  |
| { |  |  |
| Extended broadcast info |  |  |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =01 |  |  |
| { |  |  |
| Delta information |  | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| } | — | — |
| } | — | — |

-------------------------------------------text #1 end----------------------------------------------