| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Management Messages (15.2.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Xiangying Yang, Avishay Shraga Joey Chou Changhong Shan Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.4” |  |
| Abstract | This contribution proposes MAC management messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Control Messages (15.2.4)

Xiangying Yang, Avishay Shraga, Joey Chou, Changhong Shan

Intel

- Introduction

This contribution proposes the MAC control messages.

- Proposed text

----------------------------------------------- Start of the proposed text ------------------------------------------

15.2.4 MAC Control messages

The peer-to-peer protocol of MAC layers in ABS and AMS communicate using the MAC control messages to perform the control plane functions. MAC control messages shall be carried in a MAC PDU to be transported in broadcast, unicast or random access connections. There are two types of unicast control connections – Basic connection and Primary connection. HARQ shall be enabled for MAC control messages sent on the unicast control connections. Encryption may be enabled for unicast MAC control messages. MAC control messages may be fragmented. MAC control messages shall not be sent using multiplexing extended header. Table 675 lists the MAC control messages that shall be defined in the ASN.1 format, as shown in <<<Appendix X>>>. The indication to the receiver whether the PDU is encrypted is indicated by the EC=1 in FEH extended header. Whether the encryption is applied on a management message or not shall be determined by the message type and MAC procedure context, which is define in Table-1. A messages included in a PDU whose EC bit value does not match the combined message type and corresponding context defined in the Table-1 shall be discarded. Encrypted and non encrypted MAC control messages shall not be sent in the same PDU.

Table 675: MAC Control Messages

| Message Names | Message Description | Security | Connections |
| --- | --- | --- | --- |
| AAI_SCD | System configuration descriptor | N/A | Broadcast |
| AAI_SII-ADV | Service Identity Information Advertisement | N/A | Broadcast |
| AAI_RNG-ACK | Aggregated CDMA Ranging Acknowledge | N/A | Broadcast |
| AAI_RNG-ACK | Ranging acknowledgement | N/A | Broadcast |
| AAI_RNG-REQ | Ranging Request | Null: during ranging procedure when there is no SA already established or pre-updated. CMAC: all other cases | Initial Ranging or Basic |
| AAI_RNG-RSP | Ranging Response | Null: during ranging procedure when there is no SA already established or pre-updated. Encrypted/ICV: all other cases | Basic |
| AAI_REG-REQ | Registration Request | Encrypted/ICV | Primary |
| AAI_REG-RSP | Registration Response | Encrypted/ICV | Primary |
| AAI_SBC-REQ | SS Basic Capability Request | null | Basic |
| AAI_SBC-RSP | SS Basic Capability Response | null | Basic |
| AAI_DREG-REQ | Deregistration Request | Encrypted/ICV | Basic |
| AAI_DREG-CMD | Deregistration Command | Encrypted/ICV | Basic |
| AAI_DSA-REQ | Dynamic Service Addition Request | Encrypted/ICV | Primary |
| AAI_DSA-RSP | Dynamic Service Addition Response | Encrypted/ICV | Primary |
| AAI_DSA-ACK | Dynamic Service Addition Acknowledge | Encrypted/ICV | Primary |
| AAI_DSC-REQ | Dynamic Service Change Request | Encrypted/ICV | Primary |
| AAI_DSC-RSP | Dynamic Service Change Response | Encrypted/ICV | Primary |
| AAI_DSC-ACK | Dynamic Service Change Acknowledge | Encrypted/ICV | Primary |
| AAI_DSD-REQ | Dynamic Service Deletion Request | Encrypted/ICV | Primary |
| AAI_DSD-RSP | Dynamic Service Deletion Response | Encrypted/ICV | Primary |
| AAI_DSX-RVD | DSx Received | Encrypted/ICV | Primary |
| AAI_PKM-REQ | Privacy Key Management Request | Before AK is derived: null After AK is derived: CMAC | Primary |
| AAI_PKM-RSP | Privacy Key Management Response | Before AK is derived: null After AK is derived: CMAC | Primary |
| AAI_ARQ-Feedback | Stand-alone ARQ Feedback | Encrypted/ICV | Basic |
| AAI_SLP-REQ | Sleep Request | Encrypted/ICV | Basic |
| AAI_SLP-RSP | Sleep Response | Encrypted/ICV | Basic |
| AAI_TRF-IND | Traffic Indication | N/A | Broadcast |
| AAI_NBR-ADV | Neighbor Advertisement | Null: in unicast N/A: in broadcast | Primary or broadcast |
| AAI_SCN-REQ | Scanning Interval Allocation Request | Encrypted/ICV | Basic |
| AAI_SCN-RSP | Scanning Interval Allocation Response | Encrypted/ICV | Basic |
| AAI_SCN-REP | Scanning Result Report | Encrypted/ICV | Primary |
| AAI_HO-REQ | AMS Handover Request | Encrypted/ICV | Basic |
| AAI_HO-CMD | ABS Handover Command | Encrypted/ICV | Basic |
| AAI_HO-IND | AMS Handover Indication | Encrypted/ICV | Basic |
| AAI_TRF_IND-REQ | Traffic indication request | Encrypted/ICV | Basic |
| AAI_TRF_IND-RSP | Traffic indication response | Encrypted/ICV | Basic |
| AAI_L2-XFER | AAI L2 Transfer | Encrypted/ICV | Primary |
| AAI_PAG-ADV | BS Paging Advertisement | N/A | Broadcast |
| AAI_UL_Power_Control | UL power control | N/A | Broadcast |
| AAI-MC-ADV | Multi-Carrier Advertisement | N/A | Broadcast |
| AAI_MC-REQ | Multi-Carrier Request | Null | Basic |
| AAI_MC-RSP | Multi-Carrier Response | Null | Basic |
| AAI_CM-CMD | Carrier Management Command | Encrypted/ICV | Basic |
| AAI_CM-IND | Carrier Management Indication | Encrypted/ICV | Basic |
| AAI_RES-CMD | Reset commend | Before authentication :Null After authentication : Encrypted/ICV | Basic |
| PGID Info | paging group advertisement | N/A | Broadcast |