| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Key usage during 16m handover (15.2.5.2.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Shraga Avishay Xiangying Yang Changhong Shan Intel Corporation Youngkyo Baek Samsung Corporation Chengyan Feng Yang Liu ZTE Corporation Gene Beck Hahn Kiseon Ryu Ronny Yongho Kim LG Electronics | E-mail: avishay.shraga@intel.com xiangying.yang@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.5.2.4” |  |
| Abstract | This contribution proposes the texts for AMS privacy section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

key usage during 16m handover (15.2.5.2.4)

Avishay Shraga, Xiangying Yang, Changhong Shan

Intel

Youngkyo Baek

Samsung Corporation

Chengyan Feng, Yang Liu

ZTE Corporation

Gene Beck Hahn, Kiseon Ryu,Ronny Yongho Kim

LG Electronics

# 1. Introduction

HO procedure requires security update to achieve seamless operation.

# 2. Text Proposal

Insert the underline text in Section 15.2.5.2.4.

======================== Start of Proposed Text =====================

15.2.5.2.4 Key Usage

15.2.5.2.4.1 TEK usage

15.2.5.2.4.2 TEK update

15.2.5.2.4.3 TEK update after Re-authentication

15.2.5.2.4.4 Key update during Handover

During Handover (with handover process optimization bitmap bit#1=1 “omit PKM”), the AMSID* shall be updated, then AK, CMAC keys and TEKs shall be derived by the target ABS and AMS respectively based on the updated AMSID* as described in 15.2.5.2. In particular,

- AMSID* derivation uses the i) Nonce associated with the current active security context, which is known to both the network and AMS before handover, and ii) target ABS BSID

- In AK derivation, the CMAC_KEY_COUNT is managed on AMS and target ABS sides in the same way as in Section 7.2.2.2.6.1 and 7.2.2.2.9.1.

- In TEK derivation, COUNTER_TEK  is set to be 0 and 1, in order to generate two new TEKs to be used at the target ABS. Corresponding EKS is also reset to be 0 and 1 respectively.

When Seamless_Handover Flag = 1 in AAI_HO-CMD, the AMS and target ABS may use derived TEKs for the target ABS to resume data communications before network reentry procedure finishes.

When the Network_Reentry_Mode = 1, for which the AMS is to maintain communications with serving ABS during network reentry at the target ABS, the AMS shall manage two set of key context for AK, CMAC keys and TEKs, where the context associated with the serving ABS is used to maintain communications with serving ABS until Disconnection Time, and the “new” context associated with the target ABS is used to perform required network reentry procedures with target ABS. The AMS discards key context associated with the serving ABS when the network reentry procedure finishes. The serving ABS discards this MS’s key context (along with other MAC context associated with the AMS) upon either the expiration of Resource_Retain_Time or a HO-Complete signaling from target ABS via backbone.

============================== End of Proposed Text ===============

# 4. References

[1] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[3] IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010R2, “IEEE 802.16m Amendment Working Document”