| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on CDMA IE Allocation (15.2.11.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Xiangying Yang Jie Hui Yuan Zhu Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.11.2” |  |
| Abstract | This contribution proposes text for quick access message and CDMA IE Allocation format |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on CDMA IE Allocation (AWD-15.2.11)

Xiangying Yang, Jie Hui, Yuan Zhu

Intel

- Introduction

The text change propose removing Masking Indicator in CDMA_IE_allocation. Scrambling this 4-bit indicator can cause higher misdetection probability, while overhead saving is very marginal. It is suggested that this information is to carried in a regular field in the CDMA_IE_allocaiton

- Proposed Test

Text proposal

--------------------------------text start-----------------------------------------

15.2.11.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table 71635. CDMA Allocation A-MAP IE is used for allocation of bandwidth to a user that requested bandwidth using a ranging code or BR code. AMS decodes the IE and checks the MCRC field by its specific 12-bit RA-ID and 4-bit Masking Indicator. The RA-ID is calculated by a hash function with the AMS' random access attributes (i.e., frame number index, sub-frame number index, opportunity index and code index). Masking indicator indicates the identifier used for CRC masking, as shown in Table 717. The allocation shall be in the first UL subframe relevant to the A-MAP region regardless of DL/UL ratio and Nsubframe, A-MAP. ABS requests adjustment of timing and power to AMS by using Timing Adjust, Power Level Adjust and Offset Frequency Adjust in CDMA Allocation A-MAP IE for Ranging. The maximum number of the HARQ retransmission is TBD.

15.3.6.5.2.12

Table 716—CDMA Allocation A-MAP IE format

| SyntaxSize | (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  | — |
| A-MAP IE Type | 4 |  |
| Allocation Type | 2 | 00: unsynchronized ranging 01: synchronized ranging 10: BWREQ 11: resved |
| If MCRC is masked with RAID and masking indicator for BR Allocation Type=10{ |  |  |
| Resource assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level.Signed in units of 1 dB |
| Reserved | TBD |  |
| } |  |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging Allocation Type=00 or 01 { |  |  |
| Resource assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs |
| Power Level Adjust | 4 | Relative change in transmission power level.Signed in units of 1 dB |
| Offset Frequency Adjust | 6 | Relative change in transmission frequency.Signed in units of Hz |
| Reserved | TBD |  |
| } |  |  |
| MCRC | 16 | CRC masked by 12-bit RA-ID and Masking Indicator |
| } |  |  |

--------------------------------text end-----------------------------------------