| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Operation in Multi-carrier Operation in 802.16m (15.2.7.2.10.2) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.7.2.10.2” |  |
| Abstract | This contribution proposes amendment text describing paging operation in multi-carrier system for ABS Type in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Paging Operation in Multi-carrier Operation

- in 802.16m (15.2.7.2.10.2)

Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty

- Intel Corporation

- Introduction

This contribution proposes amendment text describing the paging operation when a 16m ABS supports multiple carriers.

.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D1

========================== Start of Proposed Text ==============================

          - Idle Mode

15.2.7.2.10.2.xx Paging operation

When an ABS implements multiple carriers it transmits the paging message for an idle mode AMS in one of its carriers. To determine the carrier index the ABS broadcasts the carrier hash modulo (HM) parameter HMC in the Paging-Info message. An idle mode AMS determines the carrier index (paging offset-carrier) within AMS’s paging listening interval where its paging message is transmitted using the following equations:

paging offset-carrier = AMS Temporary ID modulo HMC

The carriers are indexed in the increasing order of their center frequency, i.e., the carrier with lowest center frequency has the smallest index and the carrier with highest center frequency has the largest index. The value of HMC is the number of carriers in a BS that are used for transmitting paging message for an idle mode MS. This can be all the carriers or a subset of carriers used by a multi-carrier BS.

The contents of PAG-Info message are shown in Table 1.

Table 1: Information transmitted by the network to enable procedures of this invention.

| Field | Size | Notes |
| --- | --- | --- |
| Paging carrier indicator | 1 | Indicates if the carrier transmitting this MC-PAG-Info message is a carrier that transmits paging message for idle mode MSs or not: 0: this carrier does not transmit paging message for idle mode MS. 1: this carrier transmits paging message for idle mode MSs. |
| If (Paging carrier indicator ==1) { |  |  |
| Number of carriers used for paging message transmission (HMC) | N | Either all the carriers or a subset of carriers of a BS. N = roof (log2(HMCmax)) where HMCmax is the maximum number of carriers in a BS |
| } |  |  |
| For (i=1, i<=HMC, i++) |  |  |
| Information about carrier i | M | M bits are used to convey the information about each carrier. This information include different parameters that is required by an idle mode MS to receive the paging message on a carrier such as center carrier frequency, bandwidth of the carrier etc. If the MS receives the such information during network entry or neighbor advertisement (NBR-ADV) or multicarrier advertisement message (MC-ADV) messages, then instead of transmitting the entire information it may be sufficient to just transmit the index of a table that contains the detailed information for each carrier. This information can also be specified using a bitmap in which case instead of the for loop there is a M-bit map. The bits of this bitmap that are set indicates the carriers that are used as paging carries. |
| } |  |  |
| Carrier information change count | K | In the case that the MS receives the detailed carrier information during network entry or NBR-ADV or MC-ADV, an MS uses this field to determine if it has latest information of such kind. If the carrier information change count received by an MS in the MC-PAG-Info message is higher that the one stored in the MS, then the MS uses appropriate procedures to receive the latest information, e.g. receives the latest NBR-ADV if NBR-ADV message is used to transmit such information |

========================== End of Proposed Text ==============================