| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to define MAC control messages and extended headers for MIMO feedback (15.2.4) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Sangheon Kim, Hokyu Choi, Heewon Kang Samsung Electronics Huaning Niu, Yuan Zhu Intel Corporation Wookbong Lee LG Electronic Luciano Sarperi Fujitsu | d.mazzarese@samsung.com huaning.niu@intel.com wbong@lge.com luciano.sarperi@uk.fujitsu.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes to define MAC control messages for the feedback of single-BS MIMO and multi-BS MIMO CQI and CSI, rather than a MAC header. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal to define MAC control messages and extended headers for MIMO feedback (15.2.4)

David Mazzarese, Sangheon Kim, Hokyu Choi, and Heewon Kang

Samsung Electronics Co., Ltd.

Huaning Niu, Yuan Zhu

Intel Corporation

Wookbong Lee

LG Electronics

Luciano Sarperi

Fujitsu

# Introduction

Feedback of MIMO channel state information without the use of a fast feedback channel has benefits, since it allows to take advantage of link adaptation on the uplink and it doesn't have stringent size constraints. So 802.16m should be equipped with IEs for single-BS MIMO feedback and multi-BS MIMO feedback using MAC messages.

This contribution proposes to move the MIMO channel feedback header currently in D1 to MAC control messages and extended headers for the feedback of the single-BS MIMO and multi-BS MIMO feedback information. The use of a header for MIMO feedback is not appropriate, since it would complicate the usage of GMH and CMH, and possibly require to re-design the GMH and CMH. We define the MIMO feedback IE to be sent in the AAI_SingleBS_MIMO_FBK and AAI_MultiBS_MIMO_FBK MAC control messages, or using extended headers for fixed-length feedback contents such as transmit correlation matrix and wideband CQI.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Delete section 15.2.2.1.3 MIMO channel feedback header, and its contents >>

-------------------------------  Start of Text Proposal 1 ---------------------------------------------------

15.2.2.1.3 MIMO channel feedback header

The header shown in Table 656 includes MIMO channel feedback header.

Table 656—AAI_FFR-REP message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| if (ICT == 0b00, 0b01, 0b10, 0b10 or 0b11) { |  | ICT is defined in Feedback Polling A-MAP IE |
| if (ICT ==0b10 or 0b11){ |  | Co-MIMO or CL-MD |
| CQI | [4] |  |
| } |  |  |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| Temp_BSID | [TBD] | Diversity set member ID |
| PMI | [3 to 6] | PMI from the rank-1 base codebook [or base codebook subset] |
| If (ICT==0b00 or 0b01) { |  |  |
| Additional measurement metric |  |  |
| PMI_coordination_subset | [1] | Set of PMIs from the rank-1 base codebook or base codebook subset |
| } |  |  |
| If (ICT==0b10 or 0b11 ) { |  | ICT is defined in Feedback Polling A-MAP IE |
| If (CPI == 0b0){ |  | CPI is defined in Feedback Polling A-MAP IE |
| CPMI | [TBD] | Concatenating PMI for neighboring cells |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal 1  ---------------------------------------------------

<< Add two rows at the end of Table 675 on page 29 line 53 in section 15.2.4 >>

-------------------------------  Start of Text Proposal 2 ---------------------------------------------------

15.2.4 MAC Control messages

Table 675—MAC Control Messages

| Message name | Message description | Connection |
| --- | --- | --- |
| … | … | … |
| AAI_SingleBS_MIMO_FBK | Single-BS MIMO feedback | Basic |
| AAI_MultiBS_MIMO_FBK | Multi-BS MIMO feedback | Basic |

-------------------------------  End of Text Proposal 2  ---------------------------------------------------

<< Add two rows at the end of Table 658 on page 11 line 4 in section 15.2.2 >>

-------------------------------  Start of Text Proposal 3 ---------------------------------------------------

15.2.2.2 Extended header formats

Table 658—Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.2.2.1 |
| TBD | Fragmentation extended header | See 15.2.2.2.2 |
| TBD | Multiplexing extended header | See 15.2.2.2.4 |
| TBD | Sleep Control extended header | See <<<15.2.3.x>>> |
| TBD | Correlation matrix feedback extended header | See 15.2.2.2.7 |
| TBD | MIMO feedback extended header | See 15.2.2.2.8 |
| TBD | Reserved |  |

-------------------------------  End of Text Proposal 3  ---------------------------------------------------

<< Create section “15.2.2.2.7 Correlation matrix feedback extended header” under section “15.2.2 Extended header formats” on page 21 line 58 >>

-------------------------------  Start of Text Proposal 4 ---------------------------------------------------

15.2.2.2.7 Correlation Matrix Feedback Extended Header (CMFEH)

This Correlation matrix feedback extended header format is defined in Table X1. This header may be used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas.

Table X1 - Correlation matrix feedback extended header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CMFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | CMFEH type |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal 4  ---------------------------------------------------

<< Create section “15.2.2.2.8 Correlation matrix feedback extended header” under section “15.2.2 Extended header formats” on page 21 line 58 >>

-------------------------------  Start of Text Proposal 5 ---------------------------------------------------

15.2.2.2.8 MIMO feedback extended header (MFEH)

This MIMO feedback extended header format is defined in Table X2. This header is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback only the wideband information for MFM 0, 1, 4 or 7, or only the subband information for one subband for MFM 2, 3, 5 or 6.

Table X2 – MIMO feedback extended header format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MFEH() { |  |  |
| LAST | 1 | Set to “1” if last EH, else, set to “0” |
| Type | TBD | MFEH type |
| MFM | 3 |  |
| If (MFM == 0 or 1){ |  |  |
| Wideband CQI | 4 |  |
| Wideband STC rate | 1 to 2 | Depending on MaxMt |
| } |  |  |
| If (MFM == 4 or 7){ |  |  |
| Wideband CQI | 4 |  |
| Wideband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 2){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 3){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband STC rate | 1 to 3 | Depending on MaxMt |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| If (MFM == 5){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband stream index | 1 to 3 | Depending on MaxMt |
| } |  |  |
| If (MFM == 6){ |  |  |
| Subband index | 3 to 5 | Depending on bandwidth |
| Subband CQI | 4 |  |
| Subband PMI | 3 to 6 | Depending on Nt and CS |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal 5  ---------------------------------------------------

<< Create section “15.2.6.31 AAI_SingleBS_MIMO_FB” under section “15.2.6 AAI MAC Management Messages” on page 86 line 48 >>

-------------------------------  Start of Text Proposal 6 ---------------------------------------------------

15.2.6 AAI MAC Management Messages

…

15.2.6.31 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table Y1. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas.

Table Y1. AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| If (Transmit_Correlation_Matrix == 1){ |  | Only for 8Tx |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| }else{ |  |  |
| MFM_bitmap | 4 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with the MFM requested in a Feedback Polling IE. Only MFM 2, 3, 5 and 6 are allowed. |
| If (LSB #0 MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 2, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| } |  |  |
| } |  |  |
| If (LSB #1 MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 3, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If (LSB #2 MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Stream index | 1 to 3 | Best stream index of m-th subband in bitmap, according to CS and MaxMt |
| } |  |  |
| } |  |  |
| If (LSB #3 MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If ((LSB #1 MFM_bitmap == 1) or (LSB #3 MFM_bitmap == 1)){ |  | MFM 3 or MFM 6 |
| If (CM == 0b1){ [TBD] |  |  |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal 6 ---------------------------------------------------

<< Create section “15.2.6.32 AAI_MultiBS_MIMO_FBK” under section “15.2.6 AAI MAC Management Messages” on page 86 line 48 >>

-------------------------------  Start of Text Proposal 7 ---------------------------------------------------

15.2.6 AAI MAC Management Messages

…

15.2.6.32 AAI_MultiBS_MIMO_FBK

The AAI_ MultiBS _MIMO_FBK message format is defined in Table Y2. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting multi-BS MIMO feedback.

Table Y2. AAI_MultiBS_MIMO_FBK message format

| Name | Length (bit) | Description |
| --- | --- | --- |
| AAI_MultiBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| if (ICT ==0b10 or 0b11){ |  | Co-MIMO or CL-MD |
| CQI | 4 |  |
| } |  |  |
| for (i = 1; i <= N_multiBS_reports; i++) { |  |  |
| Temp_BSID | [TBD] | Diversity set member ID |
| PMI | 3 to 6 | PMI from the rank-1 base codebook or base codebook subset |
| If (ICT==0b00 or 0b01) { |  |  |
| Additional measurement metric | [TBD] | [TBD] |
| PMI_subset_size | 1 | Indication whether 1 PMI or a PMI set is feedback 0b0: 1 PMI 0b1: multiple PMIs |
| If (PMI_subset_size == 0b1){ |  |  |
| PMI_coordination_subset | 1 | Set of PMIs from the rank-1 base codebook or base codebook subset 0b0: correlation level n1 (cf Table 54) 0b1: correlation level n2 (cf Table 54) |
| } |  |  |
| } |  |  |
| If (ICT==0b10 or 0b11 ) { |  | ICT is defined in Feedback Polling A-MAP IE |
| If (CPI == 0b0){ |  | CPI is defined in Feedback Polling A-MAP IE |
| CPMI | [TBD] | Concatenating PMI for neighboring cells. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

-------------------------------  End of Text Proposal 7  ---------------------------------------------------