| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for the single-BS MIMO feedback IE (15.2.2.1.3) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | David Mazzarese, Hokyu Choi Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes a single-BS MIMO feedback IE |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for the single-BS MIMO feedback IE (15.2.2.1.3)

David Mazzarese et Al.

Samsung Electronics

# Introduction

Feedback of MIMO channel state information without the use of a fast feedback channel has benefits, since it allows to take advantage of link adaptation on the uplink and it doesn't have stringent size constraints. So 802.16m should be equipped with an IEs for single-BS MIMO feedback.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

# Text proposal for inclusion in the 802.16m amendment

<< Create section “15.2.6.31 AAI_SingleBS_MIMO_FB” under section “15.2.6 AAI MAC Management Messages” on page 86 line 48 >>

-------------------------------  Start of Text Proposal  ---------------------------------------------------

15.2.6 AAI MAC Management Messages

…

15.2.6.31 AAI_SingleBS_MIMO_FBK

Table Y1. Single-BS MIMO Feedback IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MIMO_feedback_IE() { |  |  |
| Type | 1 | 0 = feedback IE for single-BS MIMO 1 = feedback IE for multi-BS MIMO This bit is always set to '0' |
| MFM_bitmap | 8 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with the MFM requested in a Feedback Polling IE. |
| If (MFM_bitmap bit #0 == 1){ |  | MFM 0 as specified in Feedback Polling A-MAP IE |
| For (FPI=0; FPI < FPCT; FPI ++){ |  |  |
| Wideband CQI | 4 | MCS of frequency partition FPI |
| Wideband STC_Rate | 1 to 3 | STC_Rate for MFM 0, according to MaxMt |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #1 == 1){ |  | MFM 1 as specified in Feedback Polling A-MAP IE |
| For (FPI=0; FPI < FPCT; FPI ++){ |  |  |
| Wideband CQI | 4 | MCS of frequency partition FPI |
| Wideband STC_Rate | 1 to 3 | STC_Rate for MFM 1, according to MaxMt |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #2 == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 2, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #3 == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 3, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #4 == 1){ |  | MFM 4 as specified in Feedback Polling A-MAP IE |
| For (FPI=0; FPI < FPCT; FPI ++){ |  |  |
| Wideband CQI | 4 | MCS of frequency partition FPI |
| Wideband PMI | 1 to 3 | Wideband PMI, according to CS |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #5 == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Stream index | 1 to 3 | Best stream index of m-th subband in bitmap, according to CS and MaxMt |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #6 == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap, according to CM and CS |
| } |  |  |
| } |  |  |
| If (MFM_bitmap bit #7 == 1){ |  | MFM 7 as specified in Feedback Polling A-MAP IE |
| For (FPI=0; FPI < FPCT; FPI ++){ |  |  |
| Wideband CQI | 4 | MCS of frequency partition FPI |
| Wideband PMI | 1 to 3 | Wideband PMI, according to CS |
| } |  |  |
| } |  |  |
| If ((Transmit_Correlation_Matrix == 1) or (CM == 0b1)){ |  |  |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Differential codebook feedback mode is not supported in the MIMO feedback IE.

-------------------------------  End of Text Proposal   ---------------------------------------------------