| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_NBR-REQ message format (15.2.6.31) |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0044: IEEE 802.16 Working Group Working Group Letter Ballot #30 Section 15.2.6 AAI MAC Management Messages Section 15.2.6.31 AAI_NBR-REQ |  |
| Abstract | This contribution provides detailed format of the AAI_NBR-REQ message |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_NBR-REQ message format (15.2.6.31)

Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

This contribution provides detailed format of the AAI_NBR-REQ message.

In P802.16m/D1, there is no method to trigger unicast neighbor advertisement message. We introduce new message, that is, the AAI_NBR-REQ message in the MS side, so that an AMS can send the AAI_NBR-REQ message to request the unicast AAI_NBR-ADV message.

# Proposed Text

Add following sections in the AWD.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

15.2 Medium access control

15.2.6 AAI MAC Management Messages

15.2.6.31 AAI_NBR-REQ

An AMS may send the AAI_NBR-REQ message to request the AAI_NBR-ADV message. The request should not contain the BSIDs for which full information is already provided in the AAI_NBR-ADV message.

The AAI_NBR_REQ message shall include following parameters:

- N_Neighbor_BS: 8 bits

  - Number of neighboring BS for the AAI_NBR-ADV message.

- LSB of Requested BS ID: 16 bits

  - 16-bit LSB of unique 48-bit identifier of neighbor BS

------------------------------------------------------ End of Proposed Text ------------------------------------------------------