| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Orderly Power Down Procedures in 802.16m (15.2.5.x and 15.3.12.4.1.3) |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek Inc. Ming-Hung Tao, Ying-Chuan Hsiao ITRI Hyunjeong Kang Samsung Electronics Kiran Kuchi, J. Klutto Milleth CeWit Zexian Li Nokia Ronny Yongho Kim, Jin Lee LG Electronics Muthaiah Venkatachalam and Shantidev Mohanty Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30 on “ P802.16m/D1”: Target topic: “15.2.5.x and 15.3.12.4.1.3” |  |
| Abstract | This contribution proposes amendment text describing orderly power down operation in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Deep Idle Mode Procedures in 802.16m

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Hyunjeong Kang

Samsung Electronics

Kiran Kuchi, J. Klutto Milleth

CeWit

Zexian Li

Nokia

Ronny Yongho Kim, Jin Lee

LG Electronics

Muthaiah Venkatachalam and Shantidev Mohanty

Intel Corporation

Introduction

The current IEEE P802.16m/D1 document outlines the procedures followed by an AMS during orderly power down when the AMS is in idle mode using the location update procedures as described in “15.3.12.4.1.3 Power down update”

There is a situation similar to power down where AMS goes into hibernation mode or handoff to other RAT. In this situation AMS becomes unavailable to monitor and receive paging message from the network. However, as long as AMS’s context is retained in the network during this situation, AMS can expedite network re-entry from this situation. Such an idle mode operation without paging is called as deep idle mode.

This contribution proposes the following concepts:

Procedures to be followed by an AMS for deep idle mode when the AMS is in connected mode and idle mode.

Deep idle mode procedures are performed in connected and idle mode because the AMS moves to another radio-access technology (RAT) or goes into hibernation mode.

For the first proposal, this contribution proposes that the AMS sends a DREG-REQ message to enter deep idle mode in connected mode. To differentiate the use of DREG-REQ message usage during idle mode initiation and deep idle mode in connected mode, the AMS uses a de-registration purpose code. The following values are defined for deregistration purpose code.

Deregistration code 1: Deregistration to enter idle mode

Deregistration code 2: Deregistration to enter deep idle mode

For the second proposal, this contribution proposes that when an AMS performs deep idle mode in either connected mode or idle mode, the network does not release AMS’s context.

For the last proposal, this contribution proposes the network re-entry procedure from the deep idle mode.

Deep idle mode for inter-RAT operation and hibernation:

While in connected mode if the AMS wants to enter to either another RAT or hibernation, it sends a DREG-REQ message with the Deregistration code 3 as described below.

Deregistration code 3: Deregistration to enter deep idle mode When an ABS receives a DREG-REQ message with Deregistration code 3, it deregisters the AMS and contact the appropriate network entity to store AMS’s context that can be used to expedite AMS’s network re-entry at a later point of time when the AMS returns to 16m from the other RAT or the hibernation.

While in idle mode if the AMS wants to enter to another RAT or hibernation, it performs a location update with location update purpose as transition to deep idle mode as described below.

Location update purpose code 3: Location update to transit to deep idle mode

When an ABS receives location update request with location update purpose code 3, it performs location update for the AMS and contact the appropriate network entity to store AMS’s context that can be used to expedite AMS’s network re-entry at a later point of time when the AMS returns to 16m from the other RAT or hibernation.

References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

Text proposal for inclusion in the P802.16m/D1

========================== Start of Proposed Text ==========================

[Note to Editor: Add the text as follows after line 11 in Page 212 in 15.3.12.4.1.3]

Remedy1

[Insert texts as suggested below in Page213 Line12]

15.2.X Deregistration with content retention (DCR) mode

Deregistration with content retention (DCR) mode is a mode in which an AMS is deregistered from the network while its context is kept in a network entity until the idle mode retention timer is valid.

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network re-entry. Deregistration identifier (DID) is assigned to AMS with paging cycle and paging offset value of 0.

DID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

15.2.X.1 DCR initiation in connected state

AMS may initiate DCR mode by transmitting an AAI_DREG-REQ message with the De-registration_Request_Code parameter= 0x04; request for AMS deregistration from serving ABS and retention of AMS’s connection information. The AMS may request the network to retain specific AMS service and operational information for DCR mode management purposes through inclusion of the Idle Mode Retain Information element in the AAI_DREG-REQ.  When the ABS decides to allow AMS’s DCR mode request, the ABS shall send an AAI_DREG-CMD with action code 0x08 in response to the AAI-DREG-REQ message. When ABS decides to reject AMS’s deep idle mode request, the ABS shall send an AAI_DREG-CMD with action code 0x09 in response to the AAI_DREG-REQ message.  In the AAI-DREG-CMD, DID shall be assigned which is used for network re-entry from DCR mode to identify the AMS.

15.2.X.2 DCR mode initiation from idle mode

AMS may initiate DCR mode in idle mode state by performing the location update with ranging purpose code of 0x05 in AAI_RNG-REQ message for transition to DCR mode.

When the ABS decides to allow AMS’s DCR mode request, the ABS shall send a AAI_RNG-RSP with action code 0x04. When ABS decides to reject AMS’s DCR idle mode request, the ABS shall send an AAI_RNG-RSP with action code 0x05.

Upon successful DCR mode change request, the network shall initiate DCR mode operation by retaining AMS’s information until the idle mode retention timer is valid. At the time of DCR mode change, paging cycle, paging offset, paging controller values shall be updated to be 0 for the AMS and the ID of the network entity that assigns/retains the context  shall be used to uniquely identify DCR mode AMS.

15.2.X.3 DCR mode extension

An AMS in DCR mode can extend its idle mode retention timer by sending a RNG-REQ message to extend the timer before it expires.

15.2.X.4 Network re-entry from DCR mode

For the network re-entry from DCR mode, the AMS shall initiate network re-entry with the ABS by sending an AAI_RNG-REQ message with ranging purpose code of 0x06 and the ID of the network entity that assigns/retains the context. Rest of re-entry procedure shall be performed same as network reentry from idle mode described in section 15.3.12.5.

15.2.X.5 DCR mode termination

DCR mode may only be terminated through:

- AMS reentry to the network

- Expiration of the idle mode retention timer

Remedy2 – Existing Text [Black], Add [Red]

[Modify texts as suggested below in Page71 Line56]

15.2.6.19 AAI_DREG-REQ message

The following parameter shall be included in AAI_DREG-REQ message :

Management Message Type

De-registration_Request_Code

  - 0x00 : AMS deregistration request from ABS and network

  - 0x01 :  request for AMS deregistration from serving ABS and initiation of AMS idle mode

  - 0x02 : response for the unsolicited AAI_DREG-CMD message with action code 0x05 by the ABS

  - 0x03 : reject for the unsolicited AAI_DREG-CMD message with action code 0x05 by the ABS.

  - 0x04 : request for AMS deregistration from serving ABS to enter DCR mode

  - 0x04-0x05 – 0x015 :  reserved

When De-registration_Request_Code is 0x04, the following parameters shall be included in the AAI_DREG-REQ message.

Idle Mode Retain Information element

Remedy3 - Existing Text [Black], Add [Red]

[Modify texts as suggested below in Page72 Line39]

15.2.6.20 AAI_DREG-CMD message

The following parameter shall be included in AAI_DREG-CMD message:

Management message type

Action code

  - 0x08 :  Reserved This option is valid only in response to a AAI_DREG-REQ message with De-Registration Request Code 0x04: to allow retention of AMS’s connection information

  - 0x09 : This option is valid only in response to a AAI_DREG-REQ message with De-Registration Request Code 0x04: to reject retention of AMS’s connection information

When the action code is set to 0x08, the following parameters shall be included in AAI_DREG-CMD message.

Idle Mode Retain Information

Deregistration Identifier (DID)

Remedy4 – Existing Text [Black], Add [Red]

[Update Table 682 by inserting texts as suggested below in Page48]

Table 682 – parameters for AAI_RNG-REQ

| Name | Value | Usage |
| --- | --- | --- |
| Ranging Purpose Indication | If bit #5 is set to 1, it indicates that the AMS is initiating location update for transition to DCR mode from idle mode If bit #6 is set to 1, in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting re-entry from DCR mode Bit#7 : reserved | It shall be included when the AMS is attempting to perform re-entry, HO, location update or location update for entering DCR mode.. |

Remedy4 – Existing Text [Black], Add [Red]

[Update Table 683 by inserting texts as suggested below in Page50]

Table 683 – parameters for AAI_RNG-RSP

| Name | Value | Usage |
| --- | --- | --- |
| Location Update Response | 0x00=Success of Location Update 0x01=Failure of Location Update 0x02=Reserved 0x03=Success of location update and DL traffic pending 0x04 = Allow AMS’s DCR mode request 0x05 = Reject AMS’s DCR mode request 0x06~0xFF : Reserved | It shall be included when the ABS sends an AAI_RNG-RSP message in response to an AAI_RNG-REQ message used to perform location update, or location update for entering DCR mode |

========================== End of Proposed Text ==========================