| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Global Service Class in 802.16m |  |
| Date Submitted | 2009-08-26 |  |
| Source(s) | Jie Hui Xiangying Yang Muthaiah Venkatachalam Intel Corporation | jie.hui@intel.com Xiangying.yang@intel.com |
| Re: |  |  |
| Abstract | This proposal describes addition of QoS parameters of aGP Service into Global Service Class fields. |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional Fields of aGP Service in Global Service Class

for the IEEE 802.16m Amendment

Jie Hui, Xiangying Yang, Muthaiah Venkatachalam

Intel Corporation

- Introduction

The newly defined aGP Service and its five new QoS parameters are missing from the definition of Global Service Classes. The contribution proposes those additional text and fields for aGP Service.

The original text in 16e is “Global service class name is a rules-based, composite name parsed in a variable number of information fields of format,

for I=1, format is ISBRLSPS1R and length is five bytes;

for I=0 and S2=0 or 1, format is ISBRLSPS1S2R and length is five bytes;

for I=0 and S2=2 or 3, format is ISBRLSPS1S2S3R and length is six bytes;

for I=0 and S2=4, format is ISBRLSPS1S2S3S5R and length is six bytes;

for I=0 and S2=5, format is ISBRLSPS1S2L1S3S4R and length is seven bytes;

for I=0 and S2=6, format is ISBRLSPS1S2L1S4R and length is seven bytes;”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------------------------  Text Start  -------------------------------------------------

{editorial instruction: insert the following underlined text in section 15.3.7.1}

15.3.7.1 Global Service classes

Global service class name is a rules-based, composite name parsed in a variable number of information fields of format,

for I=1, format is ISBRLSPS1R and length is five bytes;

for I=0 and S2=0 or 1, format is ISBRLSPS1S2R and length is five bytes;

for I=0 and S2=2 or 3, format is ISBRLSPS1S2S3R and length is six bytes;

for I=0 and S2=4, format is ISBRLSPS1S2S3S5R and length is six bytes;

for I=0 and S2=5, format is ISBRLSPS1S2L1S3S4R and length is seven bytes;

for I=0 and S2=6, format is ISBRLSPS1S2L1S4R and length is seven bytes;

for I=0 and S2=7, format is ISBRLSPS1S2L1S3S6S7S8S9S10R and length is eleven bytes;

{editorial instruction: insert the following underlined rows into existing Table 718, between row “Position: S5” and the last row “Position: R”}

Table 718—Global Service Class Name Information Field Parameters

| Position | Name | Size(bits) | Value |
| --- | --- | --- | --- |
| S6 | Primary GPI | 6 | Extensible look-up Table (value 0b111111 indi-cates TLV to follow). This is available only for Uplink Grant Scheduling Type = aGP Service. This field is included when I=0 and S2=7. |
| S7 | Primary Grant Size | 6 | Extensible look-up Table (value 0b111111 indi-cates TLV to follow). This is available only for Uplink Grant Scheduling Type = aGP Service. This field is included when I=0 and S2=7. |
| S8 | Secondary GPI | 6 | Extensible look-up Table (value 0b111111 indi-cates TLV to follow). This is available only for Uplink Grant Scheduling Type = aGP Service. This field is included when I=0 and S2=7. |
| S9 | Secondary Grant Size | 6 | Extensible look-up Table (value 0b111111 indi-cates TLV to follow). This is available only for Uplink Grant Scheduling Type = aGP Service. This field is included when I=0 and S2=7. |
| S10 | Adaptation Method | 3 | Extensible look-up Table (value 0b111111 indi-cates TLV to follow). This is available only for Uplink Grant Scheduling Type = aGP Service. This field is included when I=0 and S2=7. |

-------------------------------------------------  Text end  -------------------------------------------------