| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay data forwarding Mechanism (15.5 Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Hyunjeong Kang, Mingxia Xu, Rakesh Taori, Kyungkyu Kim, Youngbin Chang, Jungje Son Samsung Electronics Co., Ltd. Anil Agiwal SISO Kaushik Josiam Samsung Telecommunications America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 |  |
| Abstract | The contribution proposes the text to define addressing mechanism for relay station. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D2 Amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Relay data forwarding Mechanism

Hyunjeong Kang, Mingxia Xu, Rakesh Taori, Kyungkyu Kim, Youngbin Chang, Jungje Son

Samsung Electronics Co., Ltd.

Anil Agiwal

SISO

Kaushik Josiam

Samsung Telecommunications America

- Introduction

This contribution proposes a relay MAC PDU format and data forwarding mechanism in 16m relay system.

A MAC PDU format for relay itself follows the same format as defined in P802.16m/D1 [1] as below.

When a MAC PDU for an AMS is transmitted via a relay station, the MAC PDU is encapsulated in a relay MAC PDU. The relay MAC PDU consists of a header and a payload, where the header carries a station identifier of an AMS to identify an originator/destination of the MAC PDU and the payload carries a MAC PDU of the AMS. The MAC PDU of the AMS in the payload of a relay MAC PDU is formed with mechanisms in 15.2.2 MAC PDU formats of P802.16m/D1 [1]. One relay MAC PDU carries fragmented or packed or multiplexed MAC PDU of the AMS. The relay station can decode and encode GMH or Extended Headers of each MAC PDU of the AMS.

The Figure 1 and the Table 1 present a format of Relay MAC PDU the fields of Relay MAC Header, respectively.

Figure 1 – Relay MAC PDU formats

Table 1– RMH Fields

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Relay MAC Header() { |  |  |
| Flow ID | 3 | This field indicates the ARS’s flow ID that addresses a connection for relayed packets. |
| Extended Header Flag | 1 | Indicates whether any extended header for ABS-ARS link is present. Note: the only extended header identified right now is the piggybacked BR. |
| AMS STID | 12 | AMS’s station identifier whose packet is included the following payload. |
| } |  |  |

If multiple MAC PDUs for one AMS are transmitted over a relay station but multiplexing mechanism cannot be applied to the MAC PDUs, then a method to identify there are multiple MAC PDUs is needed. We propose to reserve one FID of relay station to identify this.

MAC PDUs for multiple AMSs under a relay station can be concatenated into a single PHY burst to be processed by the relay station.

Reference

[1] P802.16m/D1, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, July 2009

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Add the following texts:]

15.5.aa MAC PDU formats for relay

MAC PDUs on a relay link sent to/from subordinate AMS shall be of the form illustrated in Figure xxx. This format will be referred to as the Relay MAC PDU format. Each relay MAC PDU shall begin with a fixed-length relay MAC header (see 15.5.aa.1). The relay MAC header shall be followed by the payload. The payload shall consist of one or more MAC PDUs as defined in Figure 385.

MAC PDUs for the ARS itself shall follow the same format as shown in Figure 385.

Figure xxx – Relay MAC PDU formats

15.5.aa.1 Relay MAC Header (RMH)

The RMH format is defined in Table yyy.

Table yyy – RMH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Relay MAC Header() { |  |  |
| Flow ID | 3 | This field indicates the ARS’s flow ID that addresses a connection for relayed packets. |
| Extended Header Flag | 1 | Indicates whether any extended header for ABS-ARS link is present. Note: the only extended header identified right now is the piggybacked BR. |
| AMS STID | 12 | AMS’s station identifier whose packet is included the following payload. |
| } |  |  |

[Add the following texts:]

15.5.bb Relay construction and transmission MAC PDUs

All MAC PDUs to/from the subordinate AMSs shall be transmitted on the connections (identified uniquely by the FIDs) established between ABS and ARS. Each MAC PDU transmitted on the relay link, over any of these connections, shall be encapsulated in a relay MAC PDU with the relay MAC header carrying the FID of that connection. Refer to 15.5.aa and 15.5.aa.1 for the definition of the relay MAC PDU and the relay MAC header, respectively.

Multiple relay MAC PDUs belonging to different AMSs, attached to the same ARS, can be concatenated into a single PHY burst for the ARS on the ABS-ARS link.

When a relay MAC PDU includes more than one non-multiplexed MAC PDUs for an AMS, the FID field in the RMH shall be set to a predefined FID to identify that multiple non-multiplexed MAC PDUs are being transmitted.

The MAC header and the MAC PDU format of all the MAC PDUs destined for the ARS itself shall have the form illustrated in Figure 385 and Figure 652, respectively. The FID field in the MAC header of the MAC PDUs destined for the ARS will be the ARS’s FID (i.e. basic or primary management connection for the ARS).

-------------------------------  Text End  ---------------------------------------------------