| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on MS Network Entry Support (15.5 Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Rakesh Taori, Youngbin Chang, Jungje Son Samsung Electronics Co., Ltd. Anil Agiwal SISO Kaushik Josiam Samsung Telecommunications America | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 |  |
| Abstract | The contribution proposes the text to support AMS’s network entry in a network with relay stations. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D2 Amendment text |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on MS Network Entry Support

Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Rakesh Taori, Youngbin Chang, Jungje Son

Samsung Electronics Co., Ltd.

Anil Agiwal

SISO

Kaushik Josiam

Samsung Telecommunications America

- Introduction

This contribution proposes an ARS operation to support AMS’s network entry procedure where the AMS enters a network through the ARS.

The ARS performs adjustments directly with the AMS when it receives initial ranging sequence from an AMS with no interaction with its ABS. The handling of contention-based initial ranging for AMS is defined in [2]. After the contention-based initial ranging, the ARS shall forward network entry related MAC management messages between the AMS and its ABS. The ARS transmits its own A-PREAMBLE and its system configuration information such as SFH and AAI_SCD for the AMS.

Reference

[1] P802.16m/D1, Draft amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, July 2009

[2] C80216m-09_Relay initial ranging, Proposal on Initial Ranging of Relay Station

- Suggested remedy

-------------------------------  Text Start  ---------------------------------------------------

[Add the following texts:]

15.5.aa ARS behavior during AMS network entry

In DL channel scanning phase, an AMS may select an ARS as its access station. The AMS shall establish synchronization with the ARS and obtain DL/UL parameters by reading SFHs and AAI_SCD message from the ARS.

After ranging procedure described in subclause 15.3.10.3.1, the ARS shall relay the management messages for the remaining network entry procedures i.e., AAI_SBC-REQ, AAI_SBC-RSP, AAI_PKM-REQ, AAI_PKM-RSP, AAI_REG-REQ and AAI_REG-RSP messages between the AMS and the ABS.

-------------------------------  Text End  ---------------------------------------------------