| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for MIMO-midamble support on relay links (Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Alexander Maltsev, Jerry Sydir, Alexey Khoryaev, Alexei Davydov, Andrey Chervyakov, Mikhail Shilov Intel Corporation | e-mail: alexander.maltsev@intel.com |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 P802.16m/D1 topic: Relay |  |
| Abstract | This contribution mainly proposes support of MIMO midamble transmissions on relay links |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for MIMO Midamble Support on Relay Links

Alexander Maltsev, Jerry Sydir, Alexey Khoryaev,
Alexei Davydov, Andrey Chervyakov, Mikhail Shilov

Intel Corporation

- Introduction

This contribution proposes amendment text describing transmission of MIMO midamble to support MIMO channel estimation on relay links (ABS-ARS) and to be included in the 802.16m amendment draft [5]. The proposed text is developed so that it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4]. Current SDD [3] is used as a basis for the text development.

In order to provide efficient relay link utilization and achieve high transmission rates, the closed-loop SU- and MU-MIMO techniques have to be supported on relay link. For the purpose of MIMO channel estimation we propose to introduce additional MIMO midamble signaling on the relay link. The proposed periodicity of MIMO midamble signaling is equal to 20 ms (i.e. once per superframe) since the relay links are almost static and do not suffer from Doppler fading effect significantly.

- References

- IEEE P802.16Rev2/D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” March 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D1

============================= Start of Proposed Text ==============================

- Advanced Air Interface

  - Introduction

  - Medium access control

  - Physical layer (PHY)

    - Downlink physical structure

15.3.5.4.2

[Insert the following after the first paragraph of current section (15.3.5.4.2)]

ARS transmits MIMO midamble in the 16m DL Access zone. This transmission is time aligned with ABS MIMO midamble transmission.

[Insert the following subclause 15.3.5.6]

      - Relay DL physical structure

15.3.5.6.x MIMO midamble support on relay link

ABS shall transmit additional MIMO midamble in the 16m DL Transmit zone of the first frame of each superframe. MIMO midamble transmission shall occupy the last OFDMA symbol of the last subframe of the 16m DL Transmit zone. If the last subframe in the 16m DL Relay zone is the type-3 subframe, then the MIMO midamble shall be transmitted in the last OFDMA symbol of the nearest preceding 16m DL Relay zone type-1 or type-2 subframe. The MIMO midamble shall not be transmitted in the 16m DL Relay zone if there are no type-1 or type-2 subframes in the 16m DL Relay zone.

If MIMO midamble is present in the 16m DL Transmit zone, then the subframe where the MIMO midamble is located shall be transformed to the subframe type where the number of symbols is reduced by one. For the type-1 subframe case, the remaining 5 consecutive symbols form a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols form a type-1 subframe.

The MIMO midamble signal format is defined in Section 15.3.5.4.2.

============================= End of Proposed Text ==============================