| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for multicell MIMO trigger mechanism (15.3.14.3) |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Shuangfeng Han, David Mazzarese, Sangwoo Lee, Cheng shan, Hokyu choi, Heewon Kang Samsung Electronics Co., Ltd. | shuangf.han@samsung.com d.mazzarese@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes trigger mechanism for multicell MIMO to be included in the 802.16m D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for multicell MIMO trigger mechanism (15.3.14.3)

Shuangfeng Han, David Mazzarese, Sangwoo Lee, Cheng shan, Hokyu choi, Heewon Kang

Samsung Electronics

- Introduction

In current IEEE 802.16m D1 [1], two types of multicell MIMO schemes are specified.

- Single BS precoding techniques with Multi-BS coordination, which includes PMI coordination, supported by codebook-based feedback, and interference nulling, supported by codebook-based feedback or by uplink sounding. Single BS precoding with Multi-BS Coordination may be enabled by the ABS for one or several AMSs when CL MIMO precoding is applied in the serving and neighboring cells. The inter-cell interference can be mitigated by coordinating the precoders applied in neighboring cells via higher layer signaling, based on feedback from AMSs to their respective serving ABSs. With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction, as instructed by the ABS.

- Joint MIMO transmission across multiple ABSs. The ABS and AMS may optionally support both adaptive precoding based multi-BS joint processing e.g. Closed-loop Marco Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission and non-adaptive precoding based multi-BS joint processing. Multi-BS joint MIMO processing may be enabled by the ABS for one or several AMSs when adaptive or non-adaptive precoding is applied in the serving and neighboring cells and user data is shared among multiple cells. With adaptive precoding, the precoder matrix Wk is derived from the feedback of the MS, e.g. codebook-based feedback and sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For the above 2 types of Multicell MIMO, it is necessary to design trigger mechanism. Otherwise, many cell edge MSs feed back their measurement information to serving BS, like RSSI, SINR, PMI for serving and interfering BS, normalized interference power of each interfering BS, or interference power of each interfering BS. It would be extremely tough for each BS to schedule based on so much different feedback information. In order to make Multicell MIMO more efficient and less complicated, we propose in this document a Multicell MIMO trigger mechanism, which can be utilized in IEEE 802.16m.

- References

[1] IEEE 802.16m/D1, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD

------------------------------------------------------  Text Start  ---------------------------------------------------------------

Inserting in section 15.3.14.3 with the following text.

15.3.14.3.4 Multicell MIMO trigger mechanism

ABS broadcasts or unicasts normalized interference power (NIP) thresholds for 2 types of multicell MIMO schemes. The NIP is defined as ratio of average interference power (with or without transmitter precoder) from one dominant interference BS to the total interference power received by AMS. AMS will accordingly request for proper multicell MIMO scheme. The operation procedure of multicell MIMO trigger follows the steps below, as shown in Fig. 1.

- AMS in Normal process of Single-Cell CL MIMO feeds back information for CL MIMO operation, including CQI, PMI, rank etc.

- ABS requests NIP feedback from several AMSs in the network through feedback_Polling_IE. This request is based on AMS’ CQI and RSSI feedback.

- Upon receiving feedback_polling_IE, the corresponding MSs feed back their NIPs to the serving BS through MAC management message.

- ABS selects a NIP threshold NIP_th_1 for type 1 Multicell MIMO scheme and NIP_th_2 for type 2 Multicell MIMO scheme based on network measures and broadcasts the NIP threshold NIP_th_1 and NIP_th_2 in a DL broadcast message (Additional Broadcast Information). Alternatively, ABS sends the NIP threshold NIP_th_1 and NIP_th_2 in a DL unicast message for 1 AMS.

- AMS computes its NIP and check the trigger conditions, if NIP >= NIP_th_1 and NIP <NIP_th_2, AMS will request for type 1 Multicell MIMO.

  - AMS feedback its event-driven request to operate with multicell MIMO type 1 to its serving BS through MAC management message

  - Once an AMS receives a Feedback_Polling_IE, the procedure outlined in 15.3.14.3.2.1 will be followed

- If NIP >=NIP_th_2, AMS will request for type 2 Multicell MIMO.

  - AMS feedback its event-driven request to operate with multicell MIMO type 2 to its serving BS through MAC management message

  - Once an AMS receives a Feedback_Polling_IE, the procedure outlined in 15.3.14.3.3.1 will be followed.

- If NIP < NIP_th_1, AMS continues single cell CL MIMO mode.

Fig. 1 Flow chart for multicell MIMO trigger mechanism

-------------------------------  Text End  -----------------------------------------------------------------