| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for multicell MIMO trigger mechanism (15.3.14.3) |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Shuangfeng Han, David Mazzarese, Sangwoo Lee, Cheng shan, Hokyu choi, Heewon Kang Samsung Electronics Co., Ltd. Luciano Sarperi Fujitsu Wookbong Lee LG Electronics | shuangf.han@samsung.com d.mazzarese@samsung.com luciano.sarperi@uk.fujitsu.com wbong@lge.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes trigger mechanism for multicell MIMO to be included in the 802.16m D1. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for multicell MIMO trigger mechanism (15.3.14.3)

Shuangfeng Han, David Mazzarese, Sangwoo Lee, Cheng shan, Hokyu choi, Heewon Kang

Samsung Electronics

Luciano Sarperi

Fujitsu

Wookbong Lee

LG Electronics

- Introduction

In current IEEE 802.16m D1 [1], two types of multicell MIMO schemes are specified.

- Single BS precoding techniques with Multi-BS coordination, which includes PMI coordination, supported by codebook-based feedback, and interference nulling, supported by codebook-based feedback or by uplink sounding. Single BS precoding with Multi-BS Coordination may be enabled by the ABS for one or several AMSs when CL MIMO precoding is applied in the serving and neighboring cells. The inter-cell interference can be mitigated by coordinating the precoders applied in neighboring cells via higher layer signaling, based on feedback from AMSs to their respective serving ABSs. With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction, as instructed by the ABS.

- Joint MIMO transmission across multiple ABSs. The ABS and AMS may optionally support both adaptive precoding based multi-BS joint processing e.g. Closed-loop Marco Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission and non-adaptive precoding based multi-BS joint processing. Multi-BS joint MIMO processing may be enabled by the ABS for one or several AMSs when adaptive or non-adaptive precoding is applied in the serving and neighboring cells and user data is shared among multiple cells. With adaptive precoding, the precoder matrix Wk is derived from the feedback of the MS, e.g. codebook-based feedback and sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For the above 2 types of Multicell MIMO, it is necessary to design trigger mechanism. Otherwise, many cell edge MSs feed back their measurement information to serving BS, like RSSI, SINR, PMI for serving and interfering BS, normalized interference power of each interfering BS, or interference power of each interfering BS. It would be extremely tough for each BS to schedule based on so much different feedback information. In order to make Multicell MIMO more efficient and less complicated, we propose in this document a Multicell MIMO trigger mechanism, which can be utilized in IEEE 802.16m.

- References

[1] IEEE 802.16m/D1, “802.16m Amendment Working Document (AWD)”

- Text proposal for inclusion in the 802.16m AWD

## Proposed Text 1 ##

<< Create Section 15.3.14.3.4 in section 15.3.14 >>

------------------------------------------------------  Text 1 Start  ---------------------------------------------------------------

15.3.14.3.4 Multi-BS MIMO trigger mechanism

ABS broadcasts normalized interference power (NIP) thresholds for 2 types of multi-BS MIMO schemes. The NIP is defined as ratio of average interference power (with or without transmitter precoder) from one dominant interference BS to the total interference power received by AMS. The AMS may accordingly request the preferred multi-BS MIMO scheme. The operation procedure of the multi-BS MIMO trigger follows the steps below.

- AMS in normal process of Single-Cell CL MIMO feeds back information for CL MIMO operation.

- ABS selects a NIP threshold NIP_th_1 for single-BS precoding with multi-BS coordination, and a threshold NIP_th_2 for multi-BS joint MIMO processing based on network measurements. Then the ABS broadcasts the NIP thresholds NIP_th_1 and NIP_th_2 in an Additional Broadcast Information (ABI).

- AMS computes its average NIP and checks the trigger conditions. If NIP ≥ NIP_th_1 and NIP <NIP_th_2, AMS may request operation with single-BS precoding with multi-BS coordination using the AAI_MULTI_BS_MIMO-REQ message.

  - AMS feedback its event-driven request to operate single-BS precoding with multi-BS coordination to its serving BS through MAC management message which includes a 2bit NIP difference measurement with respect to NIP_th_1

  - Once an AMS receives a Feedback_Polling_IE, the procedure outlined in 15.3.14.3.2.1 will be followed

- If NIP ≥ NIP_th_2, AMS may request operation with multi-BS joint MIMO processing using the AAI_MULTI_BS_MIMO-REQ message.

  - AMS feedback its event-driven request to operate with multi-BS joint MIMO processing to its serving BS through MAC management message which includes a 2bit NIP difference measurement with respect to NIP_th_2

  - Once an AMS receives a Feedback_Polling_IE, the procedure outlined in 15.3.14.3.3.1 will be followed.

-------------------------------  Text 1 End  -----------------------------------------------------------------

## Proposed Text 2 ##

<< Add two rows in Table 51 and modify Table 52, section 15.3.14.3.1 >>

------------------------------------------------------  Text 2 Start  ---------------------------------------------------------------

Table 51—Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| Parameter | Description | Value | Contol channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| NIP_th_1 | NIP threshold for Single BS precoding with Multi-BS Coordination trigger | [2-4 bits] | Additional broadcast information (ABI) |  |
| NIP_th_2 | NIP threshold for DL Multi-BS Joint MIMO Processing trigger | [2-4] bits | Additional broadcast information (ABI) |  |

Table 52 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for serving and neighboring cell | For PMI coordination among multiple ABSs |
|  | PMI_coordination_subset indication | Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell |
|  | Additional measurement metric | Optional additional feedback for indicating the potential interference level improvement associated with the PMIs reported for the neighboring cells. SINR gain assuming the reported PMI set is coordinated. This can be used for resolving conflict from multiple AMS. |
|  | Normalized interference power (NIP) | Ratio of average interference power (with or without transmitter precoder) from ABS with BS-ID, to the total interference power plus noise received by AMS, which serves as an indicator of severity of interference. |
|  | CPMI for neighboring cell | Concatenating PMI for neighboring cell |
| Event-driven feedback | Requesting Multi-BS MIMO | For AMS reporting of its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing using a MAC management the AAI_MULTI_BS_MIMO-REQ control message |

-------------------------------  Text 2 End  -----------------------------------------------------------------

## Proposed Text 3 ##

<< Add one row at the end of Table 675 on page 29 line 53 in section 15.2.4 >>

------------------------------  Text 3 Start  ---------------------------------------------------

15.2.4 MAC Control messages

Table 675—MAC Control Messages

| Message name | Message description | Connection |
| --- | --- | --- |
| … | … | … |
| AAI_MULTI_BS_MIMO-REQ | Multi-BS MIMO Request | Basic |

-------------------------------  Text 3 End  ---------------------------------------------------

## Proposed Text 4 ##

<< Create section “15.2.6.X AAI_MULTI_BS_MIMO-REQ” under section “15.2.6 AAI MAC Management Messages” on page 86 line 48 >>

------------------------------  Text 4 Start  ---------------------------------------------------

15.2.6 AAI MAC Management Messages

…

15.2.6.X AAI_MULTI_BS_MIMO-REQ

The AAI_MULTI_BS_MIMO-REQ message shall be transmitted by the AMS to report its preference on single BS precoding with Multi-BS MIMO coordination or multi-BS joint MIMO processing.

---ASN.1START

AAIMULTIBSMIMOREQ Message::SEQUENCE{

Multi-BS MIMO Request	Boolean,

NIP Value		ENUMERATED {n0, n1, n2, n3}

}

--ASN1STOP

Table YY – AAI_MULTI_BS_MIMO-REQ message format

| AAIMULTIBSMIMOREQ message field description |  |
| --- | --- |
| Multi-BS MIMO Request | 0: Single-BS precoding with Multi-BS Coordination 1: Multi-BS Joint MIMO Processing |
| NIP Value | Absolute NIP value encoded as difference to NIP_th_1 when Multi-BS MIMO Request = 0 Absolute NIP value encoded as difference to NIP_th_2 when Multi-BS MIMO Request = 1 The values are interpreted in the range [TBD] |

The AMS shall perform the averaging of NIP measurements according to equation (158) in section 8.4.11.3.

-------------------------------  Text 4 End  ---------------------------------------------------