| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to Mapping of miniband tables at 802.16m/D1 (Section 15.3.5.3.1 & 15.3.8.3.1)--CRU/DRU allocation |  |
| Date Submitted | 2009-8-31 |  |
| Source(s) | Jong-Kae (JK) Fwu, Hujun Yin Intel Corporation | jong-kae.fwu@intel.com |
| Re: | IEEE 802.16m/D1 Section 15.3.5.3.1 & 15.3.8.3.1 |  |
| Abstract | The contribution provides Changes to Mapping of miniband tables |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D1 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to Mapping of miniband tables at 802.16m/D1 Section 15.3.5.3.1 & 15.3.8.3.1--CRU/DRU allocation

Jong-Kae (JK) Fwu, Hujun Yin

Intel Corporation

- Introduction

The mapping between DCASMB,0 and the number of miniband-based CRUs for FP0 are shown in the Table 655-657 for DL and 726 -728 for UL for system bandwidths of 20 MHz, 10 MHz, and 5 MHz respectively. These values are too restrictive and should be modified with the following proposed modifications.

---------------------------------------------------------------------------------------

Modify Table 655, 656 and 657 in Section 15.3.5.3.1 and Table 726,727 and 728 in Section 15.3.8.3.1.

--------------------------------------------- Proposed Text for Starts -----------------------------------------------

Table 649—Mapping between DCASMB,0 and number of miniband-based CRUs for FP0 for 20 MHz

| DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 28 |
| 1 | 1 2 | 17 | 17 32 |
| 2 | 2 4 | 18 | 18 36 |
| 3 | 3 6 | 19 | 19 40 |
| 4 | 4 8 | 20 | 20 44 |
| 5 | 5 10 | 21 | 21 48 |
| 6 | 6 12 | 22 | 22 52 |
| 7 | 7 14 | 23 | 23 56 |
| 8 | 8 16 | 24 | 24 60 |
| 9 | 9 18 | 25 | 28 64 |
| 10 | 10 19 | 26 | 32 68 |
| 11 | 11 20 | 27 | 36 72 |
| 12 | 12 21 | 28 | 40 76 |
| 13 | 13 22 | 29 | 44 80 |
| 14 | 14 23 | 30 | 48 84 |
| 15 | 15 24 | 31 | 52 88 |

Table 650— Mapping between DCASMB,0 and number of miniband-based CRUs for FP0 for 10 MHz

| DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 16 |
| 1 | 1 2 | 9 | 9 18 |
| 2 | 2 4 | 10 | 10 20 |
| 3 | 3 6 | 11 | 11 22 |
| 4 | 4 8 | 12 | 12 24 |
| 5 | 5 10 | 13 | 16 38 |
| 6 | 6 12 | 14 | 20 40 |
| 7 | 7 14 | 15 | 24 42 |

Table 651— Mapping between DCASMB,0 and number of miniband-based CRUs for FP0 for 5 MHz

| DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | DCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 4 8 |
| 1 | 1 2 | 5 | 6 10 |
| 2 | 2 4 | 6 | 8 18 |
| 3 | 3 6 | 7 | 10 20 |

Table 726—Mapping between UCASMB,0 and number of miniband-based CRUs for FP0 for 20 MHz

| UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 16 | 16 28 |
| 1 | 1 2 | 17 | 17 32 |
| 2 | 2 4 | 18 | 18 36 |
| 3 | 3 6 | 19 | 19 40 |
| 4 | 4 8 | 20 | 20 44 |
| 5 | 5 10 | 21 | 21 48 |
| 6 | 6 12 | 22 | 22 52 |
| 7 | 7 14 | 23 | 23 56 |
| 8 | 8 16 | 24 | 24 60 |
| 9 | 9 18 | 25 | 28 64 |
| 10 | 10 19 | 26 | 32 68 |
| 11 | 11 20 | 27 | 36 72 |
| 12 | 12 21 | 28 | 40 76 |
| 13 | 13 22 | 29 | 44 80 |
| 14 | 14 23 | 30 | 48 84 |
| 15 | 15 24 | 31 | 52 88 |

Table 727— Mapping between UCASMB,0 and number of miniband-based CRUs for FP0 for 10 MHz

| UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 8 | 8 16 |
| 1 | 1 2 | 9 | 9 18 |
| 2 | 2 4 | 10 | 10 20 |
| 3 | 3 6 | 11 | 11 22 |
| 4 | 4 8 | 12 | 12 24 |
| 5 | 5 10 | 13 | 16 38 |
| 6 | 6 12 | 14 | 20 40 |
| 7 | 7 14 | 15 | 24 42 |

Table 728— Mapping between UCASMB,0 and number of miniband-based CRUs for FP0 for 5 MHz

| UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) | UCASMB | # of miniband based CRUs for FP0 (LMB, FP0 ) |
| --- | --- | --- | --- |
| 0 | 0 | 4 | 4 8 |
| 1 | 1 2 | 5 | 6 10 |
| 2 | 2 4 | 6 | 8 18 |
| 3 | 3 6 | 7 | 10 20 |

------------------------------------------------- Proposed Text Ends -------------------------------------------------