| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD text for SON |  |
| Date Submitted | 2009-08-29 |  |
| Source(s) | Muthaiah Venkatachalam Intel Corporation | muthaiah.venkatachalam@intel.com |
| Re: | Proposed text for section AWD 15.6 |  |
| Abstract | This contribution proposes AWD text for SON for session 63.5 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD text for SON

Muthaiah Venkatachalam

Intel Corporation

The basic idea of self optimization is that a SON server in the network can update the RF operational parameters of a WiMAX BS on a dynamic basis. Eg: RF carrier, FFR info, Tx power etc

Some of these parameter updates will need the restart of the BS operation

If an MS is being serviced in the BS when this restart has to happen, then the MS session can be lost. This contribution proposes a mechanism to not lose the session continuity of MS during such restarts due to SON updates.

========================== Start of Proposed Text =========================

15.6.x SON reconfigurations and restarts

When the BS is being asked to restart its operation by the SON server, the BS shall send a Deregistration command (DREG-CMD) message to all the MSs that are attached to it. A new DREG-CMD code shall be defined and used for this specific purpose of SON restart of the BS. When this code is used in the DREG-CMD and the MS receives the same, the MS will immediately know that this de-registration is for SON purposes and hence it is a temporary one.

As part of this DREG-CMD, following information is included:

a) Unavailable time Interval (UTI) of the BS

b) New SFH parameters of BS (post reconfig) for network re-entry of the MS

In order to maintain the session continuity of the MS, BS also indicates this restart to the ASN-GW to start buffering the data in the GW for the MS

BS and MS maintain the MAC context after DREG-CMD to enable fast network re-entry

MS gets this DREG-CMD message and deregisters itself from the BS. It waits till UTI expires. Then it scans for the BS again with the new SFH parameters. MS performs fast re-entry with existing MAC context. MS Session is not lost

Before the SON reconfig, the BS may also broadcast the following info additionally:

a) Unavailability start time (UST)

b) Unavailable Time Interval (UTI) of the BS

c) New SFH parameters for network re-entry of the MS, once the BS becomes available.

========================== End of Proposed Text =========================