| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD text for SON |  |
| Date Submitted | 2009-09-14 |  |
| Source(s) | Muthaiah Venkatachalam Intel Corporation | muthaiah.venkatachalam@intel.com |
| Re: | Proposed text for section AWD 15.6 |  |
| ABStract | This contribution proposes AWD text for SON for session 63.5 |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD text for SON

Muthaiah Venkatachalam

Intel Corporation

The basic idea of self optimization is that a SON server in the network can update the RF operational parameters of a WiMAX ABS on a dynamic basis. Eg: RF carrier, FFR info, Tx power etc

Some of these parameter updates will need the restart of the ABS operation

If an AMS is being serviced in the ABS when this restart has to happen, then the AMS session can be lost. This contribution proposes a mechanism to not lose the session continuity of AMS during such restarts due to SON updates.

========================== Start of Proposed Text =========================

15.6.x Handling reconfigurations and restarts due to SON operation

SON can cause the ABS to reconfigure and/or restart depending upon the type of configuration change. The AMSes that are attached to the ABS (and those that are planning on attaching) will need a way to be updated of the ABS reconfiguration and/or restart.

15.6.x.1 Handling ABS reconfigurations due to SON

The ABS may start broadcasting the following information prior to the SON reconfiguration using the AAI_SON-ADV message:

a) Reconfiguration time (RCT) of the ABS – the time instance when the ABS will reconfigure and switch to new system parameters

b) New SFH parameters post RCT.

15.6.x.2 Handling ABS restarts due to SON

When the ABS is going to be restarted due to SON, the ABS shall send a Deregistration command (DREG-CMD) message to all the AMSs that are attached to it with code=0x07. When this code is used in the DREG-CMD and the AMS receives the same, the AMS will immediately know that this de-registration is for SON purposes and hence it is a temporary one.

As part of this DREG-CMD, following information is included:

a) Unavailable time Interval (UTI) of the ABS

b) New SFH parameters of ABS (post reconfig) for network re-entry of the AMS

ABS and AMS maintain the MAC context after DREG-CMD to enable fast network re-entry

AMS upon receiving this DREG-CMD message, deregisters itself from the ABS. It waits till UTI expires. Then it scans for the ABS again with the new SFH parameters. AMS performs fast re-entry with existing MAC context.

In addition, before the SON restart, the ABS may also broadcast the following info in the AAI_SON-ADV message:

a) Unavailability start time (UST)

b) Unavailable Time Interval (UTI) of the ABS

c) New SFH parameters post SON restart.

========================== End of Proposed Text =========================