| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Coding Based Retransmission Scheme in E-MBS for IEEE 802.16m AWD |  |
| Date Submitted | 2009-09-18 |  |
| Source(s) | Guosen Yue, Xiaodong Wang, Sampath Rangarajan, Mohammad A. (Amir) Khojastepour NEC Laboratories America, Inc. Hung-Yu Wei, Ching-Chun Chou, Cheng-Chih Chao National Taiwan University Chun-Yen Wang, Frank Ren ITRI | E-mail: yueg@nec-labs.com |
| Re: | IEEE 802.16m-09/0037, Call for contribution on Project 802.16m Amendment Working Document (AWD) content (2009-08-30) P802.16m/D1: E-MBS |  |
| Abstract | This contribution proposes a network coding based retransmission scheme for EMBS. |  |
| Purpose | To be discussed and approved by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Coding Based Retransmission Scheme in E-MBS for IEEE 802.16m AWD

G. Yue, X. Wang, S. Rangarajan, M. A. Khojastepour

NEC-Labs America

Hung-Yu Wei, Ching-Chun Chou, Cheng-Chih Chao

National Taiwan University

Chun-Yen Wang, Frank Ren

ITRI

# 1. Introduction

In IEEE 802.16m SDD[1], for E-MBS operation with retransmission, an ABS may use a network-coding based retransmission scheme that does not require a feedback channel. In [2], the basic network-coding scheme and its two variations are introduced to obtain the redundant packets for retransmissions in E-MBS without MS feedback. However, the proposed schemes in [2] although simple do not provide a superior packet recovery performance.

In this harmonized document, we propose the network coding based retransmission scheme for E-MBS with feedback from MS’s. This harmonized document includes the text from contributions C80216m-09_1682 [3] , C80216m-09_1684 [4], and C80216m-09_2111 [5].

# 2 Simulation Results

We propose the harmonized scheme in this document. We compare the harmonized network coding scheme in C80216m-09_1682r1 without feedback and the network coding scheme with the feedback proposed in this document.

The simulation settings are as follows:

- One ABS broadcast to 10 AMSs.

- In each trial, the ABS transmits L original packets. L = 75, 100, and 125.

- The packet loss rate is 0.01, 0.02, 0.03, 0.04, 0.05, 0.07

We use the metric All perfect ratio for simulation.

Fig. 1 shows the result of L= 100. We can see that the harmonized scheme for the feedback case provide a certain performance gain for the number of retransmissions Q=14, 18 over the scheme without feedback. Similar results can be found in Fig. 2 for L=125.

Fig. 1. L=100

Fig. 2. L=125

# 3. Proposed Text

# Proposed Text to P802.16m Amendment Working Document

Add the following text into the amendment draft:

================================= Start of Proposed Text=============================

15.8 Support for Enhanced Multicast Broadcast Service

15.8.2.3.X.1 Network Coding based E-MBS Retransmission

E-MBS may support network coding retransmission with or without feedback.

15.8.2.3.X.1.2 Network Coding based E-MBS retransmission with Limited Unicast Feedback

An ABS first transmits L packets, P1,…, PL. After the L packets are transmitted, a feedback of number of lost packet W is sent by every AMS in the service to ABS. The ABS sends network coding based retransmission packets based on the received feedback. The pre-defined codebook is used to determine the coding coefficient for network coding transmission. In the Network Coding Extended Header, Mode should be set to 10, PKT_N should be set to L, and Feedback Information field may be filled based on the feedback configuration.

15.8.2.3.X.1.3 Network Coding based E-MBS retransmission with Common Feedback

AMS may signal the ABS if the number of error packet reaches the feedback threshold specified in the E-MBS packet header. The feedback signal from AMS should be sent through common feedback channel. The ABS sends network coding based retransmission packets based on the received feedback. The pre-defined codebook is used to determine the coding coefficient for network coding transmission. In the Network Coding Extended Header, Mode should be set to 11, and Feedback Information field may be filled based on the feedback configuration.

# 15.8.2.3.X.2.2 Codebook for Network Coding Based E-MBS Retransmission with Feedback.

An ABS first transmits L packets, P1,…, PL. After the L packets are transmitted, a feedback of number of lost packet W is sent by every AMS in the service to ABS. The average packet loss rate is computed by

p=kWkKL,

where K denotes the total number of AMS’s with the feedback. Given p and M, we obtain size of 0, i.e., l0 as follows.

First we obtain x* by

x=expalnp+b,		where a,b are parameters given in Table x.

Then, we obtain

N*=min{round(x*), L}

Set  M=5 if L>50, or M=4 if L<=50.

Determine ln n=0,1,…,2M-1, by

1) l0=L-N*.

2a) For n=1, …2M-1, first let ln =Z, where Z=,

2b) For the rest n*-(2M-1)Z packets, add ln by 1 based on the ascending order of the Hamming weight of the binary expansion of n until n=12M−1ln=N∗.

| M | a | b |
| --- | --- | --- |
| 4 | -1.0039 | 0.9348 |
| 5 | -0.9704 | 1.2243 |

Table x Parameters for obtaining x*.

With above process, the code structure defined by {ln} is obtained to generate the first M retransmission packets R1, …, RM. The encoding method is the same as described in Section 15.3.8.X.2.1 (See C80216m-09_1682r1). The retransmsion packets RM+1, RM+2, …, RL are generated in the same way as described in 15.3.8.X.2.1 (See C80216m-09_1682r1).

================================= End of Proposed Text=============================

# 5. References

[1] IEEE 802.16m-08/0034, “The Draft IEEE 802.16m System Description Document”.

[2] IEEE C802.16m-09/0072r1, “Network coding-based retransmission schemes for E-MBS with and without MS feedback”.

[3] IEEE C802.16m-09/1682, Proposed Text for IEEE P802.16m/D1: Network Coding based E-MBS Retransmission (E-MBS) (Ching-Chun Chou, Hung-Yu Wei, Cheng-Chih Chao, Chun-Yen Wang, Frank Ren; 2009-08-30)

[4] IEEE C802.16m-09/1684,Proposed Text for IEEE P802.16m/D1: Network Coding Codebook Design for E-MBS (E-MBS) (Cheng-Chih Chao, Hung-Yu Wei, Ching-Chun Chou, Chun-Yen Wang, Frank Ren; 2009-08-30)

[5] IEEE C802.16m-09/2111, Network Coding Based Retransmission Scheme in E-MBS for IEEE 802.16m AWD (G. Yue, X. Wang, S. Rangarajan, A. Khojasterpour; 2009-08-30)