| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | DL and UL control channel usage at ARS (Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Hyunjeong Kang Rakesh Taori, Anil Agiwal Samsung Electronics | E-mail: yb.chang@samsung.com Phone: +82-31-279-5519 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 Topic: Relay |  |
| Abstract | This contribution mainly proposes the DL and UL control channel usage at ARS. |  |
| Purpose | To be discussed and adopted in 802.16m/D1 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

DL and UL control channel usage at ARS

Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Hyunjeong Kang Rakesh Taori, Anil Agiwal

Samsung Electronics

# Introduction

This contribution proposed the usage of control channel in downlink and uplink. Most part of control channels are reused in both relay link and access link. In the relay link, ABS does not need to send an Advanced preamble and Super-frame header. An RS_Config_CMD message is used in the relay link when Super-frame header information is changed.

# Text proposal

============================= Start of Proposed Text ========================

[Modify the text from 29-40 on page 111 as follows]

15.3.10.xx DL control channels

In the access link between ARS and AMS, all DL control channels in section 15.3.6 are reused. In the relay ink between ABS and ARS, only A-MAP in 15.3.6 is reused among DL control channels. The advanced preamble is not used and the RS_Config-CMD is used instead of Superframe header.

15.3.10.xx UL control channels

In the both access link and relay link, all UL control channels in section 15.2.9 are reused.

============================== End of Proposed Text ========================