| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ operation at ARS (Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Hyunjeong Kang Rakesh Taori, Anil Agiwal Samsung Electronics | E-mail: yb.chang@samsung.com Phone: +82-31-279-5519 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 Topic: Relay |  |
| Abstract | This contribution mainly proposes the method of HARQ operation at ARS. |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ operation at ARS

Youngbin Chang, Hyunjeong Kang, Hyunjeong Kang Rakesh Taori, Anil Agiwal

Samsung Electronics

# Introduction

In the non-transparent RS and distributed scheduling mode, HARQ operates hop-by-hop mode. Therefore, whole HARQ protocol in 16m can be applied to relay link and access link independently.

# Text proposal

============================= Start of Proposed Text ========================

[Modify the text from 29-40 on page 111 as follows]

15.3.10.xx Generic HARQ operation at ARS

ARS shall perform HARQ operation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent station (ABS and ARS or ARS and AMS) follows the section 15.3.9.2.

15.3.10.xx Group resource allocation at ARS

ARS shall perform Group resource allocation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent stations (ABS and ARS or ARS and AMS) follows the section 15.3.9.3.

15.3.10.xx Persistent allocation at ARS

ARS shall perform persistent resource allocation with ABS in the relay link and AMS in the access link independently. The HARQ signaling and timing protocol between with adjacent stations (ABS and ARS or ARS and AMS) follows the section 15.3.9.4.

============================== End of Proposed Text ========================