| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | RS operational parameters configuration (Relay) |  |
| Date Submitted | 2009-08-30 |  |
| Source(s) | Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Hyunjeong Kang Rakesh Taori, Anil Agiwal Samsung Electronics | E-mail: yb.chang@samsung.com Phone: +82-31-279-5519 *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for Contributions on Project 802.16m Amendment Content: IEEE802.16m-09/0037 Topic:Relay |  |
| Abstract | This contribution mainly proposes the method of RS operational parameters configuration |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

RS operational parameters configuration

Youngbin Chang, Hyunjeong Kang, Mingxia Xu, Kyungkyu Kim, Hyunjeong Kang Rakesh Taori, Anil Agiwal

Samsung Electronics

# Introduction

This contribution proposes an ARS operational parameters configuration to support ARS in network entry and relay operational mode.

.

# Text proposal

============================= Start of Proposed Text ========================

[Add the text in the appropriate place as follows]

15.3.10.xx.3 Parameter configuration during ARS network entry

After ARS registration, the ABS shall determine the ARS’s operation parameters and send a RS_Config_CMD to configure the operational parameters for relay operation. When ARS receives RS_config-CMD message, it shall transmit AAI_MSG-ACK message. The ARS shall start RS operation and apply the configuration parameters from the time specified by ‘Super-frame number action’ in RS_config-CMD.

15.3.10.xx.4 Parameter configuration update for ARS operational mode

During ARS operation mode, when the system information in SFH or ARS configuration parameters for relay operation is changed, ABS shall send RS_config-CMD message. When ARS receives RS_config-CMD message, it shall transmit AAI_MSG-ACK message. The ARS shall apply the system information in SFH or the configuration parameters from the time specified by ‘Super-frame number action’ in RS_config-CMD.

15.x.x. RS_Config_CMD message

This field may be used by ABS to configure operational parameter for ARS

| Name | Size (bit) | Description |
| --- | --- | --- |
| SFH information | TBD | System information for AMS |
| ARS ID | 48 | ARS identification for AMS |
| ARS mode | TBD | Duplexing division mode (FDD, TDD) Scheduling mode, if needed Security mode, if needed TBD |
| ARS frame configuration | TBD | 16m DL access zone 16m DL transmit zone 16m DL receive zone 16m UL transmit zone 16m UL receive zone |
| Super-frame number action | 6 bit | LSB of super-frame number |
| ARS EIRP | TBD |  |
| ARS paging group | TBD | ARS Paging group ID |
| SA-Preamble | 9 bit | 2bit: Segment Index, 7bit: preamble sequence index |

============================== End of Proposed Text ========================