| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Group Resource Allocation IEs |  |
| Date Submitted | 2009-08-24 |  |
| Source(s) | Xianming Chen, Yangfeng Guan, Wei Xia, Ying Liu, Huiying Fang, Sun Bo ZTE Corporation | E-mail: guan.yanfeng@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D1” 15.3.6.5.4.4 Group resource allocation A-MAP IE and 15.3.6.5.4.5 Group Configuration A-MAP IE |  |
| Abstract | The contribution proposes text on MAC operation and control information for GRA in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Group Resource Allocation IEs

Xianming Chen, Yanfeng Guan, Wei Xia, Ying Liu, Huiying Fang, Bo Sun

ZTE Corporation

# 1 Introductions

This contribution provides modified text for the Group Resource Allocation in the Advanced Air Interface to be included in Section 15.2.8 and 15.3.6.5.4 of the IEEE 802.16m Draft Amendment [1] and accepted contribution [2] [3]. The proposed update is developed so that it can be readily combined with P802.16m/D1 [1], it is compliant to the 802.16m SRD [4] and the 802.16m SDD [5], and it follows the style and format guidelines in [6].

# 2 Problems

In P802.16m-D1, there are some problems on group resource allocation as follows:

- When computing the effective combinations of the HARQ Burst Size and Resource Allocation Size, the resource allocation size is calculated according to the nominal MCS, and then the resource allocation bitmap index is generated based on nominal MCS. The whole process is not compliant with the current rate matching method defined in the channel coding section.

- IEs related to group resource allocation are not still complete.

Based on the problems mentioned above, this contribution proposes the modification on Group Configuration IEs.

# 3 References

- P802.16m-D1. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface. July, 2009.

- IEEE 802.16m-09/1472r1. “Harmonized AWD Text on the MAC Procedure for Group Resource Allocation”. July, 2009.

- IEEE 802.16m-09/1476r4. “Harmonized AWD Text on the Unicast Control Information Elements for Group Resource Allocation”. July, 2009.

- IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document,” Jan. 2009.

- IEEE 802.16m-08/003r9a, “IEEE 802.16m System Description Document,” May 2009.

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Proposal Text

- Modify the DL/UL Group Configuration A-MAP IEs as follows:

----------------------------------------------------- Text Starts -----------------------------------------------------15.3.6.5.4.5 Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used to assign an AMS to a group.

Table 673- DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group identifier |
| Lowest MCS Spectral Efficiency in group | 43 | The index of the lowest MCS Spectral Efficiency that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS Spectral Efficiency in group | 43 | The index of the highest MCS Spectral Efficiency that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Spectral Efficiency Interval | TBD | Indicates spectral efficiency interval supported in the group. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | [4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | [3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |

* A 16 bit CRC is generated based on the contents of the DL Group Configuration A-MAP IE. The CRC is masked by the Station ID for the DL Group Configuration A-MAP IE.

Table 674- UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group identifier. |
| Lowest MCS Spectral Efficiency in group | 43 | The index of the lowest MCS Spectral Efficiency that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS Spectral Efficiency in group | 43 | The index of the highest MCS Spectral Efficiency that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Spectral Efficiency Interval | TBD | Indicates spectral efficiency interval supported in the group. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: reserved |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5) { |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP = 2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| }else if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP = 2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | [4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | [3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |

* A 16 bit CRC is generated based on the contents of the UL Group Configuration A-MAP IE. The CRC is masked by the Station ID for the UL Group Configuration A-MAP IE.

---------------------------------------------------------------- Text Ends ------------------------------------------------------------------