| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Tables Summarizing the DL and UL PHY Structure Parameters |  |
| Date Submitted | 2009-09-01 |  |
| Source(s) | Jeongho Park, Taeyoung Kim Samsung Electronics Fred Vook Motorola Inc. | jeongho.jh.park@samsung.com fred.vook@motorola.com |
| Re: | Category: P802.16m/D1 comments for LB30 Area: Chapter 15.3.5 (DL PHY structure) and Chapter 15.3.8 (UL PHY Structure) |  |
| Abstract | This contribution presents two tables summarizing the variables used to set the DL and UL PHY structure. |  |
| Purpose | Adopt as remedy to Comments 906 and 1225 of the LB30 comment database 802.16-09/0047 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Tables Summarizing the DL and UL PHY Structure Parameters

Fred Vook

Motorola Inc.

Jeongho Park, Taeyoung Kim

Samsung Electronics

- Introduction

A large number of variables are used to configure the DL and UL PHY structure.  Sections 15.3.5 and 15.3.8 each need a table that lists all of the variables, specifies their units (e.g., units of PRUs, units of sub-bands, etc.), and where they are signaled (if applicable).  Given the excessive complexity of this section, this type of table is essential.  This contribution proposes a table for the downlink and a table for the uplink.

- Table for DL PHY Section (15.3.5)

[ Insert the following text and table on page 272, line 42:  (Change the “YYY” to the appropriate number) ]

Table YYY presents a summary of the parameters used to configure the DL PHY structure.

Table YYY:  DL PHY Structure – Summary of parameters

|  | Operation Procedure | Related Signaling Field (BW20/10/5MHz) | Channel for Signaling | Parameters Calculated from Signaled Fields | Definition | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Sector Common | Sub-band Partitioning Mini-band Partitioning | DSAC (5/4/3 bits) | SFH – SP2 | KSB | Number of Subbands | Subbands |
|  |  |  |  | LSB=N1*KSB | Number of PRUs assigned to Subbands | PRUs |
|  |  |  |  | LMB | Number of PRUs assigned to minibands | PRUs |
|  | Frequency Partitioning | DFPC (4/3/3 bit) |  | FPCT | Number of Frequency Partitions | Frequency Partitions |
|  |  |  |  | FPSi | Number of PRUs in FPi | PRUs |
|  |  | DFPSC (3/2/1 bit) |  | KSB_FPi | Number of SBs assigned to FPi | Subbands |
|  |  |  |  | KMB_FPi | Number of Minibands assigned to FPi | Subbands (Groups of N1 PRUs) |
|  |  |  |  | LSB_FPi = N1* KSB_FPi | Number of PRUs assigned to be Subbands in FPi | PRUs |
|  |  |  |  | LMB_FPi = N2* KMB_FPi | Number of PRUs assigned to be Minibands in FPi | PRUs |
| Sector Specific | CRU / DRU Allocation | DCASSB,0 (5/4/3 bit) DCASMB,0 (5/4/3 bit) DCASi (3/2/1) bit | SFH – SP1 | LSB-CRU,FPi | Number of Subband-based CRUs in FPi | CRUs |
|  |  |  |  | LMB-CRU,FPi | Number of Miniband-based CRUs in FPi | CRUs |
|  |  |  |  | LCRU_FPi= LSB-CRU,FPi + LMB-CRU,FPi | Number of CRUs in FPi | CRUs |
|  |  |  |  | LDRU_FPi= FPSi- LCRU_FPi | Number of DRUs in FPi | DRUs |
|  | Tone Permutation | IDCell (10bit) | Obtained from SA-Preamble |  |  |  |

- Table for UL PHY Section (15.3.8)

[ Insert the following text and table on page 409, line 41 (Change the “ZZZ” to the appropriate number) ]:

Table ZZZ presents a summary of the parameters used to configure the UL PHY structure.

Table ZZZ:  UL PHY Structure – Summary of parameters

|  | Operation Procedure | Related Signaling Field (BW20/10/5MHz) | Channel for Signaling | Parameters Calculated from Signaled Fields | Definition | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Sector Common | Sub-band Partitioning Mini-band Partitioning | USAC (5/4/3 bits) | SFH – SP2 | KSB | Number of Subbands | Subbands |
|  |  |  |  | LSB=N1*KSB | Number of PRUs assigned to Subbands | PRUs |
|  |  |  |  | LMB | Number of PRUs assigned to minibands | PRUs |
|  | Frequency Partitioning | UFPC (4/3/3 bit) |  | FPCT | Number of Frequency Partitions | Frequency Partitions |
|  |  |  |  | FPSi | Number of PRUs in FPi | PRUs |
|  |  | UFPSC (3/2/1 bit) |  | KSB_FPi | Number of SBs assigned to FPi | Subbands |
|  |  |  |  | KMB_FPi | Number of Minibands assigned to FPi | Subbands (Groups of N1 PRUs) |
|  |  |  |  | LSB_FPi = N1* KSB_FPi | Number of PRUs assigned to be Subbands in FPi | PRUs |
|  |  |  |  | LMB_FPi = N2* KMB_FPi | Number of PRUs assigned to be Minibands in FPi | PRUs |
| Sector Specific | CRU / DRU Allocation | UCASSB,0 (5/4/3 bit) UCASMB,0 (5/4/3 bit) UCASi (3/2/1) bit | SFH – SP1 | LSB-CRU,FPi | Number of Subband-based CRUs in FPi | CRUs |
|  |  |  |  | LMB-CRU,FPi | Number of Miniband-based CRUs in FPi | CRUs |
|  |  |  |  | LCRU_FPi= LSB-CRU,FPi + LMB-CRU,FPi | Number of CRUs in FPi | CRUs |
|  |  |  |  | LDRU_FPi=FPSi- LCRU_FPi | Number of DRUs in FPi | DRUs |
|  | Tile Permutation | IDCell (10bit) | Obtained from SA-Preamble |  |  |  |