| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for sub-band indexing (15.3.9.3.1.3) |  |
| Date Submitted | 2009-09-22 |  |
| Source(s) | Jerry Pi, Jiann-An Tsai, Kaushik Josiam, Sudhir Ramakrishna, David Mazzarese Samsung Electronics Co., Ltd. | +1-972-761-7944 zpi@sta.samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | The contribution proposes the text for best-M subband indexing in P802.16m/D1 |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for sub-band indexing (15.3.9.3.1.3)

Jerry Pi, Jiann-An Tsai, Kaushik Josiam, Sudhir Ramakrishna, David Mazzarese

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of sub-band indexing to be included in P802.16m/D1 ([1]). The text proposal is based on an algorithm provided by Cover in 1973 ([2]).

The Matlab code of the encoding and decoding algorithm is listed as follows:

Encoding algorithm:

% N - # of subbands; M - # of reported subbands;

% s - subband selection bitmap; i - index of the best-M subband selection

function i = BestMEncoding(N, M, s)

i = 1;

for k = 1:N

if ((N - k) >= (M - sum(s(1:k-1))))

i = i + s(k) * nchoosek(N - k, M - sum(s(1:k-1)));

end

end

Decoding algorithm:

% N - # of subbands; M - # of reported subbands;

% s - subband selection bitmap; i - index of the best-M subband selection

function s = BestMDecoding(N, M, i)

s = zeros(1, N);

for k = 1:N

v = 0;

if(  sum(s(1:k-1) >= M ) break ; %Break out of FOR loop

if ((N - k) >= (M - sum(s(1:k-1))))

v = nchoosek(N-k, M-sum(s(1:k-1)));

end

s(k) = (i>v);

i = i - s(k) * v;

end

- References

[1] IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”, July 2009.

[2] T. M. Cover, “Enumerative source encoding,” IEEE Trans. Inform. Theory, vol. IT-19, pp. 73-77, January 1973

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

Insert the following text at section 15.3.9.3.1.3, page 450, line 21:

The AMS shall report the subband selection by an index

where N is the total number of sub-bands, 1 ≤ s0 < s1 <  < sM-1 ≤ N are the sorted indices of the best-M subbands, and

xy=xyx≥y0x<y

is the extended binomial coefficient.

-------------------------------  Text End  ---------------------------------------------------