| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comments resolution for ARQ section (15.3.8) |  |
| Date Submitted | 2009-09-02 |  |
| Source(s) | Youngbin Chang, Anil Agiwal Samsung Electronics | yb.chang@samsung.com, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0044, IEEE 802.16 Working Group Letter Ballot #30 / Topic: ARQ (Section 15.3.8) |  |
| Abstract | This contribution includes comment resolution recommendation regarding ARQ comments. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D1. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comments resolution for ARQ section for ARQ section

Youngbin Chang, Anil Agiwal

Samsung Electronics

# List of comments on ARQ section in 80216-09_047r2

| Comment | Type | Commenter |  | Recommended Resolution | Remedy (Reject reason) |
| --- | --- | --- | --- | --- | --- |
| 621 | Technical | Anil | Agiwal | Accept |  |
| 623 | Technical | Joey | Chou | Reject | ARQ feedback IE is transported extended header or MAC control message. |
| 624 | Editorial | Jaesun | Cha | Accept |  |
| 625 | Technical | Ming-Hung | Tao | Reject | Overhead saving is not clear. |
| 626 | Editorial | Lei | Wang | Accept |  |
| 627 | Technical | Jaesun | Cha | Reject | Headers are merged into FPEH and MEH |
| 628 | Technical | Joey | Chou | Reject | Headers are merged into FPEH and MEH |
| 629 | Technical | Stavros | Tzavidas | Accepted-Modified | Text is included in this document |
| 630 | Editorial | Naftali | Chayat | Superceded | Superceded by 632 |
| 631 | Technical | Vladimir | Yanover | Superceded | Superceded by 632 |
| 632 | Technical | Jaesun | Cha | Accepted-Modified | 1803r2 |
| 633 | Editorial | Yih Guang | Jan | Superceded | Superceded by 632 |
| 634 | Technical | Yang | Liu | Reject | There is not remedy. Problem description is not clear. |
| 635 | Technical | Lei | Wang | Accepted-Modified | Text is included in this document |
| 636 | Technical | Ronald | Murias | Superceded | Superceded by 635 |
| 637 | Technical | Baowei | Ji | Accepted-Modified | Text is included in this document |
| 638 | Technical | Lei | Wang | R | MAC CRC does not necessary. ARQ can detect an error if ARQ block is missed, which means that PHY CRC does not passed in HARQ layer. |
| 639 | Technical | Yang | Liu | Superceded | Superceded by 629 |
| 640 | Technical | Lei | Wang | Accept |  |
| 641 | Technical | Yang | Liu | Superceded | Superceded by 642 |
| 642 | Technical | Stavros | Tzavidas | Accepted-Modified | Text is included in this document |
| 643 | Technical | Ronald | Murias | Superceded | Superceded by 642 |
| 644 | Technical | Yang | Liu | Accepted-Modified |  |
| 645 | Technical | Stavros | Tzavidas | Superceded | Superceded by 644 |
| 1655L | Technical | Youngbin | Chang | Accept |  |
| 1656L | Editorial | Youngbin | Chang | Superceded | Superceded by 624 |
| 1657L | Editorial | Youngbin | Chang | Accept |  |
| 1658L | Technical | Youngbin | Chang | Accept |  |
| 1659L | Technical | Youngbin | Chang | Superceded | Superceded by #629 |
| 1660L | Technical | Youngbin | Chang | Accept |  |
| 1661L | Technical | Youngbin | Chang | Accept |  |

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

<Comment #629, 1659L>

[Modify the text from line 19-21 on page 165 as follows:]

15.3.8.1.3 ARQ feedback triggering conditions

Receiver shall sends an ARQ feedback when at least one of following conditions is met. an ARQ feedback poll is received from the transmitter or when an ARQ block has been missing for a predetermined period.

- An ARQ feedback poll is received from the transmitter

- An ARQ block is declared as an error by the receiver.

- An ARQ Discard message is received from the transmitter

[Modify the text from line 11-18 on page 166 as follows:]

15.3.8.4 ARQ_ REORDERINGERROR_DETECTION_TIMEOUT

ARQ block may arrive due to out-of order due to HARQ retransmission. ARQ_REORDERINGBLOCK_ERROR_TIMEOUT is the time duration for which the receiver shall wait before declaring an ARQ block as being in error. used to reorder ARQ blocks that may arrive out-of-order due to HARQ retransmission. If an ARQ block is detected as missing, fFor example if ARQ block SN #n arrives before #n-1, this timer starts for ARQ block SN # n-1 in the receiver. When the timer expires, the corresponding ARQ block the ARQ block SN #n-1 is declared as being in error in the receiver.

<Comment#635>

[Modify the text from line 6 on page 170 as follows:]

15.3.8.7.3.1 Receiver state machine

ARQ state machine procedure in a receiver is defined in Figure 43152 (see 6.3.4.6.3). In the figure, BSN equal to SN.

<Comment #637>

[Modify the text from line 22-29 on page 170 as follows:]

The procedure of ARQ state update is as follows. If SN is equal or larger than ARQ_RX_HIGHEST_SN, receiver updates ARQ_RX_HIGHEST_SN as SN+1. If SN is less than ARQ_RX_HIGHEST_SN and more than ARQ_RX_WINDOW_START, receiver state variables are not changed,. but If SN is not equal to ARQ_RX_WINDOW_START, ARQ_RX_PURGE_TIMEOUT for this SN shall be (re)set. If SN is less than ARQ_RX_HIGHEST_SN and equal to ARQ_RX_WINDOW_START, ARQ_RX_WINDOW_START is advanced to the next lowest numbered ARQ block that has not been received. The ARQ_SYNC_LOSS_TIMEOUT shall be (re)set. After finishing ARQ state update, ARQ block is stored in the receiver buffer.

<Comment 641, 642, 643>

[Modify the text from line 14-19 on page 171 as follows:]

15.3.8.7.4 ARQ Reset procedure

When a transmitter or receiver needs to trigger a reset of the ARQ state machine, the transmitter or receiver can start the ARQ reset procedures provided in <<<figures 50 and 51>>>. When ARQ reset error happens during the ARQ reset procedure, the ABS or AMS may reinitialize its MAC. The triggering conditions of ARQ reset are implementation specific.The ARQ reset procedures defined in figures 50 and 51(see 6.3.4.6.2) shall be supported by both the ARQ receiver and ARQ transmitter. Each side (receiver or transmitter) may initiate an ARQ reset procedure when ARQ Synchronization loss (see 15.3.8.7.5) happens or for any other implementation specific reason. When ARQ reset error happens during the ARQ reset procedure, the ABS or AMS may reinitialize its MAC.

<Comment 644, 645>

[Modify the text from line 23-24 on page 171 as follows:]

15.3.8.7.5 ARQ Synchronization loss

If transmitter or receiver declares an ARQ synchronization loss, transmitter or receiver may initiate the ARQ reset procedure. The actions following the ARQ synchronization loss is implementation specific. Synchronization of the ARQ state machines is governed by ARQ_SYNC_LOSS_TIMEOUT timer managed by the transmitter and receiver state machines. Each time ARQ_TX_WINDOW_START is updated in the transmitter or ARQ_RX_WINDOW_START is updated in the receiver, the ARQ_SYNC_LOSS_TIMEOUT timer is set to zero. When the ARQ_SYNC_LOSS_TIMEOUT timer expires at the transmitter or receiver, the transmitter or receiver declares ARQ synchronization loss.

<Comment 1661L>

[Add the following text after line 18 on pgae 166:]

15.2.8.x. ARQ_POLL_TIMEOUT

ARQ_Polling_Timeout is the time duration for which transmitter shall wait ARQ feedback from receiver. When the timer expires, transmitter may retry ARQ feedback poll.

---------------------------------------------------------End of the Text----------------------------------------------------------