| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of DL Multi-BS MIMO section and Table 51 (15.3.14) |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | David Mazzarese, Shuangfeng Han, Hokyu Choi | d.mazzarese@samsung.com |
| Re: | P802.16m/D1 LB30 |  |
| Abstract | This contribution proposes clean-up text for the control parameters for DL Multi-BS MIMO |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 1) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean-up of DL Multi-BS MIMO section and Table 51 (15.3.14)

David Mazzarese et Al.

Samsung Electronics

# Introduction

Table 51 needs updating following the adoption of the updates to the Feedback Polling A-MAP IE at session #63 [2]. Editorial clean-up of section 15.3.14.3 (DL Multi-BS MIMO) is also proposed in this contribution.

Additional technical changes are summarized below:

- Maximum number of users supported in Co-MIMO (MaxUser): allow values 1, 2, 3 and 4. This is consistent with the DL MU-MIMO, which allows up to 4 users on the same resource.

- The title of Table 51 is also changed with the deletion of “supported by codebook based feedback”, since MaxUser is also applicable to Co-MIMO performed with uplink sounding feedback instead of codebook.

- We specified that only DL MIMO modes 2 (SU CL MIMO) and 4 (MU CL MIMO) may be supported with Single BS precoding with Multi-BS Coordination and DL Multi-BS Joint MIMO Processing, since only these two MIMO modes make use of the PMI feedback to perform adaptive precoding at the ABS. These two modes using adaptive precoding may also be enabled by uplink sounding.

- We added the reference to the AAI_IM message in ABI, as accepted at session #63 [3].

- We added the reference to the Multi-BS MIMO feedback MAC control message, as accepted at session #63 [4].

- We removed the usage of DL Multi-BS Joint MIMO Processing with non-adaptive precoding, since it is not feasible. Joint processing requires adaptive precoding, obviously, and it depends on the feedback of channel state information (codebook or sounding).

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D1, July 2009.

[2] C80216m-09_2039r1, “Proposed changes to the feedback polling A-MAP IE (15.3.6.5.4.12)”, David Mazzarese et. Al, IEEE 802.16 Session #63, September 2009, Jeju, Korea.

[3] C80216m-09_2042r3, “Propose text of AAI PHY control Messages (15.2.6.x)”, Wookbong Lee et. Al, IEEE 802.16 Session #63, September 2009, Jeju, Korea.

[4] C80216m-09_2073r1, “Proposal to define MAC control messages and extended headers for MIMO feedback (15.2.4)”, David Mazzarese et. Al, IEEE 802.16 Session #63, September 2009, Jeju, Korea.

- Proposed text

< Modify section 15.3.14.3 on page 277 as follows >

-------------------------------------------------------Start of the Text-------------------------------------------------------

15.3.14.3 DL Multi-BS MIMO

Multi-BS MIMO techniques improve sector throughput and cell-edge throughput through multi-BS cooperative signaling. These include single-BS precoding with multi-BS coordination and multi-BS joint processing.

### 15.3.14.3.1 DL/UL Signaling

Table 51 – Control parameters for DL Multi-BS MIMO supported by codebook based feedback

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - ICT | - Interference coordination type | - 0b00: PMI restriction - 0b01: PMI recommendation - 0b10: CL-MD - 0b11: Co-MIMO | - Feedback_Polling_IE - Feedback Polling A-MAP IE | - Indicates which Multi-BS MIMO mode |
| - TRU | - Target resource unit | - TBD 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 - 0b11: boosted FFR partition | - Feedback_Polling_IE - Feedback Polling A-MAP IE | - Indicates resource units for measurement |
| - CPI | Concatenating PMI indication | 0b0: feedback - 0b1: not feedback | - Feedback_Polling_IE - Feedback Polling A-MAP IE | - Only used in CL-MD or Co-MIMO |
| - MaxUser | Maximum number of users supported in Co-MIMO | Maximum supported user number = MaxUser+2, e.g., 0b000 means 2 AMSs are supported in maximum. 0b00: MaxUser = 2 0b01: MaxUser = 3 0b10: MaxUser = 4 0b10: Reserved | - Feedback_Polling_IE - Feedback Polling A-MAP IE | - Only used in Co-MIMO. In CL-MD, the maximum supported user number is 1. |

Co-MIMO and CL-MD can be supported as instructed by the BS in the sounding command A-MAP IE  Sounding_command_IE.

Table 52 – Feedback information for DL Multi-BS MIMO supported by codebook based feedback

|  | Feedback information type | Description |
| --- | --- | --- |
| Periodic feedback | Base station ID | Report BS_ID using Temp_BSID. Temp_BSID is diversity set member ID assigned to this ABS. |
|  | PMI report for serving and neighboring cell | For PMI coordination among multiple ABSs |
|  | PMI_coordination_subset indication | Indicating one correlation level or two correlation levels associated with the PMI report for neighboring cell |
|  | Additional measurement metric | Optional additional feedback for indicating the potential interference level improvement associated with the PMIs reported for the neighboring cells |
|  | CPMI for neighboring cell | Concatenating PMI for neighboring cell |
| Event-driven feedback | Requesting Multi-BS MIMO | For AMS reporting of its preference on single BS precoding with Multi-BS MIMO coordination using a MAC management message |

Uplink sounding can be used to support Co-MIMO and CL-MD.

In a multi-BS joint processing mode (i.e., CL-MD mode or Co-MIMO), an ABS shall inform the corresponding AMS of the set of the ABSs participating in joint processing. The IDs of the joint processing ABSs should be transmitted from the serving ABS to the AMS by using the following message format.

Table 53. Parameters to be transferred from serving ABSs to AMSs.

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| N_preferred_C_ABSs | 3 | N_preferred_C_ABSs denotes the number of preferred cooperating ABSs excluding the serving ABS, which is expressed as a 3-bit binary number. For example, N_preferred_C_ABSs=0b001 means that the serving ABS is going to cooperate with one neighbor ABS for the AMSs. |
| Preferred_C_ABS IDs | [TBD] | Note: IDs of the preferred cooperating ABSs |

### 15.3.14.3.2 Single BS precoding with Multi-BS Coordination

This subclause describes interference mitigation techniques based on the MIMO schemes defined in << Section 15.3.7 >> applicable with DL MIMO modes 2 and 4 with codebook-based feedback mode, with additional inter-ABS coordination mechanisms and interference measurement support. The inter-ABS coordination mechanisms in this subclause do not require data forwarding between different ABSs.

### 15.3.14.3.2.1 Operation procedure

Two types of single BS precoding techniques with Multi-BS coordination may be supported in AAI. One is PMI coordination, supported by codebook-based feedback, and the other is interference nulling, supported by codebook-based feedback or by uplink sounding.

Single BS precoding with Multi-BS Ccoordination may be enabled by the ABS for one or several AMSs when CL MIMO precoding with DL MIMO mode 2 or 4 is applied in the serving and neighboring cells. The inter-cell interference can be mitigated by coordinating the precoders applied in neighboring cells via higher layer signaling, based on feedback from AMSs to their respective serving ABSs.

With codebook-based feedback, PMI coordination can be applied as either PMI recommendation or PMI restriction, as instructed by the ABS in Feedback_Polling_IE Feedback Polling A-MAP IE.

If ICT (interference coordination type) is set to 0b00 in Feedback_Polling_IE Feedback Polling A-MAP IE, then the MS finds the PMI which acts as the strongest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE Feedback Polling A-MAP IE.

If ICT (interference coordination type) is set to 0b01 in Feedback_Polling_IE Feedback Polling A-MAP IE, then the MS finds the PMI which acts as the weakest interference for the neighboring cell in the frequency resource unit indicated by TRU (target resource unit) indicated in Feedback_Polling_IE Feedback Polling A-MAP IE.

Restricting or recommending the usage of rank-1 codebook elements as a response to the neighboring cell’s request is done by the BS transmission of BC_SI in additional broadcast information with the AAI_IM Message. Details are in << 15.3.7.2.6.6.2.4 >>.

The operation procedure of PMI coordination follows the steps below:

1.	PMI coordination may be triggered by an AMS in an unsolicited manner. Based on the channel measurements of the desired and interference links reference signals, an AMS may request operation with PMI coordination in an unsolicited manner via an event-driven feedback report to its serving BS using a MAC management message. the AAI_MULTI_BS_MIMO-REQ message.

2.	The ABS may accept or reject the request from the AMS. The ABS shall send a Feedback_Polling_IE to the AMS to accept the request, or it may do nothing to reject the request. The ABS may also send a Feedback_Polling_IE Feedback Polling A-MAP IE to an AMS in an unsolicited manner. The ABS shall indicate operation with PMI recommendation or restriction in the Feedback_Polling_IE Feedback Polling A-MAP IE.

3.	Once an AMS receives a Feedback_Polling_IE Feedback Polling A-MAP IE, it shall send a periodic AAI_MultiBS_MIMO_FBK MAC control message feedback header with the requested information. This information may include a PMI or a set of PMIs, Temp_BSID (diversity member ID) and additional measurements.

For reporting a set of PMIs, the following procedure shall be performed at the MS after determining the recommended (or restricted) PMI wk:

- Considering all PMIs (w0, w1, …, wm) in the rank-1 DL base codebook C(2,1,3,m) for m = 0 to 8 with 2Tx, or in the rank-1 DL base codebook subset C(4,1,6,m) for m = 0 to 15 with 4Tx, or in the rank-1 DL base codebook C(8,1,4,m) for m = 0 to 15 with 8Tx, the AMS calculates the cross correlation of each PMI to the recommended (or restricted) PMI wk. The cross-correlation between PMIs i and k is defined in equation (174), with superscript H indicating conjugate transpose.

i=1,…, N										(174)

Assume that the N correlation values,, are sorted in descending order and then renamed as, such that:

(175)

- The AMS determines the size of the subset of PMIs to be jointly recommended (or restricted), based on two fixed correlation levels determined by n1 and n2 in equation (2). The AMS indicates the selection of n1 or n2 via PMI_coordination_subset in a feedback header. The value of PMI_coordination_subset is specified in Table 54.

Table 54 - PMI_coordination_subset

| PMI_coordination_subset | Value |
| --- | --- |
| 0b0 | n1 |
| 0b1 | n2 |

4.	Upon receiving feedback from multiple AMSs, an ABS should communicate with neighboring ABSs to coordinate their usage of PMIs via higher layer signaling. The ABS should then broadcast codebook subset information in BC_SI in Additional Broadcast Information (ABI) message AAI_IM to all AMSs in its cell. BC_SI is indicated by a bitmap.

5.	The ABS may send a Feedback Allocation A-MAP IE with CM set to 0b11 to selected AMS. Consequently, these AMSs should feedback their desired PMI in the codebook subset broadcasted in BC_SI.

Inter-cell interference nulling can be done using PMI which acts as a strongest interference for the neighboring cell or overhearing neighboring cell’s sounding signal.

### 15.3.14.3.3 DL Multi-BS Joint MIMO Processing

This subclause introduces interference mitigation techniques based on joint MIMO transmission across multiple ABSs. The ABS and AMS may optionally support one or both adaptive precoding based multi-BS joint processing techniques, e.g. Closed-loop Marco Macro Diversity (CL-MD) and Collaborative MIMO (Co-MIMO) transmission and non-adaptive precoding based multi-BS joint processing. CL-MD may be used with DL MIMO mode 2, while Co-MIMO may be used with DL MIMO mode 4. Multi-BS joint MIMO processing may be enabled by the ABS for one or several AMSs when adaptive or non-adaptive precoding is applied in the serving and neighboring cells and user data is shared among multiple cells.

### 15.3.14.3.3.1 Operation procedure

With adaptive precoding, the precoder matrix Wk is derived from the feedback of the MS, e.g. with codebook-based feedback and or sounding-based feedback. Two types of adaptive precoding based multi-BS joint processing are supported, CL-MD and Co-MIMO. When CL-MD is enabled, a single AMS is served jointly by multiple coordinating ABSs. When Co-MIMO is enabled, several AMSs are served jointly by the multiple coordinating ABSs through MU-MIMO scheduling and precoding.

For codebook-based feedback, the AMS(s) choose the PMIs for the serving cell and the neighboring cells based on the respective estimated channel state information. Optionally, the serving ABS can also instruct the AMS(s) to feedback a concatenating PMI (CPMI) for the neighboring cells to further improve the system performance.

When DL Multi-BS joint processing is enabled, radio resource allocation, data mapping and pilot pattern allocation should be aligned among coordinating ABSs. The same data packet is transmitted by the coordinating ABSs on the same time and frequency resources. The same pilot patterns without interlacing shall apply to the coordinating BSs.

The operation procedure of DL Multi-BS Joint Processing follows the steps below:

4)1)	Once an AMS receives a Feedback_Polling_IE Feedback Polling A-MAP IE, it shall send a periodic Multi-BS MIMO feedback MAC control message (AAI_MultiBS_MIMO_FBK) feedback header with the requested information in case of codebook feedback, or send UL sounding as instructed by Sounding_Allocation_IE() sounding command A-MAP IE in case of sounding feedback.

5)2)	Upon receiving feedback from multiple AMSs In for the case of codebook feedback, the ABS shall forward the PMIs and CPMI (in case of CPI feedback enabled in Feedback Polling A-MAP IE) to neighboring ABSs to coordinate the usage of PMIs. In case of sounding feedback, each involved ABS can perform precoding based on the received sounding signal(s) from single AMS for CL-MD or from multiple AMSs for Co-MIMO.

The default number of neighboring ABSs coordinated to support Collaborative MIMO (Co-MIMO) transmission is three.

The Collaborative MIMO Zone (Co-MIMO Zone) is defined to facilitate inter-ABS coordination for supporting Co-MIMO transmission. Co-MIMO Zone is a radio resource region composed by LRUs and sub-frames, where the Co-MIMO Zone utilized by neighboring ABSs for Co-MIMO transmission will associate to the same LRU and sub-frames. The permutation of the Co-MIMO Zone for these ABSs shall be the same.

Each Co-MIMO Zone will be used for serving one or multiple AMS, and each ABS can have multiple Co-MIMO Zones. The associated LRUs, permutation and sub-frames for each Co-MIMO Zone will be negotiated by the ABSs involved in Co-MIMO transmission through the backhaul network before the allocation.

--------------------------------------------------------End of the Text-------------------------------------------------------