| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the Contention-Based BR Procedure (15.2.11.1.1) |  |
| Date Submitted | 2009-09-17 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | Letter Ballot #30 on the Draft Amendment (IEEE P802.16m/D1) |  |
| Abstract | This contribution proposes the NACK message format for BR header collision |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the Contention-Based BR Procedure (15.2.11.1.1)

Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Proposed Text

Apply the following text modification to P802.16m/D1, Section 15.2.11.1.1 at page 153, line 21 as follows:
---------------------------------------------------------- Text Start ----------------------------------------------------------------

In case the quick access message is not decodable, a 5-step BR procedure as a fallback mode for the 3-step BR procedure is illustrated in Figure 429—. The AMS obtains resource grant from ABS using CDMA Allocation A-MAP IE at step 2. At step 3, the AMS transmits a standalone BW REQ header or use the given UL resource for its uplink data transmission instead of the BW-REQ message. In both cases, STID shall be carried in the uplink transmission. ABS may allocate UL grant for uplink transmission at step 4. The AMS then performs its uplink transmission at step 5. After step 3, the AMS shall start the BR timer with the differentiated value defined during DSx transaction. The ABS shall transmit AAI_BRH-NACK message for the corresponding standalone BR header in case it is not decoded in step 3. If AMS receives the negative acknowledge, it may restart the BR procedure, otherwise, it shall start the BR timer with the differentiated value defined during DSx transaction. The AMS may restart the BR procedure if timer is expired.

Figure 429—Example of 5-step random access BR procedure as a fallback mode for the 3-step BR procedure

In 5-step regular random access BR procedure, AMS sends a bandwidth request indicator only. After step 2, only fixed value timer is activated. The rest of the BR procedure shall be the same as the 5-step procedure in Figure 429

The AAI_BRH-NACK message includes the resource assignment information for the BR header which is not decoded.

------------------------------------------------------------ Text End ---------------------------------------------------------------